SA WG2 Meeting #160-Ad-Hoc-e	S2-2400556

# January 22th – 29th, 2024; Elbonia													revision of S2-2312650

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1013 | rev | 1 | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addressing EN on reference of MDA service |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_ph3 |  |  |  |  |  | Date: |  |  | 2024-01-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | There is an EN in the R18 TS 23.288 for NWDAF data collection from OAM as following. Editor's note: The MDA services are to be defined by SA WG5 and NWDAF could subscribe to them. The reference to SA WG5 specification will be added when defined in SA WG5. During R18 work phase, mechanism and procedure for NWDAF collecting analytics from MDAF have already been defined in clause 6.2.14, TS 23.288, where the MDA service is referred to TS 28.104. Thus, this EN can be deleted and add reference to SA5 TS 28.104. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Delete the Editor's note: The MDA services are to be defined by SA WG5 and NWDAF could subscribe to them. The reference to SA WG5 specification will be added when defined in SA WG5. - Reference to TS 28.104 for MDA service is added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specification is not complete with the legacy EN. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * First Change * * * *

6.2.3	Data Collection from OAM

6.2.3.1	General

The NWDAF may collect relevant management data from the services in the OAM as configured by the PLMN operator.

‐	NG RAN or 5GC performance measurements as defined in TS 28.552 [8].

‐	5G End to end KPIs as defined in TS 28.554 [10].

NWDAF shall use the following services to have access to the information provided by OAM:

-	Generic performance assurance and fault supervision management services as defined in TS 28.532 [6].

‐	PM (Performance Management) services as defined in TS 28.550 [7].

‐	FS (Fault Supervision) services defined in TS 28.545 [9].

- MDA (Management Data Analytics) services as defined in TS 28.104 [45].

NWDAF can be configured to invoke the existing OAM services to retrieve the management data that are relevant for analytics generation, which may include NF resources usage information (e.g. usage of virtual resources assigned to NF) and NF resource configuration information (e.g. life cycle changes of NF resource configurations).

OAM perform the required configuration in order to provide the information requested by NWDAF subscription and perform the tasks, e.g. data collection, data processing, associated with the subscribed request from NWDAF.

Another usage of OAM services is when the target of data collection is a specific UE, via MDT based retrieval of information:

-	Measurement collection for MDT as defined in TS 37.320 [20].

In addition, NWDAF can be provisioned with Network Slice information (i.e. as defined by the NetworkSliceInfo specified in TS 28.541 [22]) when a slice is created or modified via OAM configuration mechanism as defined in TS 28.541 [22] and TS 28.532 [6].

* * * * End of Change * * * *