SA WG2 Meeting #160-AH-e	S2-2400568

Janaury 22 - 29, 2024, Electronic Meeting

Source:	InterDigital Inc., Nokia, Nokia Shanghai Bell

Title:	FS_UIA_ARC Terms

Document for:	Approval

Agenda Item: 	19.8

Work Item / Release:	FS_UIA_ARC /Rel-19

Abstract of the contribution: This paper proposes text for the terms section of the FS_UIA_ARC TR 23.700-32.

# 1	Discussion

This paper proposes text for the terms section of the FS_UIA_ARC TR 23.700-32.

# 2	Proposal

It is proposed to include the following changes in TR 23.700-32 V0.0.0.

* * * * Start of Changes * * * *

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Link: An association between a User Identifier and a Subscription.

NOTE: 	A human User Identifier may be linked to zero or more 3GPP subscriptions. A 3GPP subscription can be linked to zero or more human User Identifiers. A human User Identifier must be linked to a 3GPP subscription before it can be used to access network services (i.e., before it can be activated).

Activated User Identifier: A User Identifier that is actively using a 3GPP Subscription to access network services (e.g., voice, SMS, and data, etc.).

Human User Identifier: An identifier containing a human’s user identity that can be used to access services of the 5GS using a 3GPP subscription (SUPI / GPSI).

Device User Identifier: An identifier containing a non-3GPP device’s user identity that can be used to access services of the 5GS using a 3GPP subscription (SUPI / GPSI).

Gateway UE: a UE, which acts as a gateway providing access to and from the 3GPP network for one or more non-3GPP devices that are connected to the gateway UE.

* * * * End of Changes * * * *