3GPP TSG-SA2 Meeting #160AH-e	S2-2400590

Electronic meeting, 22nd - 29th January 2024	(revision of S2-240xxxx)

Source:	LG Electronics

Title:	New KI: ATSSS enhancement to support architecture not based on current TNGF/N3IWF

Document for:	Approval

Agenda Item:	19.13

Work Item / Release:	FS_MASSS / Rel-19

Abstract of the contribution: This paper proposes a new key issue for WT#3.

Discussion

FS_MASSS (SP-231802) includes the following WT related with ATSSS_Ph4:

WT#3: Study whether and how to define a functional architecture and procedures for steering, switching and splitting of traffic not based on current TNGF/N3IWF to simplify the operation over non-3GPP access without compromising the security of the 5G network.

WT#3.1: Study whether to keep the NAS signalling connection on non-3GPP access or not, whether to eliminate IPSec tunnel encapsulation on the user plane only or both on the control plane and the user plane, simplifying the protocol stack, reduce the user plane overhead.

WT#3.2: Study whether and how to support splitting, switching, steering between 3GPP access and "non-3GPP access without 5G NAS". Study whether and how to enhance registration and security aspects for supporting "non-3GPP access without 5G NAS".

NOTE 10:	This WT requires coordination with SA3 for handling security aspects.

This paper proposes a new KI related to WT#3.

Proposal

It is proposed to agree the following changes into TR 23.700-54.

* * * * Start of 1st Change * * * *

!!! All New Texts !!!

5.2.x	Key Issue #X: ATSSS enhancement to support architecture not based on current TNGF/N3IWF

5.2.x.1 	Description

This key issue studies:

-	whether and how to define a functional architecture and procedures for steering, switching and splitting of traffic not based on current TNGF/N3IWF to simplify the operation over non-3GPP access without compromising the security of the 5G network.

-	whether to keep the NAS signalling connection on non-3GPP access or not, whether to eliminate IPSec tunnel encapsulation on the user plane only or both on the control plane and the user plane, simplifying the protocol stack, reduce the user plane overhead.

-	whether and how to support splitting, switching, steering between 3GPP access and "non-3GPP access without 5G NAS". Study whether and how to enhance registration and security aspects for supporting "non-3GPP access without 5G NAS".

NOTE:	This key issue requires coordination with SA3 for handling security aspects.

* * * * End of Changes * * * *