3GPP TSG-WG SA2 Meeting #160-Ad Hoc-e 	S2-2400596

E-meeting, January 22 – 29, 2024

Source:	China Mobile

Title:	KI #5, New Sol: Dynamic Change Indication of XR traffic for enhanced QoS Handling

Document for:	Approval

Agenda Item:	19.3

Work Item / Release:	FS_XRM_Ph2 / Rel-19

Abstract: Propose a solution for dynamic change indication of XR traffic for QoS handling enhancement related to KI #5.

1. Introduction/Discussion

In SA2#159, The key issue of QoS handling when traffic characteristics change dynamically has been agreed with the following description:

“This key issue proposes to study whether and how to support dynamic change (via user plane) in traffic characteristics (e.g., burst related parameters), provided by the application in the DN.

The key issue includes the following aspects:

-	Identify what traffic characteristics are dynamically changed.

-	Whether and how the 5G network can be enhanced to know about dynamic changes in traffic characteristics of GBR and non-GBR flows.

-	What information from the Application in the DN, if any, is needed by the 5G network to be able to know the dynamic changes in traffic characteristics and how this information is provided by the Application in the DN.

-	At what granularity does the 5GS need to know of changes in traffic characteristics (e.g., QoS Flow granularity).

-	What handling may be needed when the 5G network knows about changes in the traffic characteristics.”

With this agreed KI, this contribution proposes a new solution to support dynamic change indication of XR traffic for adaptive QoS adjustment in 5GS.

For XR and media services, the traffic pattern can change dynamically, e.g. the size of media frames or the length of data bursts vary due to scene changes that may occur at times such as initial start-up or due to user drags on a progress bar. Data burst related parameters could be one of traffic characteristics to depict the XR service with highly dynamic change.

To indicate the dynamic change of XR traffic, AF could provide the 5G network with the information of data burst e.g. start and/or end information of a data burst. With the knowledge of traffic change of XR service, the 5GS could adjust QoS handling to optimize the network resource scheduling and guarantee the QoS target.

This contribution proposes a solution for data burst indication of XR traffic when traffic characteristics change dynamically to enhance QoS handling. The solution applies to KI #5.

2. Text Proposal

It is proposed to capture the following changes in TR 23.700-70.

# * * * * First change(all new) * * * *

## 6.X	Solution #X: QoS Handling Enhancement based on Dynamic Change Indication of XR traffic

6.X.1	Key Issue mapping

This solution is for KI #5 QoS Handling when Traffic Characteristics Change Dynamically.

6.X.2	Description

This solution proposes a dynamic change indication mechanism of XR traffic between AF and 5GS to enhance QoS handling.

XR and media service traffic may change dynamically due to the traffic pattern change, e.g. frame rate or periodicity, GOP structure or dynamic GOP of different video frame types, the length of data bursts may vary from scenes, which makes fulfilling the QoS targets of XR services highly challenging. In order to capture the traffic changes, data burst related parameters could be one of traffic characteristics to depict the XR service with highly dynamic changes, which can be provided to the 5GS for flexible QoS handling.

In this solution, to achieve efficient QoS adjustment of RAN side, the data burst information carried in the XR service data flow could be provided by Application via user plane. The UPF may identify the data burst information and provide the available data burst information to the RAN.

6.X.3	Procedures

The procedure of dynamic change indication assisted QoS handling is described in the Figure 6.X.3.1-1.

Figure 6.X.3.1-1: High level procedure of dynamic change indication assisted QoS handling

1.	The AF requests the 5GC to fulfill the QoS requirements for XR services, and provides the Protocol Description which will be used for detecting data burst information (e.g. start information of data burst, end information of data burst) in the service data flow via user plane.

2.	The NEF performs authorization of the AF request. If the request is authorized, the NEF provides the Alternative Service Requirements to PCF by invoking the Npcf_PolicyAuthorization_Create request. Otherwise, NEF responses to AF that the request is not authorized and the procedure stops.

3.	Based on the QoS Requirements and Protocol Description from AF, the PCF generates PCC rules for QoS flows and may provide the Protocol Description to the SMF for data burst information indication.

4.	Based on the received PCC Rule, the SMF instructs the UPF to perform data burst identification and marking and may provide the UPF the Protocol Description used by the service data flow.

5.	The UPF applies the rules for data burst identification and marking, and provides the available data burst information to the RAN in the GTP-U header. The data burst information includes start information of data burst, end information of data burst, etc.

6.	Based on the data burst indication carried in the GTP-U header from UPF, RAN will adjust resource allocation for precise QoS handling to accommodate dynamic changes of data bursts for XR services.

6.X.4	Impacts on services, entities and interfaces

AF

- Provides the QoS requirements and the Protocol Description that is used by the UPF to detect data burst information received via N6, e.g. start information of data burst, end information of data burst.

AS

- Sends the traffic with carrying start information of data burst, end information of data burst in the packet header.

PCF

- Generates a PCC, which derives QoS parameters this PCC rule based on the AF request. The PCC rule may provide the Protocol Description.

- Send the PCC rule to SMF.

SMF

- Provides the UPF with indication of data burst identification and marking, as well as the received Protocol Description used by the service data flow.

RAN

- Adjusts resource allocation for precise QoS handling based on the data burst information indication received from UPF.

UPF

- Identifies and marks the start of a data burst based on the start information within the data packet header information, or based on UPF detection.UPF provides the data burst information (e.g. start information of data burst, end information of data burst) to the NG-RAN over GTP-U.

# * * * * End of change * * * *