3GPP TSG-WG SA2 Meeting #160-Ad Hoc-e	S2-2400639

Online, Jan 22 – 29, 2024

Source:	Huawei, HiSilicon

Title:	Architecture assumptions for FS_VMR_Ph2

Document for:	Approval

Agenda Item:	19.6

Work Item / Release:	FS_VMR_Ph2/ Rel-19

Abstract: The architecture assumptions for FS_VMR_Ph2 are proposed.

1. Introduction

This pCR proposes architecture assumptions for FS_VMR_Ph2.

2. Discussion

Based on the agreed SP-231799, the architectures with full gNB onboard of relay is considered in Rel-19 to support the additional use cases for VMR. The gNB in the relay establishes N2/N3 interface to an AMF/UPF residing in the 5GC over a PDU session.

In addition, according to the agreed RP-234041, it is assumed that Wireless Access Backhaul (WAB) is aligned with VMR use cases and with the SA2-endorsed SID on architectural enhancements for Rel-19 VMR. The architectures with full gNB onboard the WAB is described in RP-231973(see Figure 2-1).

Figure 2-1: Wireless Access and Backhauling (WAB)

The NR Uu is used for the radio link between a Mobile WAB(MWAB) and served UEs. In order to make sure the N2/N3 connection is not failed, the PDU session of the MWAB-UE for the wireless backhauling shall be kept in active state and the MWAB-UE is kept in CM-CONNECTED state.

In addition, it is proposed that only one MWAB is involved between the served UE and its serving PLMN.

2. Text Proposal

It is proposed to capture the following changes in TR 23.700-06.

# * * * * First change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[x]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

# * * * * Second change * * * *

4	Architecture assumptions and requirements

4.1	Architecture assumptions

The study should be based on the following architecture assumptions:

-	This study uses the architecture of a full gNB onboard of MWAB node and a PDU session for the wireless backhauling of the N2/N3 interfaces.

-	Only one MWAB node is involved between the served UE and its serving PLMN.

NOTE 1:	FS_VMR_Ph2 is aligned with the architectural enhancements for Wireless Access Backhaul (WAB) in RAN. Coordination with RAN is needed due to the dependency with RAN.

NOTE 2:	The wireless backhauling of N2/N3 may use the existing TN and NTN.

-	The MWAB node may serve UEs located inside or outside the vehicle mounted with the relay;

-	NR Uu is used for the radio link between a MWAB node and served UEs.

-	LCS framework as defined in TS 23.273 [x] is used for providing the location service to the served UEs;

-	The PDU session of MWAB-UE for the wireless backhauling shall be kept in active state and the MWAB-UE is kept in CM-CONNECTED state, in order to make sure the N2/N3 connection is not failed.

-	There is no enhancement to the existing QoS mechanisms for the wireless backhaul.

# * * * * End of changes * * * *