3GPP TSG-SA2 Meeting #106-Ad Hoc-e	S2-2400651

22-29 Jan 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | draftCR | Rev |  | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Enhancing Disaster Condition notification and determination procedures in the case of core network failure with 5G MOCN (WT1 of TEI19_NetFailTol) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19_NetFailTol |  |  |  |  |  | Date: |  |  | 2024-01-22 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Based on SA1 requirements introduced in R19 MINT_Ph2, in the case of core network failure, the shared-RAN of the PLMN with Disaster Condition shall be able to broadcast the Disaster Condition to notify the affected UE to deregister from the PLMN with Disaster Condition, and perform Disaster Roaming. This enhancement minimizes the interval between a UE losing service and attempting Disaster Roaming. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Enhancing Disaster Condition notification and determination procedures in the case of core network failure with 5G MOCN |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE experiences a long period of service interruption. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.40.1, 5.40.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change * * * *

5.40.1	General

Subject to operator policy and national/regional regulations, 5GS provides Disaster Roaming service (e.g. voice call and data service) for the UEs from PLMN(s) with Disaster Condition. The UE shall attempt Disaster Roaming only if:

-	there is no available PLMN which is allowable (see TS 23.122 [17]);

-	the UE is not in RM-REGISTERED and CM-CONNECTED state over non-3GPP access connected to 5GCN;

-	the UE cannot get service over non-3GPP access through ePDG;

-	the UE supports Disaster Roaming service;

-	the UE has been configured by the HPLMN with an indication of whether Disaster roaming is enabled in the UE set to "disaster roaming is enabled in the UE" as specified in clause 5.40.2; and

-	a PLMN without Disaster Condition is able to accept Disaster Inbound Roamers from the PLMN with Disaster Condition.

In this Release of the specification, the Disaster Condition is not limited to the failure of NG-RAN nodes, as long as the PLMN with Disaster Condition is able to authenticate its subscribers.

* * * * Start of 2nd Change * * * *

5.40.3	Disaster Condition Notification and Determination

The NG-RAN in the PLMN that provides Disaster Roaming service, broadcasts an indication of accessibility for Disaster Roaming service, and optionally, a 'list of one or more PLMN(s) with Disaster Condition for which Disaster Roaming service is offered by the available PLMN' in the impacted area as described in TS 38.304 [50] and TS 38.331 [28].

A UE determines the Disaster Condition based on the information broadcasted from the NG-RAN providing Disaster Roaming service, and performs the network selection and the access control for the Disaster Roaming as described in TS 23.122 [17] and TS 24.501 [47].

In the case of core network failure, the shared NG-RAN of the PLMN with Disaster Condition broadcasts the Disaster Condition. A UE determines the Disaster Condition based on the information broadcasted from the shared NG-RAN of the PLMN with Disaster Condition, deregisters from the PLMN with Disaster Condition and performs the network selection and the access control for the Disaster Roaming as described in TS 23.122 [17] and TS 24.501 [47].

NOTE 1:	How a PLMN is notified that another PLMN is a PLMN with Disaster Condition and how a PLMN is notified of the area where the associated Disaster Condition applies is managed by the government agencies or the authorities, and is out of scope of 3GPP.

NOTE 2:	The broadcast for Disaster Roaming service from the NG-RAN occurs only during the Disaster Condition.

NOTE 3:	The NG-RAN shared by the PLMN with Disaster Condition and the PLMN providing Disaster Roaming service can broadcast the Disaster Condition with the PLMN ID used for the PLMN with Disaster Condition, and broadcast an indication of accessibility for Disaster Roaming service with the PLMN ID used for the PLMN providing Disaster Roaming service.

* * * * End of Change * * * *