SA WG2 Meeting #160AH-e	S2-2400670

# E-meeting, January 22nd – 29th, 2024

Source: 	China Telecom,

Title: 	New Key Issue on WT #1.1

Document for: 	Approval

Agenda Item: 	19.7

Work Item / Release:	FS_5G_ProSe_Ph3 / Rel-19

Abstract of the contribution:

- Discussion

This contribution proposes a new Key Issue to address work task #1.1 of the FS_5G_ProSe_Ph3 SID (SP-231798) approved at the SA #102 meeting.

- WT-1: Enhance ProSe to support multi-hop over NR PC5 reference point - WT-1.1: for UE-to-Network Relay - WT-1.2: for UE-UE Relay NOTE 1: The UE-to-Network Relay include both Layer-3 and Layer-2 Relays. UE-to-UE Relay include L3 Relay only. NOTE 2: This study requires coordination with RAN WGs.

2. Proposal

It is proposed to adopt the following text in TR 23.700-03.

*** Start of the 1st change (all new text) ***

5.X	Key Issue #X: Support of Multi-hop UE-to-Network Relay

5.X.1	General description

This key issue intends to support 5G ProSe Communication via multi-hop over NR PC5 reference point for Layer-3 and Layer-2 UE-to-Network Relay as illustrated in figure 5.x.1-1. This key issue intends to support multi-hop UE-to-Network relays for in coverage and out of coverage operation.

Figure 5.x.1-1: Example scenario of support of Multi-hop UE-to-Network Relay

When studying the above aspect, the following needs to be considered:

- Whether and how to restrict the combination of Layer-2 and Layer-3 UE-to-Network Relay in a multi-hop route.

- How to discover multi-hop UE-to-Network Relay(s) and create a route, including how to prevent routing Loops.

- Whether and how the network can control multi-hop UE-to-Network Relay operation, at least including:

-	Whether and how to authorize the multi-hop UE-to-Network Relay, e.g. authorize a UE as multi-hop UE-to- Network Relay.

-	Whether and how to authorize the Remote UE to use a multi-hop UE-to-Network Relay.

-	How to transfer data between the Remote UE and the network over the multi-hop UE-to-Network Relay.

-	How to support end-to-end requirements between Remote UE and the network via multi-hop UE-to-Network Relay, including QoS (such as data rate, reliability, latency), and how the network allows and controls the QoS requirement for 5G ProSe multi-hop UE-to-Network relay.

-	How to enhance the system architecture to provide security/privacy protection for a relayed connection.

-  Whether and how to provide a mechanism for a path changing in the case of e.g. UE-to-Network Relay changes, or re-create a route.

-  Whether and how to enhance the existing mechanisms for IP address/prefix allocation.

*** End of the 1st change ***