3GPP TSG-SA WG2 Meeting #160-Ad Hoc-e	S2-24xxxxx

# E-meeting, January 22 – 29, 2024											 (revision of S2-24xxxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | DraftCR | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | UE requested URSP provisioning |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19_URSPreq |  |  |  |  |  | Date: |  |  | 2024-01-02 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To support UE requested URSP provisioning, UE triggered URSP provisioning procedure is proposed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add UE triggered URSP provisioning procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE requested URSP provisioning is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS 23.503 CRxxxx |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change * * * *

4.X	Procedure for UE triggered URSP provisioning

The UE may request URSP provisioning to the PCF when the UE determines the URSP rules need to be provided by the PCF (e.g., URSP rules are missing or invalid, when the UE's direct Uu signal strength is getting worse, when the UE is getting out of network coverage).

Figure 4.X-1 illustrates the UE triggered URSP Provisioning procedure.

Figure 4.X-1: UE triggered URSP provisioning procedure

1.	The UE sends UL NAS TRANSPORT message carrying the UE Policy Container (UE Policy Provisioning Request to request URSP) to the AMF.

2.	The AMF sends the Namf_Communication_N1MessageNotify request to the PCF including the UE Policy Container received from UE.

3.	The PCF receives UE Policy Container which indicates UE Policy Provisioning Request to request URSP. The PCF performs the UE Policy delivery procedure defined in clause 4.2.4.3.

* * * * End of Changes * * * *