3GPP WG SA2 Meeting #160AHE	S2-2400715

# 22 - 29 January, 2024, Electronic			         (was S2-24aaaaa)

Source: 	NEC

Title: 	New KI: Interworking between CAG cells and CSG cells

Document for: 	Approval

Agenda Item: 	19.4

Work Item / Release:	FS_5G_Femto / Rel-19

Abstract of the contribution: This contribution proposes a new key issue based on WT#2.

# 1	Discussion

This contribution proposes a new Key Issue based on WT#2.

The objective of WT#2 of FS_5G_Femto is :

- WT#2: How to enable interworking between CAG and CSG cells.

# 2 Proposal

It is proposed to include the below new Key Issue in the TR.

* * * Start of change * * * *

## 5.x	Key Issue #x: Interworking between CSG and CAG cell

### 5.x.1	Description

### The concept of Home NodeB (HNB) and Home eNodeB (HeNB) (also called femto cell) has been introduced in UMTS and EPS respectively to enables small access points deployed in customer premises (on a campus or at home) for access to operator, Internet and local services like local printers or local servers.

### 3GPP has agreed a SID to study how to enhance 5GS to introduce such a feature with focus on enabling access control for “5G NR Femto” use cases. For example, 5G NR Femto extends e.g., coverage using higher frequency bands (e.g., FR2 bands), also improves 5G indoor coverage, offloads traffic from the macro network, enables better voice quality, and better supports enterprise mobility. In 5G architecture, it has been decided to use the Closed Access Group (CAG) concept to provide access control function.

### When an operator deploys CSG cells and CAG cells to provide same set of services There can be scenario when an operator may provide CSG service in EPS and 5GS under same subscription or when the coverage of CGS cells and CAG cells are overlapping at then it is not clear how the cell reselection/selection will take place.

### In order to support the UE mobility between a CSG cell and CAG cell the SA2 should study:

- How does 5GS or EPS support idle mode/connected mode mobility cell between a CSG cell and CAG cell;

- How does an operator provision UE with CSG and CAG subscription when an enterprise has both EPS Femto cell and 5GS Femto cell.

* * * End of change * * * *

`