SA WG2 Meeting #160AHe	S2-2400753

E-meeting, January 22 – 29, 2024

Source: 	Samsung

Title: 	Key issue for WT1.3: Enhancements for DualSteer – Session Management Aspects

Document for: 	Approval

Agenda Item: 	19.13

Work Item / Release:	FS_MASSS / Rel-19

Abstract of the contribution: Propose key issue for WT1.3 on Enhancements for DualSteer in Session Management aspects.

1 Discussion

This key issue is related to WT1.3 as below.

WT#1: Study the overall architecture and function enhancements to 5GS to support a DualSteer Device (see TS 22.261 for definition of DualSteer Device). A DualSteer Device supports traffic steering and switching of user data (for different services) across two 3GPP access networks; it can be (a) a single UE, in case of non-simultaneous data transmission over the two networks, or (b) two separate UEs in case of simultaneous data transmission over the two networks. The subscriber of the DualSteer Device has two subscriptions/SUPIs, sharing one subscription profile from the same operator. For any particular service, at any given time, the DualSteer Device shall transmit all traffic of that service using only a single 3GPP access network.

The following scenarios are considered:

1.	Two NR/5GC accesses in a single PLMN (HPLMN or VPLMN) with each access being NR TN or NR NTN;

2.	Two NR/5GC accesses in two different PLMNs (including two VPLMNs or a VPLMN and the HPLMN) with each access being NR TN or NR NTN;

3.	NR/5GC access and E-UTRA/EPC access in two different PLMNs (including two VPLMNs or a VPLMN and the HPLMN);

4.	NR/5GC access and E-UTRA/EPC access in a single PLMN (HPLMN or VPLMN);

5.	PNI-NPN (integrated with the HPLMN or integrated with the VPLMN) and PLMN access (TN/NTN plus TN or NTN). This scenario assumes only non-simultaneous transmission.

NOTE 2: 	The study assumes there is no coordination in RAN between the two 3GPP access networks where the DualSteer Device is accessing simultaneously.

NOTE 3:	For the PNI-NPN scenario, the subscriber is assumed to be a subscriber of the PNI-NPN.

NOTE 4:	The 5GC-EPC scenarios will be studied once the baseline 5GC-5GC scenarios are stable.

NOTE 5:	Each subscription / SUPI of the DualSteer Device is used to connect to only one of the 3GPP access at any given point in time.

To support network enhancements for DualSteer Device, the following issues need to be considered:

-	WT#1.3: Session management enhancements and policies:

-	Study whether and how to enhance session management procedures for initial steering and potential subsequent switching. Traffic policies are in full control of the HPLMN. For sessions subject to potential switching, assuming data anchoring in the HPLMN or, in case of PNI-NPN plus PLMN, assuming data anchoring in the PNI-NPN, study how to select the PSA UPF(s) in the HPLMN to allow routing the traffic across 3GPP accesses towards the same PSA UPF during the switching.

2 Proposal

It is proposed to update TR 23.700-54 as follows.

# * * * * First change * * * *(all new texts)

5.1.X	Key Issue #X: Enhancements for DualSteer – Session Management Aspects

5.1.X.1	Description

This key issue studies whether and how to enhance overall architecture and function enhancements to 5GS to support a DualSteer Device.

The objective of this Key Issue is to study whether and how to enhance session management procedures for initial steering and potential subsequent switching. Traffic policies are in full control of the HPLMN. For sessions subject to potential switching, assuming data anchoring in the HPLMN or, in case of PNI-NPN plus PLMN, assuming data anchoring in the PNI-NPN, study how to select the PSA UPF(s) in the HPLMN to allow routing the traffic across 3GPP accesses towards the same PSA UPF during the switching.

In particular, the key issue includes enhancements for each of the following scenarios:

1.	Two NR/5GC accesses in a single PLMN (HPLMN or VPLMN) with each access being NR TN or NR NTN;

2.	Two NR/5GC accesses in two different PLMNs (including two VPLMNs or a VPLMN and the HPLMN) with each access being NR TN or NR NTN;

3.	NR/5GC access and E-UTRA/EPC access in two different PLMNs (including two VPLMNs or a VPLMN and the HPLMN);

4.	NR/5GC access and E-UTRA/EPC access in a single PLMN (HPLMN or VPLMN);

5.	PNI-NPN (integrated with the HPLMN or integrated with the VPLMN) and PLMN access (TN/NTN plus TN or NTN). This scenario assumes only non-simultaneous transmission.

# * * * * End of change * * * *