3GPP SA WG2 Meeting #160AHE	S2-2400757

22 – 29 January, 2024, Electronic

Source:	NEC

Title:	KI#3, New Sol: NF/NF service discovery for network energy saving and efficiency

Document for:	Approval

Agenda Item:	19.4

Work Item / Release:	FS_EnergySys / Rel-19

Abstract of the contribution: The contribution proposes a solution to Key Issue #3: 5GS enhancements for network energy saving and efficiency in TR 23.700-66.

1. Introduction

This contribution proposes a solution to Key Issue #3: 5GS enhancements for network energy saving and efficiency. It focuses on NF selection/re-selection related functionalities considering energy saving and energy efficiency based on e.g., NF energy states, analytics, and energy related information.

‐	Whether and how to enhance the NF selection/re-selection related functionalities considering energy saving and energy efficiency based on e.g., NF energy states, analytics, and energy related information.

2. Proposal

It is proposed to agree the following solution to 3GPP TR 23.700-66.

* * * Start of 1st change * * * *

6	Solutions

Editor's note:	This clause is intended to document the agreed solutions. Each solution should clearly describe which of the key issues it covers and how.

6.0	Mapping of Solutions to Key Issues

Editor's note:	This clause describes the mapping between solutions and key issues.

Table 6.0-1: Mapping of Solutions to Key Issues

| Solutions | Key Issues |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | … | X |
| 1 |  |  |  |  |  |
| 2 |  |  |  |  |  |
| … |  |  |  |  |  |
| X |  |  | X |  |  |

* * * End of 1st change * * * *

* * * Start of 2nd change * * * *

6.x	Solution #X:  NF/NF service discovery for network energy saving and efficiency

6.x.1	Key Issue mapping

The solution applies to Key Issue #3: 5GS enhancements for network energy saving and efficiency.

6.x.2	Functional Description

Assumptions:

- As described in the clause 4.17.1, TS23.502 [2], NF profile is configured by OAM system.

- It is assumed that the NF profile includes a new information about energy savings and efficiency.

6.x.3	Procedures

Figure 6.X.3.1 NF discovery and selection

- As described in the clause 4.17.1, TS23.502 [2], NF profile is configured by OAM system. The target NF registes the NF profile via Nnrf_NFManagement_NFRegister service.

- A NF service consumer intends to discover services available in the network based on service name and target NF type. The NF service consumer invokes Nnrf_NFDiscovery_Request from an appropriate configured NRF.

For NF discovery to determine a set of NF instance(s) with a feature to execute the NF with lower energy consumptions, the parameter may include optionally a new information about energy efficiency mode (e.g. energy efficient mode, non-energy efficient mode). In addition, the parameter may include expected energy efficiency (e.g. 10% energy efficiency). The parameter is uniequely identified by identifier, name, etc.

For NF discovery (e.g. by AMF) to determine a set of SMF instance(s) with a feature to select a set of UPF(s) based on energy savings and efficiency, the parameter may include optionally a new information about UPF selection model (e.g. current or predicted energy savings and efficiency model). The parameter is uniequely identified by identifier, name, etc.

For NF discovery to determine a set of NWDAF instance(s) with a feature to analyse energy savings and efficiency, the parameter may include optionally a new information about types of energy efficiency analytics (e.g. current or predicted energy savings and efficiency). The parameter is uniequely identified by identifier, name, etc.

- The NRF discovers a set of NF instance(s), matching with the queried parameters about energy savings and efficiency mode in the Nnrf_NFDiscovery_Request.

- The NRF reponds to the NF consumer with the NF profile(s) of the determined NF instances.

For NF discovery to determine a set of NF instance(s) with a feature to execute the NF with lower energy consumptions, the NF profile may support information about energy efficiency mode (e.g. energy efficient mode, non-energy efficient mode) supported by the target NF.

For NF discovery to determine a set of SMF instance(s) with a feature to select a set of UPF(s) based on energy savings and efficiency, the NF profile may support information about UPF selection model (e.g. current or predicted energy savings and efficiency) supported by the target SMF.

For NF discovery to determine a set of NWDAF instance(s) with a feature to analyse energy savings and efficiency, the NF profile may support information about types of energy efficiency analytics (e.g. current or predicted energy savings and efficiency) supported by the target NWDAF.

- The NF consumer selects NF instances from the list of the response.

6.x.4	Impacts on existing services, entities and interfaces

NRF – a new NF/NF service discovery with new parameters such as energy efficient mode, energy efficiency analytics, UPF selection based on energy efficiency. NF profile supports information about the new parameters.

* * * End of 2nd change * * * *