3GPP TSG-SA WG2 Meeting #S2-160AH-e	S2-2400771

22  – 29 January, 2024, Electronic meeting

Source: 	Samsung

Title: 	Solution for KI#1: Admission control of QoS flow based on PDU Set QoS parameters

Document for: 	Approval

Agenda Item: 	19.3

Work Item / Release:	FS_XRM_Ph2 / Rel-19

Abstract of the contribution: This contribution proposes a new solution for admission control of QoS flow based on PDU Set parameters (related to KI #1).

- Introduction

Per KI #1, we study PDU set based QoS handling enhancements considering both control plane and user plane perspectives:

-	whether, what and how PDU Set based handling (e.g. new standardized 5QI, enhancements to Alternative QoS profiles, FEC, etc.) and PDU Set information (including Control Plane and/or User plane information) provided by the AF/AS are enhanced.

Application may provide some service requirements (e.g. PDU Set QoS parameters) in 5GS, but in case when NG-RAN expect it could not meet the service requirement for a QoS flow during PDU Session establishment or modification or handover, NG-RAN may perform admission control for the QoS flow(e.g. a QoS flow setup or handover to target RAN is rejected or accepted, or PDU Set QoS parameters are modified(e.g. directly or via alternative PDU Set QoS parameters), or QoS flow is accepted but applying PDU Set handling). Moreover, according to application’s characteristics the application may have preference among the above admission control.

So such preference can be provided to the 5GS and if it is expected that some service requirements do not meet so that QoS flow handling is changed by preference( or policy), then such result of admission control should be notified to application server, which may perform some sequential update e.g. codec downgrade.

2 Proposal

It is proposed to add the following contents to TR 23.700-70.

# * * * * First change * * * *

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

| Solutions |  |  |
| --- | --- | --- |
|  | <Key Issue #1> | <Key Issue #2> |
| #xx | O |  |
| #2 |  |  |

# * * * * 2nd change(new text) * * * *

6.X	Solution #X: <Admission control of QoS flow based on PDU Set QoS parameters>

6.X.1	Key Issue mapping

Editor's note:	This clause lists the key issue(s) addressed by this solution.

The solution addresses Key Issue #1(e.g. QoS handling modification based on PDU Set QoS parameters and admission control policy from AF).

6.X.2	Description

Editor's note:	This clause will describe the solution principles and architecture assumptions for corresponding key issue(s). Sub-clause(s) may be added to capture details.

AF may provide some service requirement(e.g. PDU Set QoS requirement) for a QoS flow to 5GS. Moreover for the case that expectedly the service requirement can not meet sufficiently, AF may provide its preference or admission control policy on how to handle the QoS flow e.g. modify some PDU Set QoS parameters(e.g. via alternative QoS profile including alternative PDU Set QoS parameters) or reject the QoS flow or accept the QoS flow not applying PDU Set handling etc.

The above admission control policy and the alternative QoS profile including alternative PDU Set QoS parameters may be provisioned to PCF and SMF and to NG-RAN.

So during PDU Session establishment or PDU Session modification procedure or handover, NG-RAN may perform admission control for the QoS flow(e.g. a QoS flow setup or handover is rejected or accepted, or PDU Set QoS parameters are modified(e.g. directly or via alternative PDU Set QoS parameters), or QoS flow is accepted but applying PDU Set handling), where the admission control policy follows the above AF’s input. Then, NG-RAN report results of admission control to SMF and SMF notifies such changed QoS handling situation to application server via PCF, so that application server may perform some sequential update e.g. codec downgrade in accordance with the notification from 5GS.

6.X.3	Procedures

Editor's note:	This clause describes high-level procedures and information flows for the solution.

6.X.4	Impacts on services, entities and interfaces

Editor's note:	This clause captures impacts on existing 3GPP nodes and functional elements.

AF:

-	provides admission control policy together with PDU Set QoS parameters to PCF via NEF.

-  provides alternative QoS profile including alternative PDU Set QoS parameters to PCF via NEF.

SMF:

-	provides NG-RAN with admission control policy together with PDU Set QoS parameters.

-	provides NG-RAN with alternative QoS profile including alternative PDU Set QoS parameters.

-  notify PCF based on the NG-RAN’s report and the admission control policy

NG-RAN:

-	perform modification on QoS flow handling as the admission control policy during PDU Session modification and or handover procedure, and send a report to SMF if it can not meet the requested PDU Set QoS parameters.

# * * * * End of changes * * * *