SA WG2 Meeting #S2-160AHE 	S2-2400852

# 22 - 29 January, 2024, Electronic		(revision )

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0492 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to SL-MO-LR and SL-MT-LR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Ranging_SL |  |  |  |  |  | Date: |  |  | 2024-1-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | It is common understanding that supplementary service message is used to delivery the info(e.g., the capability, assistant data and Location info) of the UE2-n with supplementary service message embedded SLPP container(s). For the info of UE1 itself, it still uses the SLPP, this principle should be reused on the Uu. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that: - the info(e.g., the capability, assistant data and Location info) of the UE2-n is gathered to UE1 and is sent by UE1 to LMF using the supplementary service message embedded SLPP container(s). - the info of UE1 itself is sent by UE1 using the SLPP |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not need to support SLPP between UE1 and LMF, is not aligned with common understanding before. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.20.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 6.20.1	Procedures of SL-MO-LR involving LMF

Figure 6.20.1-1 illustrates a procedure to enable a UE to obtain Ranging/Sidelink Positioning location results using one or more other UEs with the assistance of an LMF in a serving PLMN for UE1.

The Ranging/Sidelink Positioning location results may include absolute locations, relative locations, i.e. distances and/or directions from other UEs, velocities and relative velocities, depending on the service request.

If the Target UE decides to initiate SL-MO-LR procedure, it includes one or multiple SL reference UE(s) / Located UE (s) in the service request. See TS 23.586 [40] for more information on how this generic procedure can be used.

Figure 6.20.1-1: SL-MO-LR Procedure

Precondition:	UE1 is in coverage and registered with a serving PLMN that supports Ranging/Sidelink Positioning. UEs 2 to n may or may not be in coverage and, if in coverage and are registered in the same serving PLMN as UE1 as specified in clause 5.5.1 of TS 23.586 [40].

In this procedure, the information in the SLPP messages from UE 2 to n received by UE-1 is included in the SLPP container in the supplementary service messages from UE-1 to LMF. For this case, the supplementary service messages are used in the following steps 8, 9, 10, 11, 13, 14, 15, 19 and 23.

1.	The procedures and signalling specified in clause 6.2 of TS 23.586 [40] are used to provision the Ranging/Sidelink positioning service authorization and policy/parameter provisioning to UEs 1 to n.

2.	Based on a trigger of service request (e.g. received from the application layer), which includes UE1/.../UEn, UE discovery is performed for Ranging/Sidelink positioning as specified in clause 6.4 of TS 23.586 [40]:

-	If UE1 is the target UE, UE1 discovers UEs 2 to n.

3.	Secure groupcast and/or unicast links are established between UEs 1 to n to enable UE1 to exchange Ranging and Sidelink Positioning Protocol (RSPP) messages over PC5-U reference point with each of UEs 2 to n and possibly enabling UEs 2 to n to exchange RSPP over PC5-U between each other as defined in clause 5.3 of TS 23.586 [40]. Groupcast and/or unicast links are only established between UEs 1 to n that are registered in the same PLMN.

Security mechanisms defined for V2X unicast mode communication in TS 33.536 [44] and for 5G ProSe unicast mode Direct Communication in TS 33.503 [45] are reused. The Security procedures for RSPP signalling using groupcast mode is specified in TS 33.533 [46].

4.	UE1 and UEs 2 to n may communicate over PC5 for authorization of Ranging/Sidelink positioning. Each of UEs verifies that Ranging/Sidelink positioning is permitted, including whether Ranging/SL positioning results may be transferred to an LCS Client or AF if this is used, according to any service authorization and policy/parameter provisioning received at step 1. This step is specified in TS 33.533 [46].

5.	The Ranging/Sidelink positioning capabilities exchange between UE-1 and UEs 2 to n is performed using SLPP as specified in TS 38.355 [47] via the groupcast and/or unicast links established in step 3.

Step 4 and 5 may be performed to transfer the information of UEs which are not served by the LMF.

NOTE 1:	UE2/.../UEn is not assumed to be served by the same LMF serving UE1.

6.	Based on the Ranging/Sidelink positioning capabilities of UE1/.../UEn, the target UE determines SL-MO-LR is to be performed.

7.	If UE1 is in CM-IDLE state, UE1 instigates a UE triggered Service Request in order to establish a signalling connection with the serving AMF of UE1.

8.	UE1 sends a supplementary services SL-MO-LR request message to LMF via the serving AMF in an UL NAS TRANSPORT message. The SL-MO-LR request indicates the other UEs 2 to n (using Application Layer ID), indicates any assistance data needed, indicates whether location calculation assistance is needed, and indicates whether location results should be transferred to an LCS client or AF. The message may include the identity of the LCS client or the AF and may include the address of the GMLC through which the LCS client or AF (via NEF) should be accessed. In addition, a Service Type indicates which MO-LR service of the LCS Client is requested by the UE may be included. For location calculation assistance from the LMF, the preferred type of Ranging/Sidelink positioning location results (e.g. absolute locations, relative locations or distances and directions between pairs of UEs, velocities and relative velocities) and the required QoS are included.

9.	The serving AMF selects an LMF serving UE1 (e.g. an LMF that supports Ranging/Sidelink positioning) and sends an Nlmf_Location_DetermineLocation service operation towards the LMF with the information from the SL-MO-LR Request. The service operation includes a LCS Correlation identifier. AMF may include its stored information from step 21, the sidelink positioning capabilities of UE1 in the service operation.

10.	The LMF may send a request to UE1 for the required capabilities of UEs 2 to n using supplementary service message with embedded SLPP container(s), and LMF sends SLPP message to request UE1 capabilities as specified in TS 38.355 [47].

NOTE 2:	UE2/.../UEn is not assumed to be served by the same LMF serving UE1.

11.	UE 1 returns its capabilities to the LMF using SLPP message and the capabilities of UE2 to n using the supplementary service message with embedded SLPP container(s) as specified in TS 38.355 [47] if requested by the LMF at step 10. UE1 requests the capabilities of UE2 to n sepretely by SLPP over PC5 if step5 is not performed. After checking the capabilities of the UEs, LMF may downselect the UEs (so called, down-selected list of UEs) for SL positioning operation.

NOTE 3:	The embedded SLPP container(s) can be differentiated by the correlated Application layer ID(s) included in the supplementary service message.

12.	If Target UE's absolute location information is required at step 8, LMF can either retrieved the location of the Located UE(s) locally if available or triggers 5GC-MT-LR procedure to the GMLC to acquire the absolute location of the Located UE(s) using Application Layer ID of the Located UE(s). LMF may use the QoS requirement for Target UE's positioning received at step 8 to derive the required QoS for Located UE(s) positioning and includes the required QoS for Located UE positioning in the request to GMLC. If scheduled location time is used in step 15, LMF includes the same scheduled location time in the request to GMLC.

13.	UE1 may send a request for specific assistance data to the LMF, if not requested in step 8.

-	For the specific assistance data used by UE1, the request is transmitted by SLPP message.

-	For the specific assistance data used by UE2/…/UEn, the request is transmitted by supplementary service message with embedded SLPP container(s).

14.	LMF sends the requested assistance data used by UE1 using the SLPP message, and the assistance data used by UE2/…/UEn and optionally a down-selected list of UEs using supplementary service message with embedded SLPP container(s) as specified in TS 38.355 [47], and UE1 sends the assistance data received from LMF to UE2/.../UEn (or the indicated downselection thereof) using SLPP as specified in TS 38.355 [47]. The assistance data may assist UEs 1 to n (or the indicated downselection thereof) to obtain Sidelink location measurements at step 16 and/or may assist UE1 to calculate Ranging/Sidelink positioning location results at step 18 and may include the absolute location of Located UE(s) in case the LMF determines to use UE based calculation and if absolute location information is requested in step 8 and if sharing the location is allowed by the Located UE(s) privacy profile.

NOTE 4:	Steps 10 and 11 can be omitted if UE1 includes a message containing the capabilities of UEs 1 to n in the SL-MO-LR request at step 8 and step 9. Step 13 can be omitted if UE1 includes a message containing the request for specific assistance data in the SL-MO-LR request at step 8.

15.	If the SL-MO-LR request at step 8 indicated location calculation assistance is needed and/or indicated transfer of Ranging/Sidelink positioning location results to an LCS Client or AF, the LMF sends requests to UE1for location information of UE1 by SLPP message and location information of UE2-UEn by supplementary service message. If LMF determines to apply UE based Sidelink Positioning, LMF includes in the request the indication of UE based Sidelink Positioning. LMF may also provide the list of candidate Located UE(s), identified by the Application Layer ID by supplementary service message to UE1, if absolute location is requested at step 8. If scheduled location time is not received at step 9. LMF may generate a scheduled location time, e.g. based on response time, and include the scheduled location time in the request.

16.	UE1 performs a Ranging/Sidelink positioning procedure among UEs 1 to n (or the indicated downselection thereof) in which UEs obtain Sidelink location measurements and UEs 2 to n (or the indicated downselection thereof) transfer their Sidelink location measurements to UE 1. If scheduled location time is received at step 15, Sidelink positioning/ranging is performed at the scheduled location time. This procedure is specified in TS 38.355 [47].

17. For the case of UE based SL Positioning (i.e., the indication of UE based SL Positioning is received in step 15), If Target UE's absolute location information is required at step 8 and if absolute location of Located UE(s) is not available, the Target UE may send a request to the Located UE(s) to trigger 5GC-MO-LR procedure to let the Located UE(s) acquire their own absolute location, after which a Located UE may provide the location of the Located UE to the Target UE if allowed by its privacy profile. The QoS requirement for Target UE's positioning, is used by the Target UE to derive the required QoS for Located UE(s) positioning. The required QoS for Located UE(s) positioning is included in the request.

18.	If LMF determined in step 15 to use UE based calculation, at least one of UE1/.../UEn calculates Ranging/Sidelink positioning location results based on the Sidelink location measurements obtained at step 16 and possibly using assistance data received at step 14 as specified in TS 38.355 [47]. The Ranging/Sidelink positioning location results can include absolute locations, relative locations i.e. distance and/or directions related to the UEs.

19.	If UE1 received requests for location information at step 15, UE1 sends responses of its own location information by SLPP and the location information of UE2-n using supplementary service message with embedded SLPP container(s) as specified in TS 38.355 [47] to the LMF and includes the Sidelink location measurements obtained at step 16 and the Ranging/Sidelink positioning location results obtained at step 18 if step 18 was performed. In the response message, UEs 2 to n (or the indicated downselection thereof) are identified by its Application Layer ID.

20.	The LMF calculates Ranging/Sidelink positioning location results for the target UE based on the Sidelink location measurements received at step 19 and absolute location of Located UE(s) at step 12 or step 17. The Ranging/Sidelink positioning location results can include absolute locations, relative locations i.e. distance and/or directions related to the UEs, depending on the location request received in step 8.

21.	If location results is requested to be transferred to an LCS client or AF in step 8, the LMF returns an Nlmf_Location_DetermineLocation service operation response to the AMF and includes the Ranging/Sidelink positioning location results received at step 19 or calculated at step 20. The service operation may also contain the UE1's sidelink positioning capabilities if the capabilities are received in step 11 including an indication that the capabilities are non-variable and not received from AMF in step 9.

22.	If Ranging/Sidelink positioning location results were received at step 21, the AMF performs steps 7-12 of clause 6.2 to send the Ranging/Sidelink positioning location results to the GMLC and to an AF or LCS Client if this was requested at step 8. The Ranging/Sidelink positioning location results include the identities for the respective UEs received at step 8. The AMF may store the UE1's sidelink positioning capabilities from step 21.

NOTE 5:	Sending location results and global identities for UEs to an AF or LCS Client may require privacy verification from UEs and/or from the HPLMNs of UEs.

23.	The LMF returns a supplementary services SL-MO-LR response to UE1 in a DL NAS TRANSPORT message and includes any Ranging/Sidelink positioning location results calculated at step 20 if step 20 was performed.

# * * * * End of changes * * * *