SA WG2 Meeting #160-Ad Hoc-e	S2-2400896

E-meeting, January 22 – 29, 2024                  	(revision of S2-24xxxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1384 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update on UE Centric AR communication Procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC |  |  |  |  |  | Date: |  |  | 2024-01-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In Figure AC.9.3.1-1 Establishment of UE Centric AR communication session, the step 4 and 5 are described as: 4. Send rendered Audio/Video data over RTP 5. Send AR data to the UE-A While terms “data” and “media” are different in the context of AR communication. It is proposed to revise “data” as “media” to align with the procedure description; and some clarification is given in the procedure description. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed to revise “data” as “media” in step 4&5 in Figure AC.9.3.1-1 Establishment of UE Centric AR communication session; and some clarification is given in the procedure description. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The steps in Figure AC.9.3.1-1 Establishment of UE Centric AR communication session are not aligned with the procedure description. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | AC.9.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

AC.9.3.1	UE Centric Procedure

Figure AC.9.3.1-1: Establishment of UE Centric AR communication session

Figure AC.9.3.1-1 depicts a typical call flow procedure to establish a UE centric AR IMS session from UE-A perspective. The main steps in the call flow are as follows:

1.	UE-A initiates an IMS communication with UE-B, including establishment of bootstrap data channel.

2.	The user of UE-A upgrades the IMS session with AR experience.

UE-A initiates a re-INVITE adding media descriptors required by the AR application to be established E2E, e.g. RTP and/or application data channels. Application data channel may be anchored in the MF/MRF.

In this scenario, the UE performs AR media rendering based on the AR data from its local application.

NOTE:	In other scenarios, the UE may acquire AR data from other sources, e.g. the peer UE, the AR AS etc.

3.	UE-A captures the AR data, performs AR media rendering locally and then encodes AR media e.g. audio/video media stream.

4.	UE-A sends the AR media (rendered Audio/Video media) to the peer through the established media connection(s).

5.	UE-A receives the AR media (unrendered Audio/Video media) from the peer through established media connection(s).

6.	UE-A decodes the AR media and performs AR media rendering locally and displays it on its screen.

END OF CHANGES