SA WG2 Meeting #160-Ad Hoc-e	S2-2400899

E-meeting, January 22 – 29, 2024	(revision of S2-24xxxxx)

Source:	ZTE

Title:	KI#6: Update on Standalone DC Negotiation

Document for:	Approval

Agenda Item:	19.2

Work Item / Release:	FS_NG_RTC_Ph2 / Rel-19

Abstract of the contribution: It is proposed to update the description of KI#6 to include standalone IMS data channel negotiation between two UEs.

1. Discussion

For P2P data channel session, the data channel media may exchange between two UEs without anchoring at MF/MRF. So for P2P scenario, the standalone IMS data channel negotiation between two UEs is also needed.

It is proposed to add standalone IMS data channel negotiation between two UEs in KI#6.

2. Proposal

It is proposed to capture the following changes into TR 23.700-77.

* * * * Start of Changes * * * *

## 5.6	Key Issue #6: Support of Standalone IMS Data Channel Sessions

### 5.6.1	Description

This key issue aims to study enhancements of IMS architecture, interfaces, and procedures to support standalone IMS data channel session. This includes studying following aspects:

-	How to support standalone IMS bootstrap and application data channel sessions without accompanying audio/video/messaging media in an IMS session. This includes studying the establishment and termination of standalone IMS data channel sessions.

-	Study whether and how to negotiate support of standalone IMS data channel between UE and IMS network, and between two UEs. The negotiation should include the ability to add audio/video/messaging media to an established standalone IMS data channel.

-	Whether and how to establish an (standalone) application data channel without accompanying bootstrap data channel.

-	Whether and how to establish a (standalone) bootstrap data channel without accompanying application data channel.

-	How to add audio/video/messaging media to an established standalone IMS data channel session.

-	How to remove audio/video/messaging media from an IMS session that also contains IMS data channel media.

-	Whether and how to define new service data in the HSS for support of standalone data channel or extend existing MMTel service data.

* * * * End of Changes * * * *