SA WG2 Meeting #160-Ad Hoc-e	S2-2400904

E-meeting, January 22 – 29, 2024	(revision of S2-24xxxxx)

Source:	ZTE

Title:	New KI: Support Ambient IoT Services and Exposure

Document for:	Approval

Agenda Item:	19.14

Work Item / Release:	FS_AmbientIoT / Rel-19

Abstract of the contribution: A new key issue of supporting Ambient IoT services is proposed.

1. Discussion

In the SID of FS_AmbientIoT, the following work task is introduced:

- WT#3 Ambient IoT Services

  - WT#3.1 Study how to support information transfer for Ambient IoT services and related system functionality.

NOTE 7:	including whether there is a need to support session based transfer between Ambient IoT device and the network considering the device types and capabilities.

  - WT#3.2 Study which Ambient IoT services enabled in WT#3.1 to be exposed to AF and how.

A new key issue of supporting Ambient IoT services is proposed to address WT#3.

2. Proposal

It is proposed to capture the following changes into TR 23.700-13.

* * * * Start of Changes * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[x]	3GPP TS 22.369: "Service requirements for ambient power-enabled IoT".

[y]	3GPP TR 38.848: "Study on Ambient IoT (Internet of Things) in RAN".

* * * * Next Change * * * *

5.X	Key Issue #X: Ambient IoT Services and Exposure

5.X.1	Description

The Key Issue aims to study how to support Ambient IoT services and the service exposure in following aspects:

-	Study which Ambient IoT services in TS 22.369 and TR 38.848 will be supported in this release.

-	How to support information/data transfer for different Ambient IoT services.

-	Study the impact on existing network functionalities and whether new network functionalities are needed to support Ambient IoT services.

-	Study the procedures of information/data transfer for different Ambient IoT services.

-	Study whether session based traffic transfer between Ambient IoT device and the network is needed; and if so, study the session based transfer procedures.

-	Study which Ambient IoT services can be exposed to AF.

-	How to expose Ambient IoT device information and IoT data to AF and related procedures.

* * * * End of Changes * * * *