3GPP TSG-SA2 Meeting #160-Ad Hoc-e	S2-2400917

# Online,  22nd Jan 2024 - 29th Jan 2024

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5271 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Network slice usage control in case of emergency call – alt2-1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNS_Ph3 |  |  |  |  |  | Date: |  |  | 2024-01-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The network slice usage control in case of emergency call is not clear. The objective is to (be able to) exclude the emergency service from network slice usage control. Alt1 (UE impact) UE shall not indicate the support of UE configuration of network-controlled Slice Usage Policy during emergency registration procedure. >> UE not indicating the support of the feature means that neither UE nor NW run the deregistration inactivity timer with the understanding that condition of NW running deregistration inactivity timer depends on the network slice usage control supported by the UE(reception of support indication from the UE). Also, AMF will not configure the Slice Usage Policy to the UE; hence objective achieved. Alt2 (NW impact) AMF shall neither configure network-controlled Slice Usage Policy to the UE nor run the deregistration inactivity timer(not running the timer means not monitoring e.g., inactvity of the PDU session if for emergency service) for emergency registered network slice. >> No slice usage policy configured to the UE and AMF not running the deregistration inactivity timer; hence objective achieved. With above, it is proposed to discuss and agree the way forward either Alt1 or Alt2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This propose Alt2 (NW impact) where the NW shall not monitor the usage of network slice if the network slice is associated with emergency service. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unexpectedly a user is deregistered from emergency service. Unclear network slice usage control requirement in case of emergency call will lead to implementation fragmentation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.15.15.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Change * * *

#### 5.15.15.1	General

Network Slice usage control is achieved as follows:

1)	Configuring network-controlled Slice Usage Policy to supporting UEs (see clause 5.15.15.2).

2)	Configuring PDU Sessions inactivity timers, and Network Slice deregistration inactivity timers (see clause 5.15.15.3).

NOTE:	Roaming is not supported in this Release of the specification.

Subject to operator policy or national/regional regulations, the AMF exempts Network Slice from Network Slice usage monitoring when the Network Slice is used for Emergency service.

When the AMF receives a Registration Request for an Emergency Registration, if operator policy or national/regional regulations does not allow Network Slice usage control for the Network Slice is used for Emergency service, the AMF shall not perform Network Slice usage control described above.

* * * End of Change * * *