SA WG2 Meeting #160-Ad-Hoc-e	S2-2401001

# January 22th – 29th, 2024; Elbonia               		       (revision of S2-24xxxxx)

Source: 	OPPO

Title: 	23.700-54: Key issue X: Subscription enhancement for dual steer

Document for: 	Approval

Agenda Item: 	19.13

Work Item / Release:	FS_MASSS / Rel-19

Abstract of the contribution: This paper proposes to add key issue on Subscription enhancement for dual steer to the FS_MASSS.

1. Discussion

The WT-D 1.1 of FS_MASSS SID includes the following:

WT#1.1: Study whether and how to enhance subscription aspects for each of the above scenarios.

Besides, a DualSteer Device is defined using the following descriptions:

“A DualSteer Device supports traffic steering and switching of user data (for different services) across two 3GPP access networks; it can be (a) a single UE, in case of non-simultaneous data transmission over the two networks, or (b) two separate UEs in case of simultaneous data transmission over the two networks. The subscriber of the DualSteer Device has two subscriptions/SUPIs, sharing one subscription profile from the same operator.”

The dual steer scenario should include single PLMN cases and two PLMN cases. Thus intra PLMN interaction for single PLMN and inter PLMN interaction for two PLMN at the network side to verify and correlate two subscriptions/SUPIs should be studied.

Whether and how to enhance the subscription profile at the UE side and network side to correlate and verify the two SUPIs to support dual steer should be studied as a KI.

Thus, it is proposed to agree on the following Key issue description in the TR of FS_MASSS.

2. Proposal

Start of Changes (all new text below)

## 5.1.x	Key Issue #x: Subscription enhancement for dual steer

### 5.1.x.1	Description

The subscriber of the DualSteer Device is assumed to have two subscriptions/SUPIs, sharing one subscription profile from the same operator. And the DualSteer Device can be (a) a single UE (equipped with two USIM cards), in case of non-simultaneous data transmission over the two networks, or (b) two separate UEs (belonging to the same DualSteer Device) in case of simultaneous data transmission over the two networks. This Key Issue #x aims to study:

-	whether and how to define/apply subscription profile to the DualSteer Device side with two SUPIs.

-	whether and how to apply subscription profile to the network side to verify the two subscriptions/SUPIs of the same DualSteer Device.

-  whether and how to provision the correlation of the two subscriptions/SUPIs from the HPLMN to the other PLMN for two PLMN scenarios.

End of Changes