SA WG2 Meeting #160-Ad Hoc-e	S2-2401004

# E-meeting, January 22 – 29, 2024

Source: 	OPPO

Title: 	23.700-54: New KI for NTN+TN and NTN+NTN scenarios

Document for: 	Approval

Agenda Item: 	19.13

Work Item / Release:	FS_MASSS / Rel-19

Abstract of the contribution: This contribution proposes a new Key Issue to study the potential enhancements in the case of NTN+TN/NTN for FS_MASSS

1	Discussion

The New SID on Multi-Access was agreed in SP-231802. The study on DualSteer aims to consider the following scenarios:

1.	Two NR/5GC accesses in a single PLMN (HPLMN or VPLMN) with each access being NR TN or NR NTN;

2.	Two NR/5GC accesses in two different PLMNs (including two VPLMNs or a VPLMN and the HPLMN) with each access being NR TN or NR NTN;

3.	NR/5GC access and E-UTRA/EPC access in two different PLMNs (including two VPLMNs or a VPLMN and the HPLMN);

4.	NR/5GC access and E-UTRA/EPC access in a single PLMN (HPLMN or VPLMN);

5.	PNI-NPN (integrated with the HPLMN or integrated with the VPLMN) and PLMN access (TN/NTN plus TN or NTN). This scenario assumes only non-simultaneous transmission.

As shown above, a mix of terrestrial and non-terrestrial NR, as well as dual non-terrestrial NR access (using same or different NTN orbits, e.g., GEO/MEO/LEO) will be studied. This paper aims to identify the potential enhancements specific to the NTN+TN/NTN scenarios, in addition to the enhancements to other DualSteer scenarios.

2	Proposal

Approve the proposed Architecture Assumptions and Requirements for TR 23.700-54 as follows.

Start of Changes

## 5.1.x	Key Issue #x: Additional enhancement to support NTN related scenarios

### 5.1.x.1	Description

To support traffic steering and/or switching between a mix of terrestrial and non-terrestrial NR, as well as dual non-terrestrial NR access, the following issues are proposed to be studied:

- Whether and what enhancements to 5GS are needed specific to the NTN +NTN/TN scenarios, in addition to the enhancements for other scenarios.

- If one of the access is regenerative based NTN access, what are the minimum necessary set of core network elements/network functions that should be placed on board the satellite.

-  What is the potential architecture and function enhancements to support NTN + NTN/TN, if only gNB is place on board the satellite(s).

-	 What is the potential architecture and function enhancements to support NTN + NTN/TN, if gNB and UPF are place on board the satellite(s).

NOTE: Coordination with FS_5GSAT_ARCH_Ph3 SID may be needed.

End of Changes