3GPP TSG-WG SA2 Meeting #160-Ad Hoc-e	S2-2401013

Online, Jan 22 – 29, 2024	(revision of S2-240xxxx)

Source:	Huawei, HiSilicon

Title:	New Key Issues for 5G Femto

Document for:	Approval

Agenda Item:	19.12

Work Item / Release:	FS_5G_Femto / Rel-19

Abstract: This contribution would like to propose new key Issues for 5G Femto.

1. Introduction/Discussion

There are two WTs captured in the SID objective of FS_5G_Femto:

WT#2: How to enable interworking between CAG and CSG cells.

WT#3: Study whether and how to support enabling the provisioning of subscribers allowed to access CAG cell and to manage access control by the CAG owner or an authorized administrator.

This contribution proposes new Key Issues corresponding to these two work tasks.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-45.

# * * * * First change * * * *

5.x	Key Issue #x: Enabling interworking between CAG and CSG cells

5.x.1	Description

The 4G CSG mechanism is defined for HeNB access control. UE can only access the allowed CSG cells based on the subscription data. In order to enable the mobility between HeNB and HgNB, this key issue aims at studying how to enable interworking between CAG cell and CSG cell.

The corresponding solutions need to consider following assumptions:

-	Impacts on EPC (e.g. MME), E-UTRAN and NG-RAN shall be avoided.

This key issue aims to address the following aspects:

-	How to enhance the Mobility management to enable the UE mobility from a CAG cell to a CSG cell and vice versa.

# * * * * Second change * * * *

5.y	Key Issue #y: Enabling provisioning of subscribers allowed to access CAG cell

5.y.1	Description

The 5G NR Femto re-uses existing CAG mechanism defined for PNI-NPN for access control. In order to add flexibility to the 5G NR Femto, the owner of 5G NR Femto needs to be able to control which UE(s) can access to the HgNB.

NOTE X: Ownership of the 5G NR Femto (or CAG or both) concept and a mechanism will be defined in coordination with SA3.

This key issue aims to address the following aspects:

-	Whether and how can the CAG owner or an authorized administrator to enable provisioning CAG information to the network for 5G Femto only.

-	Whether and how to provide the CAG information to the UE to enable the UE to access to a CAG cell of a 5G Femto.

-	Whether and how can the CAG owner or an authorized administrator update access control information of the CAG cells of a 5G Femto.

# * * * * End of changes * * * *