3GPP TSG-WG SA2 Meeting #160-Ad Hoc-e	S2-2401015

Online, Jan 22 – 29, 2024	(revision of S2-240xxxx)

Source:	Huawei, HiSilicon

Title:	Architectural assumptions and requirements for ATSSS_Ph4

Document for:	Approval

Agenda Item:	19.13

Work Item / Release:	FS_MASSS / Rel-19

Abstract: This pCR would like to propose the architectural assumptions and requirements for ATSSS_Ph4 part of FS_MASSS

1. Introduction/Discussion

Based on WT#2 and WT#3 in the SID objective of FS_MASSS, the architectural assumptions and requirements for ATSSS_Ph4 are proposed correspondingly.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-54.

# * * * * First change * * * *

4.1.2	Architectural Assumptions for ATSSS_Ph4

The following architectural assumptions are applied to the ATSSS_Ph4 aspect study:

For Key Issues related to MPQUIC functionality:

-	The study takes the Rel-18 ATSSS architecture (see TS 23.501 [x]) as a baseline.

-	The study shall not have RAN impact.

-	It is assumed that an MA PDU Session is required for supporting all ATSSS capabilities. Only ATSSS enhancements using an MA PDU Session are considered.

-	All ATSSS steering functionalities (existing and new) shall reside in the UE and in an UPF. Steering functionalities outside either of the UE or the UPF are not considered.

-	Any new steering functionality shall be based on the multipath extensions of the QUIC protocols as defined in IETF.

For Key Issue related to non-3GPP access simplification:

-	The solution proposed to KI#X (non-3GPP access simplification) is not applicable to 5G-RGs.

# * * * * Second change * * * *

4.2.2	Architectural Requirements for ATSSS_Ph4

The following architectural requirements are applied to the ATSSS_Ph4 aspect study:

For Key Issues related to MPQUIC functionality:

-	New ATSSS capabilities shall either be able to co-exist with the existing ATSSS capabilities or any dependencies between new and existing capabilities shall be explicitly defined.

For Key Issue related to non-3GPP access simplification:

-	The solutions proposed to KI#X (non-3GPP access simplification) will also be analyzed whether it can be applied to independent non-3GPP access connection or shall be applied with the use of ATSSS feature.

NOTE:	Co-ordination with BBF and CableLabs will take place as needed during the study.

# * * * * End of changes * * * *