3GPP TSG-WG SA2 Meeting #160-Ad Hoc-e	S2-2401018

Online, Jan 22 – 29, 2024	(revision of S2-240xxxx)

Source:	Huawei, HiSilicon

Title:	Key Issue for WT#3 of ATSSS_Ph4

Document for:	Approval

Agenda Item:	19.13

Work Item / Release:	FS_MASSS / Rel-19

Abstract: The pCR would like to propose a Key Issue corresponding to the WT#3 of ATSSS_Ph4 of FS_MASSS.

1. Introduction/Discussion

There is a WT captured in the SID objective of FS_MASSS:

WT#3: Study whether and how to define a functional architecture and procedures for steering, switching and splitting of traffic not based on current TNGF/N3IWF to simplify the operation over non-3GPP access without compromising the security of the 5G network.

WT#3.1: Study whether to keep the NAS signalling connection on non-3GPP access or not, whether to eliminate IPSec tunnel encapsulation on the user plane only or both on the control plane and the user plane, simplifying the protocol stack, reduce the user plane overhead.

WT#3.2: Study whether and how to support splitting, switching, steering between 3GPP access and "non-3GPP access without 5G NAS". Study whether and how to enhance registration and security aspects for supporting "non-3GPP access without 5G NAS".

Therefore, the corresponding key issue is proposed as below.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-54.

# * * * * First change * * * *

5.2.x	Key Issue #X: Non-3GPP access simplification

5.2.x.1 	Description

Currently, Access Traffic Steering, Switching and Splitting feature requires MA PDU Sessions over integrated trusted or untrusted non-3GPP access. This means that to enable ATSSS either a TNGF (Trusted Non-3GPP Gateway Function) or an N3IWF (Non-3GPP InterWorking Function) shall be deployed in the PLMN.

This key issue aims to simplify the operation over non-3GPP access without compromising the security of 5G network. This includes considering following aspect for the two identified scenarios:

-	Protocol stack simplification

-	whether and how to keep the NAS signalling connection on non-3GPP access or not and

- 	whether and how to eliminate IPSec tunnel encapsulation on the user plane only or both on the control plane and the user plane, simplifying the protocol stack, reduce the user plane overhead.

-	ATSSS support over "non-3GPP access without 5G NAS”

-	Study whether and how to enhance registration and security aspects for supporting "non-3GPP access without 5G NAS".

-	Study whether and how to support splitting, switching, steering between 3GPP access and "non-3GPP access without 5G NAS.

-	Study how to manage the connection over non-3GPP access when the connection over 3GPP access is released.

NOTE:	Coordination with SA3 for handling security aspects is needed.

# * * * * End of changes * * * *