3GPP TSG-SA WG2 Meeting #160 Ad hoc-e	S2-2401020

22–29 Jan. 2024, Emeeting

Source:	Xiaomi

Title:	KI #1, New Sol: Support of Regenerative-based satellite access.

Document for:	Approval

Agenda Item:	19.1

Work Item / Release:	FS_5GSAT_ARCH_Ph3 / Rel-19

Abstract of the contribution: The contribution proposes a solution to Key Issue #1: support of regenerative-based satellite access.

1. Introduction

This contribution proposes a solution to Key Issue #1: Support of regenerative based satellite access.

2. Proposal

It is proposed to agree the following solution to 3GPP TR 23.700-29-020.

* * * Start of change * * * *

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| Solutions | #1 | #2 | #3 |  |
| 6.x | X |  |  |  |

* * * Next change * * * *

6.x	Solution X:	Support of Regenerative-based satellite access

6.x.1	Description

This solution resolves KI#1 about any impact of RAN nodes changing for any given 5GC/EPC and for a given area in the case of RAN nodes on-board moving.

Embarking gNB on-board LEO/MEO implies the on-board gNB serving for the UE access will be change as the satellite moving, which may cause the following network impacts assuming the UE is not moving:

-	N2/N3 connection frequent change (disconnection of N2/N3 between old serving gNB and AMF/UPF, reconnection of N2/N3 between new serving gNB and AMF/UPF) will cause lots of signalling on N2 interface.

-	Frequent change of the gNB serving for the TA the UE located which may cause AMF unware of which gNB should paging the UE.

The solution proposes to enhance 5G network architecture to reduce the impact of 5GC. Solution#8 in TR 23.737-h20 is proposed to reuse to address the above mentioned issues.

In this solution:

- Deploying distributed AMFs close to gateways on earth. In this option, the N2 signalling is handled by AMF locally.

- Deploying AMF agents close to gateways on earth as shown in Figure 6.x.1-1. The AMF agent is used to handle N2 signalling and forward the N2 messages between gNB and AMF.

Figure 6.x.1-1: Support of LEO/MEO satellite based gNB using AMF agent

NOTE 1:	A similar mechanism can also be applied for EPS where the gNB uses eNB instead.

The funcations of AMF agent include:

-	Establishing TNL Associations with the gNB, and establishing TNL Associations with the AMF;

-	Terminating the NG AP message(s) from on-board gNB for NG Setup or RAN Configuration Update;

-	Sending a NG AP message to the AMF to establish or update NG connection by including a proxy RAN node ID and/or proxy RAN node name which are statically configured with same format as RAN node ID and RAN node name;

-	Storing the mapping between TA(s) that each connected on-board gNB serves and the gNB ID;

-	Receiving Paging message from AMF, and forwarding the Paging message to the gNB(s) which is serving the Registration Area allocated to the UE;

-	Allocating a RAN UE NGAP proxy ID which has same format as RAN UE NGAP ID allocated by the gNB. The RAN UE NGAP proxy ID doesn't change as long as the UE is within the serving area of the AMF agent;

-	Storing 1:1 mapping between RAN UE NGAP ID and RAN UE NGAP proxy ID;

-	Updating the mapping between RAN UE NGAP ID and RAN UE NGAP proxy ID if RAN UE NGAP ID allocated by the gNB is changed;

-	Replacing the RAN UE NGAP ID included in any UE associated NG AP message from the gNB with the corresponding RAN UE NGAP proxy ID;

-	Replacing the RAN UE NGAP proxy ID included in any UE associated NG AP message from the AMF with the corresponding RAN UE NGAP ID.

NOTE 2:	Security related functions need to be addressed by SA WG3 during the normalisation phase.

Existing Registration management and connection management can be reused in the 5G network with support of gNBs on NGSO satellites with following changes:

-	The distributed AMF or the AMF agent needs to identify which on-board gNB(s) is serving the Registration Area allocated to a UE before paging the UE.

6.x.2	Procedures

The following call flow should be applied to AMF agents or distributed AMFs.

Figure 6.X.2: Call Flow for Distributed AMF or AMF agents

1.	The address of an AMF agent is configured to the earth station, so that the NG setup messages sent by satellite RAN nodes can be sent to the AMF agent.

2.	The AMF agent stores the TA list supported by the satellite RAN node, and replaces the RAN node ID with a Proxy RAN node ID which is allocated by the AMF agent. The AMF agent stores the mapping between the RAN node ID and the Proxy RAN node ID. The AMF agent may provide its serving area (TA list) to the AMF.

3-4.The AMF responds the request, the AMF agent replaces the Proxy RAN node ID with the RAN node ID, and forwards the response message to the NG RAN node.

5.	If satellite RAN node changes the supported TA list due to mobility, then the satellite RAN node sends RAN Configuration Update message including the updated TA list the RAN node supports. The message is not required to be sent to the AMF.

6.	The AMF agent sends an acknowledge message.

7.	When a UE registers via satellite RAN. The NAS transport message carrying Registration Request will be sent to the AMF agent first.

8.	The AMF agent needs to allocate a RAN UE NGAP proxy ID which is similar as RAN UE NGAP ID, and replaces the RAN UE NGAP ID in the NAS transport message with the allocated RAN UE NGAP proxy ID (so that the downlink message can be routed to the AMF agent). The AMF agent stores the mapping between RAN UE NGAP ID and RAN UE NGAP proxy ID.

9-10.	The AMF responds the registration request, the AMF agent replaces the RAN UE NGAP proxy ID with the RAN UE NGAP ID, and forwards the response message.

6.x.3	Impacts to Services, Entities and Interfaces

If local AMF is used, the AMF needs to store the TAs that each connected on-board gNB serves;

If AMF agent is used, there is no impact to existing nodes other than AMF.

* * * End of change * * * *