3GPP TSG-WG2 Meeting #160-AHe	S2-2401031

# Electronic, 22-29 January 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 4720 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Removal of the option to use TSCTSF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | GMEC |  |  |  |  |  | Date: |  |  | 2024-01-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Clause 4.15.6.14 describes two options for AF request to provision QoS and traffic characteristics for a group of users; either by invoking TSCTSF, or invoking PCF via UDR. According to clause 6.1.3.28 in TS 23.503, the AF request that is targeted to a GPSI or External Group ID may contain traffic characteristics. If TSCTSF is not involved, the NEF stores the traffic characteristics (among other QoS information in the request) to the UDR, and UDR notifies the PCF for the content of the request. This means the PCF needs to compose a TSC Assistance Container and include it in PCC Rules; the TSCAC contains the given traffic characteristics, such as Periodicity, Burst Arrival Time, etc. This means that there is no need to involve the TSCTSF to compose the TSCAC, and the option to use TSCTSF can be removed from clause 4.15.6.14. The current procedure describes that "the AF may also provide one or more of the parameters that describe the traffic characteristics as described in clause 6.1.3.23 or 6.1.3.23a of TS 23.503". However, neither of these clauses define the content of "traffic characteristics". The "traffic characteristics" definition in TS 23.503 is in clause 6.1.3.22, which in turn refers to TSCAC in TS 23.501. However, some TSCAC attributes may not be applicable for the request that is targeted to a group of users (e.g. BAT), since there is no guarantee that the impacted PDU Sessions are using a common UPF. It is therefore better to list the applicable TSCAC attributes explicitly. Further, the name of the service "Nnef_AF_Request_for_QoS" does not meet the naming conventions for the NF services as documented in Annex A. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove the option to invoke TSCTSF in clause 4.15.6.14. Other changes: - clarifies what Data Subset is used. - correction in step 12 that the request may be targeted to GPSI/SUPI as well. - correction of the message name in step 13. - removal of incorrect NOTE. - list the applicable TSCAC attributes explicitly. - rename the service to "Nnef_AFRequestForQoS". |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TSCTSF is involved even though it does not provide any value to the flow. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.15.6.14, 5.2.6.1, 5.2.6.33 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS 23.501 CR5283, TS 23.503 CR1259 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*************************** Start of Changes ***************************

#### 4.15.6.14	Procedures for AF requested QoS for a UE or group of UEs not identified by a UE address

Figure 4.15.6.14-1: Procedure for AF requested QoS for a UE or group of UEs not identified by a UE address

0.	When a new PDU Session is established, the PCF subscribes to notifications for Application Data, Data Subset = "AF request for QoS information" from UDR.

The PCF registers to BSF as described in TS 23.503 [20].

1.	The AF sends a request to reserve resources using Nnef_AFRequestForQoS_Create request message (GPSI or External Group ID, AF Identifier, Flow description(s) or External Application Identifier, QoS reference or individual QoS parameters, Alternative Service Requirements (as described in clause 6.1.3.22 of TS 23.503 [20]), DNN, S-NSSAI) to the NEF. Optionally, a period of time or a traffic volume for the requested QoS can be included in the AF request. The AF may, instead of a QoS Reference, provide the individual QoS parameters. Regardless, whether the AF request is formulated using a QoS Reference or Individual QoS parameters, the AF may also provide one or more of the following parameters that describe the traffic characteristics: flow direction, Periodicity, Survival Time, Periodicity Range. The optional Alternative Service Requirements provided by the AF shall either contain QoS References or Requested Alternative QoS Parameter Set(s) in a prioritized order as described in clause 6.1.3.22 of TS 23.503 [20].

2.	This step is the same as step 2 in clause 4.15.6.6.

3.	(Conditional) If the AF request in step 1 includes DNN and S-NSSAI, the NEF uses the Nudm_ServiceSpecificAuthorisation service to determine whether the DNN and S-NSSAI are authorized for the given user or group of users. This step is the same as steps 2-4 in clause 4.15.6.7a.

4.	The NEF uses the Nudr_DM service to store the QoS information related to the DNN and S-NSSAI, and Internal Group ID or SUPI in UDR. The information is stored as Application Data, Data Subset = "AF request for QoS information" in UDR. If the AF requested for notifications of Resource allocation status or other events, the NEF includes the information required for reporting the event, including the Notification Target Address pointing to the NEF or AF and the Notification Correlation ID containing the AF Transaction Internal ID.

5.	The NEF replies to the AF with Nnef_AF_Request_QoS_Create response.

6.	The UDR notifies the PCF(s) that have subscribed the Application Data for the given DNN/S-NSSAI and/or SUPI or Internal Group ID.

7.	The PCF(s) identifies the active PDU Sessions associated with the data received from UDR. The PCF(s) manages the AF request for QoS information as defined in clause 6.1.3.28 in TS 23.503 [20].

8.	When an event condition is met, e.g. that the establishment of the transmission resources corresponding to the QoS update succeeded or failed, the PCF sends a notification to NEF or AF as applicable. This step is the same as steps 7-8 in clause 4.15.6.6.

*************************** Next Change ****************************

#### 5.2.6.1	General

The following table shows the NEF Services and Service Operations:

Table 5.2.6.1-1: NF Services provided by the NEF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nnef_EventExposure | Subscribe | Subscribe/Notify | AF, NWDAF |
|  | Unsubscribe |  | AF, NWDAF |
|  | Notify |  | AF, NWDAF |
| Nnef_PFDManagement | Fetch | Request/Response | SMF |
|  | Subscribe | Subscribe/Notify | SMF |
|  | Notify |  | SMF |
|  | Unsubscribe |  | SMF |
|  | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Delete | Request/Response | AF |
| Nnef_EASDeployment | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Delete | Request/Response | AF |
|  | Subscribe | Subscribe/Notify | SMF |
|  | Unsubscribe |  | SMF |
|  | Notify |  | SMF |
| Nnef_ECSAddress | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Delete | Request/Response | AF |
|  | Subscribe | Subscribe/Notify | SMF |
|  | Unsubscribe |  | SMF |
|  | Notify |  | SMF |
| Nnef_TrafficInfluenceData | Subscribe | Subscribe/Notify | SMF |
|  | Unsubscribe |  | SMF |
|  | Notify |  | SMF |
| Nnef_ParameterProvision | Update | Request/Response | AF |
|  | Create | Request/Response | AF |
|  | Delete | Request/Response | AF |
|  | Get | Request/Response | AF |
| Nnef_Trigger | Delivery | Request/Response | AF |
|  | DeliveryNotify | Subscribe/Notify | AF |
| Nnef_BDTPNegotiation | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Notify |  | AF |
| Nnef_TrafficInfluence | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Delete | Request/Response | AF |
|  | Get | Request/Response | AF |
|  | Notify | Subscribe/Notify | AF |
|  | AppRelocationInfo | Subscribe/Notify | AF |
| Nnef_ChargeableParty | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Notify | Request/Response | AF |
| Nnef_AFsessionWithQoS | Create | Request/Response | AF |
|  | Notify | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Revoke | Request/Response | AF |
| Nnef_MSISDN-less_MO_SMS | Notify | Notify | AF |
| Nnef_ServiceParameter | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Delete | Request/Response | AF |
|  | Get | Request/Response | AF |
| Nnef_APISupportCapability | Subscribe | Subscribe/Notify | AF |
|  | Unsubscribe | Subscribe/Notify | AF |
|  | Notify | Subscribe/Notify | AF |
| Nnef_NIDDConfiguration | Create | Request/Response | AF |
|  | TriggerNotify | Subscribe/Notify | AF |
|  | UpdateNotify | Subscribe/Notify | AF |
|  | Delete | Request/Response | AF |
| Nnef_NIDD | Delivery | Request/Response | AF |
|  | DeliveryNotify | Subscribe/Notify | AF |
|  | GroupDeliveryNotify | Notify | AF |
| Nnef_SMContext | Create | Request/Response | SMF |
|  | Delete | Request/Response | SMF |
|  | DeleteNotify | Subscribe/Notify | SMF |
|  | Delivery | Request/Response | SMF |
| Nnef_AnalyticsExposure | Subscribe | Subscribe/Notify | AF |
|  | Unsubscribe |  | AF |
|  | Notify |  | AF |
|  | Fetch | Request/Response | AF |
| Nnef_UCMFProvisioning | Create | Request/Response | AF |
|  | Delete | Request/Response | AF |
|  | Update | Request/Response | AF |
| Nnef_ECRestriction | Get | Request/Response | AF |
|  | Update | Request/Response | AF |
| Nnef_ApplyPolicy | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Delete | Request/Response | AF |
| Nnef_Location | LocationUpdateNotify | Notify | AF |
| Nnef_TimeSynchronization | ConfigUpdate | Request/Response | AF |
|  | ConfigCreate | Request/Response | AF |
|  | ConfigDelete | Request/Response | AF |
|  | ConfigUpdateNotify | Subscribe/Notify | AF |
|  | CapsSubscribe | Subscribe/Notify | AF |
|  | CapsUnsubscribe | Subscribe/Notify | AF |
|  | CapsNotify | Subscribe/Notify | AF |
| Nnef_ASTI | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Delete | Request/Response | AF |
|  | Get | Request/Response | AF |
|  | UpdateNotify | Subscribe/Notify | AF |
|  | Subscribe |  |  |
|  | Unsubscribe |  |  |
| Nnef_AMPolicyAuthorization | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Delete | Request/Response | AF |
|  | Notify | Subscribe/Notify | AF |
|  | Subscribe |  | AF |
|  | Unsubscribe |  | AF |
| Nnef_AMInfluence | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Delete | Request/Response | AF |
|  | Notify | Subscribe/Notify | AF |
| Nnef_UEId | Get | Request/Response | AF |
| Nnef_SMService | MoForwardSm | Request/Response | SMS-SC |
| Nnef_PDTQPolicyNegotiation | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Notify |  | AF |
| Nnef_MemberUESelectionAssistance | Subscribe | Subscribe/Notify | AF |
|  | Unsubscribe |  | AF |
|  | Notify |  | AF |
| Nnef_AFRequestForQoS | Create | Request/Response | AF |
|  | Notify | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Revoke | Request/Response | AF |
| Nnef_DNAIMapping | Subscribe | Subscribe/Notify | AF, NWDAF |
|  | Unsubscribe |  | AF, NWDAF |
|  | UpdateNotify |  | AF, NWDAF |
| Nnef_UEAddress | Get | Request/Response | AF |

*************************** Next Change ****************************

#### 5.2.6.33	Nnef_AFRequestForQoS service

##### 5.2.6.33.1	General

See clause 4.15.6.14.

This service is also used to support subscription and notification of QoS Monitoring, as described in clause 5.33.3.2 of TS 23.501 [2].

##### 5.2.6.33.2	Nnef_AFRequestForQoS_Create service operation

Service operation name: Nnef_AFRequestForQoS_Create

Description: The consumer requests the network to provide a specific QoS for a traffic flow for a UE or a group of UEs.

Inputs, Required: AF Identifier, Target UE identifier (GPSI or External Group Identifier), Flow description(s) or External Application Identifier, QoS Reference or individual QoS parameters as described in clause 6.1.3.22 of TS 23.503 [20].

Inputs, Optional: time period, traffic volume, Alternative Service Requirements (containing one or more QoS reference parameters or Requested Alternative QoS Parameter Sets in a prioritized order), QoS parameter(s) to be measured, Reporting frequency, Target of reporting and optional an indication of local event notification as described in clause 6.1.3.21 of TS 23.503 [20], DNN if available, S-NSSAI if available, the following traffic characteristics: flow direction, Periodicity, Survival Time, Periodicity Range.

Outputs, Required: Transaction Reference ID, result.

Output (optional): None.

##### 5.2.6.33.3	Nnef_AFRequestForQoS_Notify service operation

Service operation name: Nnef_AFRequestForQoS_Notify

Description: NEF reports the QoS Flow level event(s) to the consumer.

Inputs, Required: Reports of the events as defined in clause 6.1.3.18 of TS 23.503 [20].

Inputs, Optional: When the event report is for QoS Monitoring, includes Packet delay for UL, DL, or round trip of the single UP path or two UP paths in the case of redundant transmission, as defined in clause 5.33.3.2 of TS 23.501 [2].

Outputs, Required: None.

Output (optional): None.

##### 5.2.6.33.4	Nnef_AFRequestForQoS_Revoke service operation

Service operation name: Nnef_AFRequestForQoS_Revoke

Description: The consumer requests the network to revoke the Service Requirement(s) and/or additional Alternative Service Requirement(s) for a UE or a group of UEs.

Inputs, Required: Transaction Reference ID.

Inputs, Optional: None.

Outputs, Required: Transaction Reference ID, result.

Output (optional): None.

##### 5.2.6.33.5	Nnef_AFRequestForQoS_Update service operation

Service operation name: Nnef_AFRequestForQoS_Update

Description: The consumer requests the network to update the Service Requirement(s) and/or additional Alternative Service Requirement(s) for a UE or a group of UEs.

Inputs, Required: Transaction Reference ID.

Inputs, Optional: Flow description information (as described in clause 6.1.3.6 of TS 23.503 [20]), QoS reference or individual QoS parameters as described in clause 6.1.3.22 of TS 23.503 [20], time period, traffic volume, Alternative Service Requirements (containing one or more QoS reference parameters or Requested Alternative QoS Parameter Sets in a prioritized order), QoS parameter(s) to be measured, Reporting frequency, Target of reporting and optional an indication of local event notification as described in clause 6.1.3.21 of TS 23.503 [20], the following traffic characteristics: flow direction, Periodicity, Survival Time, Periodicity Range.

Outputs, Required: Result.

Output (optional): None.

*************************** End of Changes ****************************