# SA WG2 Meeting #160-AH-e	S2- 2401048

# January 22 - 29, 2024, Electronic Meeting                                       (revision of S2-2313653)

Title:	[Draft] Reply LS on SA5 LS to SA2 on OAM input data for QoS Sustainability Analytics

Response to:	Reply LS on OAM input data for QoS Sustainability Analytics (S5-235776)

Release:	Release 18

Work Item:	PM_KPI_5G_Ph3 (960025)

Source:	3GPP SA2

To:	3GPP SA5

Cc:

Contact Person:	Aihua Li

liaihua@chinamobile.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments: N/A

1. Overall Description:

SA2 thanks SA5 on discussion of OAM input data for QoS Sustainability Analytics related to  “UL/DL capacity GTP” and  “UL/DL available capacity GTP”.

For first question:

- Please provide more information on capacity GTP and available capacity GTP (e.g.,the GTP capacity measurements, how to calculate the capabilities and etc.). If possible, please also provide the related parameters may influence these capacities.

SA2 Answer: SA2 believes that Capacity GTP and available capacity GTP can refer to IP-layer section capacity and IP-layer available section capacity (in addition to Table 6.9.2-3 of TS 23.288). Some companies suggest to refer to chapter 6.11.1 of ITU-T Y.1540.

For the second question:

- After checking ITU-T Y.1540, there are no direct descriptions on capacity GTP and available capacity GTP between UPF and NG-RAN/between UPF and UE. However, we found one which may be able to be referred, “The UDP capacity represents the maximum achievable IP rate for each line”. Therefore, the one of the possible measurements for capacity GTP and available capacity GTP may be defined as follow: capacity GTP: the maximum achievable GTP data volume available capacity GTP: the available GTP data volume Please confirm whether this performance measurements have met your expectations for capacity GTP and available capacity GTP.

SA2 Answer: Some companies believe that the IP-layer section capacity and IP-layer available section capacity description from ITU-T Y.1540 can be used for the following measurements:

capacity GTP: the maximum achievable GTP transmission rate, i.e., data volume per time interval [t, t + Δt].

available capacity GTP: the available GTP transmission rate, i.e., data volume per time interval [t, t + Δt].

2. Actions:

To SA5

ACTION:

SA2 asks SA5 to take SA2’s confirmation into account in future work.

3. Dates of next TSG SA WG 2 meetings:

TSG-SA2 Meeting #161		February 26 – March 1, 2023            	Athens, GR

TSG-SA2 Meeting #162		April 15 – April 19, 2023            	             TBD, CN