3GPP TSG-WG SA2 Meeting #160E e-meeting 	S2-2401065

Elbonia, January 22th – 29th, 2024	(revision of S2-240xxxx)

Source:	CMCC

Title:	New KI on Enhancement of LCS to support AI/ML-based positioning (WT1.4)

Document for:	Approval

Agenda Item:	19.15

Work Item / Release:	FS_AIML_CN / Rel-19

Abstract: Propose a new key issue to study the New KI on Enhancement of LCS to support AI/ML-based positioning to WT1.4.

1. Introduction/Discussion

This contribution is related to WT#1.4:

WT1.4:	Study whether and how to consider enhancements to LCS to support AI/ML based Positioning considering the conclusions in 3GPP TR 38.843.

According to TR 38.843, the following Cases are considered for the study:

-	Case 1: UE-based positioning with UE-side model, direct AI/ML or AI/ML assisted positioning

-	Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning

-	Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning

-	Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning

-	Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning

For positioning enhancement use case:

-	For model training, training data can be generated by UE/PRU/gNB/LMF.

-	For LMF-side model inference (Case 2b, Case 3b), input data can be generated by UE/gNB and terminated at LMF.

-	For gNB-side model inference (Case 3a), input data is internally available at gNB.

-	For UE-side model inference (Case 1, Case 2a), input data is internally available at UE.

-	For performance monitoring at the LMF side, calculated performance metrics (if needed) or data needed for performance metric calculation (if needed) can be generated by UE/gNB and terminated at LMF.

-	For performance monitoring at the gNB side, calculated performance metrics (if needed) or data needed for performance metric calculation (if needed) can be generated by at least gNB.

For LMF-side model inference (Case 2b, Case 3b), input data can be generated by UE/gNB and terminated at LMF.

Considering that the core network has defined the intelligent network element NWDAF since R-16, and has formed the capabilities of data collection, AI/ML training, inference and closed-loop control by R-18, and supports the network data analytics architecture of diversified solutions, the interaction between NWDAF and LMF is considered to AI assisted positioning. The general solution is as follows:

-	AMF, LMF or other NFs with positioning information are used to assist positioning through AI/ML model training and inference services provided by NWDAF.

-	NWDAF may monitor or improve the ability to assist in positioning data accuracy, AI/ML model performance, and more.

2. Text Proposal

It is proposed to capture the following changes into TR 23.700-84.

# * * * * First change * * * * All new

5	Key Issues

5.x	Key Issue # X- Enhancement of LCS to support AI/ML-based positioning

5.x.1	Description

This key issue proposes to study whether and how to enhance LCS supports AI/ML-based positioning. In this key issue, the following aspects need to be studied:

-	Whether and how to make architectural and/or functional enhancements for enabling LCS system to support AI/ML-based positioning, if there is additional signalling load caused by any new functional enhancement and how to mitigate the signalling?

-	Whether and how the LMF can improve the correctness of AI/ML-based positioning (e.g. which existing and/or additional information may be used by the LMF to improve the correctness of AI/ML-based positioning)?

-	Whether and which action(s) should be taken by consumer NF to help improve the correctness of LMF positioning.

-	Whether and how the NWDAF can assist the LMF in supporting AI/ML-based positioning, considering the following aspects:

-	If NWDAF provides analytics to assist LMF in supporting AI/ML-based positioning. What is the potential consumer of these analytics (e.g. AF/AMF) and how does that consumer use these analytics?

-	How and which information is needed to enhance the AI/ML model provisioning to improve the correctness of AI/ML-based positioning.

-	Study mechanisms to detect that whether deviation on AI/ML prediction has happened and which action could be triggered.

NOTE X:	UE data collection, model delivery and transfer to the UE and model identification/management are not within the scope of this key issue.

# * * * * End of changes * * * *