SA WG2 Meeting #160-AH-e	S2-2401085

January 22 - 29, 2024, Electronic Meeting

Source:	China Mobile

Title:	New KI for Work Task #1 of FS_UIA_ARC

Document for:	Approval

Agenda Item: 	19.8

Work Item / Release:	FS_UIA_ARC /Rel-19

Abstract of the contribution: This paper proposes a new Key Issue to cover work task #1 of the FS_UIA_ARC SID.

1	Discussion

This paper proposes a new Key Issue to cover Work Task #1 of the FS_UIA_ARC SID (SP-231804). Work Task #1 from the FS_UIA_ARC SID is shown in the text box below.

Work Tasks 1.x focuses on supporting the use case where the user identifier of a human is associated with traffic that is to/from the UE: - WT#1.1: Define the architectural assumptions that are necessary to support identifying the user identifier that is associated with a UE’s traffic. NOTE A: When the user identifier applies to a human, only a single user identifier is associated with the UE at a given time and it is assumed that the user identifier is associated with all of services that the UE access during the time that the user identifier and UE are associated. - WT#1.2: What information is stored as part of the user profile (e.g., a user identifier, associated security credentials, associated devices, user specific QoS settings). Including how user profiles are acquired, stored, and updated in the 5GC. - WT#1.3: Whether and how user identifiers are linked and unlinked (i.e., associated) with 3GPP subscriptions in an operator-controlled manner. - WT#1.4: Whether and how user specific QoS settings are taken into account by the 3GPP system in order to provide service differentiation when providing communication services.

In the proposed KI, the WT#1.1 is not captured, which should be reflected in the section of architectural assumptions.

2	Proposal

It is proposed to include the following changes in TR 23.700-32 V0.0.0.

* * * * Start of Changes – All New Text * * * *

5.x	Key Issue #x: Identify the traffic that is sent to/from the UE with user identifier.

5.X.1	Description

This key issue focuses on the enhancement about identifying the traffic that is sent to/from the UE with user identifier. Solutions to this key issue will address:

- What information is included as part of the user profile (e.g., a user identifier, associated security credentials, associated devices, user specific QoS settings), and how to acquire, store, and update the user profiles in the 5GC,

- Whether and how user identifiers are linked and unlinked (i.e., associated) with 3GPP subscriptions in an operator-controlled manner, and

- Whether and how user specific QoS settings are taken into account by the 3GPP system in order to provide service differentiation when providing communication services.

* * * * End of Changes * * * *