SA WG2 Meeting #160-Ad Hoc-e	S2-2401135

# E-meeting, January 22 – 29, 2024			         (revision of S2-234xxxx)

Source: 	Nokia, Nokia Shanghai Bell

Title: 	FS_MASSS_Architectural Assumptions and Architectural Requirements

Document for: 	Approval

Agenda Item: 	19.13

Work Item / Release:	FS_MASSS / Rel-19

Abstract of the contribution: This contribution proposes architectural assumptions and architectural requirements for DualSteer objectives of FS_MASSS.

# 1	Discussion

FS_MASSS SID objectives have 2 sets of work tasks. WT#1 is for supporting DualSteer device. This CR proposes architectural assumptions and architectural requirements for WT#1.

This CR also updates the references section in TR 23.700-54.

# 2 Proposal

It is proposed to update TR 23.700-54 v0.0.0 on FS_MASSS as follows.

* * * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[x1]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[x2]	3GPP TS 23.502: "Procedures for the 5G system, Stage 2".

[x3]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System".

[x4]	3GPP TS 22.261: "Service requirements for the 5G system; Stage 1".

* * * * Next Change * * * *

# 4	Architectural Assumptions and Requirements

## 4.1	Architectural Assumptions

This study has following architectural assumptions:

-	The HPLMN is assumed to be in full control of the traffic policies for DualSteer provided to the DualSteer device and to the network.

-	For simultaneous transmission over two networks, a DualSteer device is assumed to include two separate UEs.

-	For traffic steering and/or switching of a DualSteer device's user data across two 3GPP access networks belonging, it is assumed that the data anchoring in done at the HPLMN.

-	For any particular service, at any given time, it is assumed that the DualSteer device transmits all traffic of that service using only a single 3GPP access network.

-

* * * * Next Change * * * *

## 4.2	Architectural Requirements

This study has following architectural requirements:

-	The 5GS shall be able to identify and link the two SUPIs/UEs associated with a DualSteer device, differentiate the two connections for the DualSteer device and minimize impacts to CN, O&M or IT systems.

-	A DualSteer device shall have two subscriptions/SUPIs, sharing one subscription profile from the same operator.

-	5G system shall be able to support mechanisms to minimize service interruption when switching a DualSteer device's user data, for one or multiple services, between two 3GPP access networks.

-	For traffic steering and/or switching of user data across two 3GPP access networks, the 5G system may be able to support mechanisms to change one 3GPP access network to the non-3GPP access network of the same subscription (and vice versa).

-	The 5G system shall be able to support traffic steering and/or switching of a DualSteer device's user data between a NPN and a PLMN, for one or more a DualSteer devices with a NPN subscription accessing NPN services, to meet specific QoS requirements for each device, assuming non-simultaneous transmission over the two 3GPP access networks.

-	The architecture to support DualSteer devices shall cover two 3GPP access networks belonging to the same PLMN, or between two different PLMNs, or between one PLMN and one PLMN-integrated NPN, over same or different RAT, which can use terrestrial and/or satellite access (including the case of two different satellite orbits). Scenarios may also include traffic steering and/or switching across LTE/EPC and NR/5GC, with anchoring in 5GC.

* * * * End of Changes * * * *