3GPP SA WG2 Meeting #160AH-e	S2-2401226

Electronic Meeting, January 22-29, 2024

Source:	Nokia, Nokia Shanghai Bell

Title:	KI description for WT#1.1

Document for:	Approval

Agenda Item:	19.9

Work Item / Release:	FS_eEDGE_5GC_ph3 / Rel-19

Abstract of the contribution: This paper proposes KI description for WT#1.1

# Discussion

The new approved SID titled Study on Enhancement of support for Edge Computing in 5G Core network — phase 3 (SP-231796 from SA#102 Plenary) includes the below aspects to be studied as part of Rel-19:

WT#1) Whether and how to support more efficient Edge Hosting Environment information management and related EAS Discovery:

-	WT#1.1: Handle the edge network and EAS related information (e.g. UPF and EAS deployment information, DNAIs) locally with less impact to 5GC central NFs (e.g. central SMF) to address more flexible EAS (re)discovery/(re)selection, local UPF (re)selection.

A Rel. 19 KI is needed for this WT. The following is proposed.

# Proposal

It is proposed to incorporate the following KI for Rel. 19 FS_eEDGE_5GC_ph3 WT#1.1 in TR23.700-49 v0.0.0.

*** START OF CHANGES (ALL NEW TEXT) ***

## 5.X	Key Issue #X: Enhancements for EAS (re)discovery and UPF (re)selection using locally handled EAS related information

### 5.X.1	Description

This key issue investigates the potential need and solutions to handle the edge network and EAS related information (e.g. UPF and EAS deployment information, DNAIs) locally with less impact to 5GC central NFs (e.g. central SMF) to address more flexible EAS (re)discovery/(re)selection, local UPF (re)selection.

The following aspects shall be studied:

-	Whether and which information (e.g. UPF and EAS deployment information, DNAIs) needs to be handled locally (centrally or both) for EAS (re)discovery and/or local UPF (re)selection.

-	How is it determined whether the above information needs to be handled locally, centrally, or both for EAS (re)discovery and/or local UPF (re)selection;

-	Which entities determine whether the above information needs to be handled locally, centrally, or both for EAS (re)discovery and/or local UPF (re)selection;

-	Whether existing procedures (e.g. discovery, ULCL/BP, mobility and influence/exposure related procedures) suffice or whether there are gaps to be addressed when above information is handled locally or both locally and centrally.

NOTE 1: Existing network functions will be considered as baseline in this Key Issue.

NOTE 2: Roaming cases are not considered as part of this Key Issue.

*** End of changes ***