SA WG2 Meeting #160-Ad Hoc-e	S2-2401234

E-meeting, January 22 – 29, 2024

Source: 	Samsung

Title: 	23.700-77: KI#8, New Sol: Standalone Application Data Channel Setup Signalling Procedure without accompanying bootstrap data channel

Document for: 	Approval

Agenda Item: 	19.2

Work Item / Release:	FS_NG_RTC_Ph2 / Rel-19

Abstract of the contribution: This paper proposes solution for KI#6.

1 Introduction

This paper proposes new solution for KI#8 in TR 23.700-77, how to support Standalone Application Data Channel Setup Signalling without accompanying bootstrap data channel.

2 Proposal

It is proposed to approve following content in TR 23.700-77.

# * * * * First change * * * *

6.0	Mapping of Solutions to Key Issues

Table6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| 1 | X |  |  |  |  |  |  |  |
| X |  |  |  |  |  | X |  |  |

# * * * * Second change * * * *

6.X	Solution #X: Standalone Application Data Channel Setup Signalling Procedure without accompanying bootstrap data channel

6.X.1	Description

As specified in 23.228, the IMS DC channel session can be supported based on the IMS MMTEL session has various limitations as follows.

In order to establish an application data channel, bootstrap data channel establishment must be performed first.

UE can establish an IMS Data Channel simultaneously while establishing an IMS audio/video session or upgrade an ongoing IMS audio/video session through a re-INVITE to an IMS Data Channel session.

When a bootstrap data channel is established successfully, the bootstrap data channel shall be kept available during the IMS MMTel session, and it shall be terminated along with the release of the call.

When the UE is establishing an IMS Application DC as defined in clause AC.7.2, the binding information is provided by the UE in the SDP offer as described in TS 26.114 [76]. The IMS AS provides the binding information to the DCSF. The DCSF may use the binding information for controlling the Application DC setup.

However, unlike before, standalone IMS DC sessions should be able to support the establishment of a standalone bootstrap data channel and a standalone application data channel. For example, when using a data channel application already stored in the terminal, such as a native application, the terminal can omit the data channel application download process through the bootstrap data channel establishment procedure. If the standalone application data channel establishment procedure is performed without the bootstrap data channel establishment procedure, the UE can add application-related information (e.g. DC Application ID) to the SIP INVITE message and use it during the application data channel establishment negotiation procedure.

6.X.2	Procedures

Figure 6.X.2-1: Standalone Application Data Channel Setup Signalling Procedure without accompanying bootstrap data channel

- UE#1 sends the SIP INVITE request with an SDP offer for application DC. DCSF decides whether DC is provided and determines DC control policy. IMS AS sends the INVITE which includes the updated SDP offer adding media information of MF or MRF via the originating S-CSCF to terminating network side and UE#2.

- UE#2 checks the previously stored data application and application binding information based on the application-related information received from UE#1.

  - If UE#2 determines that it does not support the data channel application or application binding information based on the application-related information received from UE#1, it may request establishment of a bootstrap data channel to download the data channel application or application binding information. UE#2 can download the data channel application and application binding information through the bootstrap data channel and compare it with the application-related information received from UE#1 to determine whether to accept the establishment of the application data channel.

- UE#2 sends a 200 OK response if it accepts establishment of an application data channel based on the application-related information received from UE #1.

  - UE#2 sends a 606 Not Acceptable response if it cannot accept application data channel establishment based on the application-related information

  - After receiving the 606 message in the above step 3a, UE#1 sends a message requesting to restart the bootstrap data channel establishment procedure to the network and the remote UE. UE #1 and UE #2 performs an application data channel establishment procedure by receiving proper data channel application and application binding information through the bootstrap data channel.

6.X.3	Impacts on Existing Nodes and Functionality

This solution may have the following impacts to exiting entities and interfaces:

UE:

- Initiates IMS DC session with standalone Application Data Channel;

- Initiates/releases IMS DC Session with standalone Bootstrap Data Channel.

# * * * * End of changes * * * *