3GPP TSG-WG SA2 Meeting #160AHe	S2-2401258

# Electronic meeting, 22 - 26 Jan 2024		(revision of S2-24)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.586 | CR | 0113 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to Ranging/SL Positioning service exposure through PC5 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Xiaomi |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Ranging_SL |  |  |  |  |  | Date: |  |  | 2024-01-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Considering that any further change in the SA2 procedure will greatly impact the workload of downstream working groups, it is crucial to keep the stableness of the SA2 features during maintenance phase, it is proposed to down scope the feature of Ranging/SL Positioning service exposure to the SL Positioning Client UE through PC5 for R18 as below: - Remove the support of exposing absolute location to SL Positioning Client UE - Remove the support of network-based operation in the “procedure of Ranging/SL Positioning service exposure through PC5” |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Clarified that only UE-only Operation is supported in the procedure of Ranging/SL Positioning service exposure through PC5 for this release. - Clarified that absolute location is not supported in the procedure of Ranging/SL Positioning service exposure through PC5 for this release - Removed the authurization related descriptions by referring to SA3 specification - Simplified the procedure of “Ranging/SL Positioning service exposure through PC5” by referring to clause 6.8. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unalignment between TS 23.586 and TS 23.273 on the support of Ranging/SL Positioning service exposure through PC5. Unalignment between TS 23.586 and TS 33.533 on security aspects for the support of Ranging/SL Positioning service exposure through PC5. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.6.2.2, 6.7.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

5.6.2.2	Ranging/SL Positioning service exposure through PC5

Ranging/SL Positioning service exposure to a UE through PC5 is supported in the following way:

-	SL Positioning Client UE discovers one of the two or more UEs described in clause 5.6.1, e.g. the SL Reference UE(s) or Target UE(s), using the Ranging/SL Positioning UE discovery procedure (refer to clause 6.4);

-	SL Positioning Client UE invokes the Ranging/SL Positioning service request to the discovered UE, e.g. SL Reference UE/Target UE, with the attributes described in clause 5.6.1. The service request includes the SL Positioning Client UE's user info, Target UE's user info, and SL Reference UE's user info. The Ranging/SL Positioning service request/response is transmitted in the Supplementary RSPP signalling message over SR5 and handled in the Ranging/SL Positioning layer.

-	How the SL Positioning Client UE is authurised to acquire the Ranging/SL Positioning information is  specified in TS 33.533 [13].

NOTE 1: For this release, only UE-only Operation is supported in the procedure of Ranging/SL Positioning service exposure through PC5.

NOTE 2: For this release, absolute location is not supported in the procedure of Ranging/SL Positioning service exposure through PC5.

# * * * * Next change * * * *

6.7.1.1	Procedures for Ranging/SL Positioning service exposure through PC5

Figure 6.7.1-1: Service exposure to SL Positioning Client UE via PC5

1.	When a Ranging/SL Positioning service is triggered, SL Positioning Client UE may perform discovery procedure for the Ranging/SL Positioning service defined in clause 6.4 with following updates:

-	When SL positioning service is triggered, the client UE tries to discover the target UE and/or one or more SL Reference UE(s). When Model B discovery used, in the Solicitation message, target UE's user info is included and role of target UE and role of SL Reference UE are indicated in the RSPP metadata.

-	When Ranging service is triggered, the client UE tries to discover UE1 and/or UE2. When Model B discovery used, in the Solicitation message, UE1's user info and UE2's user info are included and role of target UE is indicated in the RSPP metadata.

During discovery for SL positioning service, the information of SL reference UE(s) is acquired.

2.	If discovery procedure succeeds, the SL Positioning Client UE establishes a PC5 connection with the discovered UE.

3. The SL Positioning Client UE triggers procedures of Ranging/SL Positioning control defined in clause 6.8 to acquire the Ranging/SL Positioning information between UE1 and UE2.

# * * * * End of changes * * * *