SA WG2 Meeting S2#160-Ad Hoc-e	S2-2401278

22 - 26 January, 2024, Online	revision of S2-24xxxxx

Source:	Nokia, Nokia Shanghai Bell

Title:	KI#8: New solution on Support of IMS Avatar Communication without using data channel

Document for:	Approval

Agenda Item:	19.2

Work Item / Release:	FS_NG_RTC_Ph2

Abstract of the contribution: This contribution proposes a new solution to KI#8 without using data channel.

# Introduction

This contribution proposes a solution to KI#8 on “Support of IMS avatar communication” including architectural aspects.

# 2	Proposal

It is proposed to add following text to TR 23.700-77.

# * * * * First change* * * *

# 3	Definition, symbols and abbreviations

## 3.3	Abbreviations

DAC	Digital Asset Container

UUID	Universally Unique Identifier

# * * * * Next change* * * *

# 6	Solutions

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| 1 | X |  |  |  |  |  |  |  |
| Y |  |  |  |  |  |  |  | X |

# * * * * Next change* * * *

## 6.X	Solution #Y: Solution for support of IMS avatar communication without using data channel

### 6.X.1	Description

Figure 6.X.1-1: Architecture to support IMS avatar communication

To support of IMS avatar communication, the architecture is enhanced as follows:

Digital Asset Container (DAC):

-	The Digital Asset Container is used for storing and retrieving the avatar model from 5GC. DAC can be internal to the PLMN, e.g., a new network function, or a part of network function (HSS, UDM, IMS AS), a XR application server or a web sever within the IMS and/or 5GC; or DAC can be external to the PLMN, e.g., a webserver of a cloud provider,

XR Application Server (XR AS):

-	The XR Application Server is responsible for XR service control related to XR communication, including XR session media control, XR media capability negotiation with the UE.

Editor's note:	The definition of the reference point between DAC and XR Application Server is for FFS.

Two new information elements are defined as follows:

Avatar metadata:

-	Avatar metadata is the 2D/3D representational content/model of an avatar stored in a file, which can be accessed via a URL. The avatar metadata, or a reference of the avatar metadata (e.g., identifier), is associated per subscriber and/or per subscriber’s user identifier. Avatar metadata needs to correspond to face, full body, or a portion of the body. Additional metadata can be associated with the avatar metadata, e.g., format, resolution, access/usage rights & time, date of creation, licensing, etc.

Avatar-id:

-	An avatar metadata is addressable via an Avatar-id. It can be integer, string, UUID, or a URI (URN or URL).

Avatar media:

-	Avatar media is the user/media plane data related to an avatar animation transmitted between two UEs. It can be a combination of face/gesture movement and the avatar metadata.

This solution assumes that Avatar Id is provided in SIP/SDP and Avatar metadata are downloaded to UE without using data channel.

### 6.X.2	Procedures

Editor's note:	This clause describes high-level procedures and information flows for the solution.

#### 6.X.2.1.	Network centric IMS avatar call flow using SIP/SDP

Figure 6.X.2.1-1: Network centric IMS avatar call flow using SIP/SDP

UE-A avatar media is transmitted to UE-B and exact format is negotiated in the SDP as part of the session establishment.

At step 2, UE A provides to the IMS network the Avatar ID of the avatar to be used in the call, e.g., locally stored or provided at a previous step. Avatar ID can be e.g., sent within a m- or a- line in the SDP.

At step 3, the IMS AS provides the Avatar ID to the XR Application Server to retrieve the related representation from DAC (steps 5-6).

At Step 4, Avatar ID is exchanged with UE-B to indicate the avatar session. UE-B has the option to reject the avatar or terminate the session.

At step 8, the IMS AS confirms to UE-A that this Avatar can be used. At step 9-11, it tells the XR Application Server to request rendering of that Avatar by the MF/MRF.

At step 11, UE A sends plain video stream (no avatar) to MF/MRF.

At step 12, the MF/MRF receives the video stream from UE-A and replaces the face/body with the selected avatar at step 13.

The resulting stream is sent as regular video to UE-B in step 14. At step 15, the rendered Avatar video can be sent back to UE-A as feedback (same content as the one sent to UE-B in step 14), e.g., to display a thumbnail view of the Avatar to UE-A.

Editor's note:	Further details of the call flow are FFS.

#### 6.X.2.2	UE-A centric IMS avatar call flow using SIP/SDP

Figure 6.X.2.2-1: UE A Centric IMS avatar call flow using SIP/SDP

Avatar ID is exchanged with the IMS network for getting the avatar representation. Avatar ID is sent within SDP.

At step 3, the IMS AS provides Avatar ID to XR Application Server to retrieve the related representation from DAC (steps 5-6).

Step 14: If UE A and UE B have agreed during session setup or update that UE B uses plain video media, UE B provides plain audio/video media over RTP.

Editor's note:	Further details of the call flow are FFS.

#### 6.X.2.3	UE-B centric IMS avatar call flow using SIP/SDP

Editor's note:	Further details of the call flow are FFS.

### 6.X.3	Impacts on existing nodes and functionality

Editor's note:	This clause captures impacts on existing 3GPP nodes and functional elements.

# * * * * End of changes * * * *