SA WG2 Meeting #160-Ad Hoc-e	S2-2401292

# January 22 – 29, 2024, Elbonia

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0500 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Multiple Location Report for Next Generation Emergency Routing |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19 |  |  |  |  |  | Date: |  |  | 2024-01-11 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As per regulatory requirements described in the discussion paper - S2-2401268, location-based emergency call routing requires device location to be available on each hop without delaying the normal call set-up process. Therefore, LCS in the operator network shall provide the location to multiple LCS clients (i.e., emergency call routing entities) for the same device, and meet the QoS and delay expected by the sequence of routing entities. Multiple routing entities can have better and better location results while hop-by-hop routing is performed. More accurate result can be available to more recent LCS request. For example, the initial hop to nation-wide routing entity may be routed based on initial cell-based location. Subsequent hops can be routed based on a more accurate location estimates when more measurements are available afterwards, especially when device is nearby the border of different PSAP serving areas. Multiple Location Procedure defined in clause 6.1.3 of TS 23.273 serves only a single (PSAP) client. It shall be enhanced to provide a sequent of locations upon to be requested by multiple LCS Clients (emergency call routing entities) within a single LCS session. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding a new section to enable multiple location procedure for multiple LCS Client (i.e., emergency routing entities) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Location result for routing may not meet the regulatory delay and QoS requirements due to limited time of routing hop. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.10.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change [all new] * * * *

### 6.10.x	Multiple location procedure for multiple emergency LCS clients

Figure 6.10.x-1 illustrates an extension of multiple location procedure for the emergency regulatory location service defined in clause 6.10.4.

This procedure is applicable for providing location estimates generated by the multiple location procedure to respective emergency services entities (e.g., routing entities, etc).

Figure 6.10.x-1: Multiple Location Procedure for multiple emergency LCS clients

1.	When UE initiates an IMS emergency session, LRF service is requested (for routing) as defined in clause 4.3.4.

2 ~ 3. Steps 2-3 for 5GC-MT-LR Procedure without UDM Query in clause 6.10.2 are performed with the following differences:

-	At Step 2 of clause 6.10.2, Namf_Location_ProvidePositioningInfo applies same extension defined in Step 1 of clause 6.1.3 yielded by GMLC/LRF.

-	At Step 3 of clause 6.10.2, Nlmf_Location_DetermineLocation applies same extension defined in Step 1 of clause 6.1.3 inherited by Step 2.

4.	 Step 2 of clause 6.1.3 5GC-MT-LR multiple location procedure for the regulatory location service is performed	4.1. Step 2.1 of clause 6.1.3 5GC-MT-LR multiple location procedure for the regulatory location service is performed.

5. 	The GMLC/LRF responds to the service request based on:

- The best available result among Step 4.1 and Step 9;

- The result as per clause 7.1.3 in TS 23.167[10];

6.	A different LCS client may invoke the service request (e.g., for the next hop of emergency routing) that can be identified as the same multiple location procedure of Step 4.

7.	The GMLC/LRF responds to the service request as in Step 5.

8.	The LMF returns the Nlmf_Location_DetermineLocation Response towards the AMF.

9.	The AMF returns the Namf_Location_ProvidePositioningInfo Response towards the GMLC/LRF.

# * * * * End of changes * * * *