3GPP SA WG2 Meeting #160-Ad Hoc-e	S2-2401347

# E-meeting, January 22 – 29, 2024                		was S2-2313182

Source:	Meta USA

Title:	KI#1: New Sol: AS/AF provided PDU sets correlation information

Document for:	Approval

Agenda Item:	19.3

Work Item / Release:	FS_XRM Ph2 / Rel-19

Abstract of the contribution: Proposal on PDU set correlation info for better congestion handling by RAN.

# Proposal

Accept the following to TR 23.700-070 v0.1.0.

This is related to KI#1:

-	whether, what and how PDU Set based handling (e.g. new standardized 5QI, enhancements to Alternative QoS profiles, FEC, etc.) and PDU Set information (including Control Plane and/or User plane information) provided by the AF/AS are enhanced.

*** First change ***

# 6	Solutions

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

| Solutions |  |  |
| --- | --- | --- |
|  | <Key Issue #1> | <Key Issue #2> |
| #1 |  |  |
| #2 |  |  |
| #X; PDU set discard based on PDU set correlation info from AS/AF | X |  |

*** all new texts below ***

## 6.X	Solution #X: PDU set discard based on PDU sets correlation info from AS/AF.

### 6.X.1	Key Issue mapping

This solution is for Key Issue #1, which enhances the PDU set based QoS handling by using the additional information (i.e., PDU set correlation info) provided by AS/AF for better congestion handling.

### 6.X.2	Description

For real time application, the stale data if received by the device may not be useful for rendering and is only wasting the battery on the UE due to transceiver usage and computation processing. Considered that AR glass is limited in form factor, any radio transmission that does not produce any meaningful outcome to end-user should be avoid.

The additional information comprising the PDU set information for this solution are labelled below with **. Other PDU Set information are carried from Rel-18.

The PDU Set Information comprises:

-	PDU Set Sequence Number.

-	Indication of End PDU of the PDU Set.

-	PDU Sequence Number within a PDU Set.

-	PDU Set Size in bytes.

-	PDU Set Importance, which identifies the relative importance of a PDU Set compared to other PDU Sets within a QoS Flow.

**	PDU Set correlation, which identifies how this PDU set is related or depended with other PDU Set within the same QoS flow.

For example, AS encodes video using 15fps with I,P1,P2,P3,I,… structure. Each frame is encoded as its own PDU set from AS. The decoder needs previously encoded I/P-frames for proper rendering.

In this example, if RAN discards the first I-frame during congestion then the following three P-frames are useless to the receiver/decoder. Likewise, if RAN discards P2 then P3 is also useless to the receiver/decoder.

NOTE: 	When frame(s) are missing, decoder has its own implementation on how to minimize the bad QoE to the end-user.

This solution proposes that AS can provide PDU Set correlation info as part of the PDU Set information to allow RAN to make better discarding decision during congestion.

Avoid sending useless payload to device can help prolong better life and also allowing RAN to use those available bandwidths in more effective manner.

### 6.X.3	Procedures

Rel 18 procedure for sending PDU set information to RAN is reused (i.e, via RTP extension header over N6 and GTP-U extension header over N3).

PDU Set correlation information has the following attributes:

a)	Related PDU set(s) are part of a PDU set group.

b)	Each group has a PDU set denotes a “key” PDU set.

c)	Dependent PDU set can indicate the following:

- 	Discard if “key” PDU set has failed to be delivered to receiver (e.g., due to congestion or retransmission timeout, etc).

-	Discard if previous PDU set has failed to be delivered to receiver.

-	Discard if PSDB can’t be met.

NOTE: 	Uplink handling is assumed to be UE implementation. I.e, During UL congestion, UE can select the key frame to send with the UL grant.

### 6.X.4	Impacts on services, entities and interfaces

AS/AF

- If required, include PDU Set correlation info as part of the PDU Set Information.

NOTE: RTP extension header will need to be extended by SA4.

UPF

- Map new PDU set info from N6 to N3 using GTP-U extension header.

RAN

- Utilize the PDU set correlation info for discarding decision during congestion.

END OF CHANGES