3GPP TSG-WG SA2 Meeting #160-Ad Hoc-e	S2-2401394

# Online, Jan 22 – 29, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0489 | rev | 1 | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update on GMLC and LMF Services |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Ranging_SL |  |  |  |  |  | Date: |  |  | 2024-01-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - In clause 5.6.3 of TS 23.586, the ranging AF may contact GMLC directly or via NEF to provide a ranging or sidelink positioning operation request to the 5GS. In clause 5.6.2.3 of TS 23.586, in the control plane-based procedure, the Client UE sends the 5GC-MO-LR request to AMF and this triggers the AMF to send the Ranging/SL positioning request to the GMLC for the Ranging/SL Positioning service.In both case, GMLC needs provide service for supporting ranging to AMF or NEF. This paper proposes to update GMLC services for supporting ranging. - In LMF functionality defined in clause 4.3.8 of TS 23.586, Interaction with GMLC to get the location of Located UE/SL Reference UE using the Application Layer ID. This paper proposes to update GMLC services to support LMF get the UE location using the Application Layer ID. - In LMF functionality defined in clause 4.3.8 of TS 23.586, Support of receiving stored Ranging/Sidelink Positioning capability from AMF and support of providing updated Ranging/Sidelink Positioning capability to AMF. This paper proposes to update LMF services for interact the Ranging/Sidelink Positioning capability. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a new service Ngmlc_Location_ProvideRanging to support ranging request from AMF or NEF, update GMLC services to support LMF get the UE location using the Application Layer ID, and update LMF services for interact the Ranging/Sidelink Positioning capability. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | GMLC and LMF services are not completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.4.1, 8.4.2.1, 8.4.2.2, 8.4.2.X(new), 8.3.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 8.4.1	General

The following table shows the GMLC Services and GMLC Service Operations.

Table 8.4.1-1: List of GMLC Services

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Ngmlc_Location | ProvideLocation | Request / Response | GMLC, NEF, NWDAF, LMF |
|  | LocationUpdate | Request / Response | AMF, GMLC |
|  | LocationUpdateNotify | Notify | NEF, NWDAF |
|  | CancellLocation | Request / Response | GMLC, NEF, NWDAF |
|  | EventNotify | Notify | GMLC, NEF, NWDAF |
|  | ProvideRanging | Request / Response | AMF, NEF |

# * * * * Second change * * * *

#### 8.4.2.1	General

Service description: This service enables an NF to request location determination for a target UE or to request relative locations, distance, or direction between UEs. The following are the key functionalities of this NF service.

-	Allow the consumer NF to request the current geodetic and optionally local and/or civic location of a target UE.

-	Allow the consumer NF to subscribe/unsubscribe the geodetic and optionally local and/or civic location of a target UE for some certain events.

-	Allow the consumer NF to cancel an on-going periodic or triggered location request of a target UE.

-	Allow the consumer NF to get notified about the geodetic and optionally local and/or civic location of a target UE when some certain events are detected.

-	Allow the consumer NF to request the relative locations, distance, or direction between UEs.

# * * * * Third change * * * *

#### 8.4.2.2	Ngmlc_Location_ProvideLocation service operation

Service operation name: Ngmlc_Location_ProvideLocation

Description: Provides UE location information to the consumer NF.

NOTE 1:	For deferred location request, this service operation is used to implicitly subscribe to the notification of the UE location information.

NOTE 2:	For bulk LCS service request from NEF to GMLC, this service operation is used to implicitly subscribe to the notification of UE location information

Input, Required: UE identifier (GPSI, SUPI, Application layer ID, Internal Group Identifier or External Group Identifier), Client Type.

Input, Optional: Required QoS instance(s), Supported GAD shapes, UE privacy requirements, LCS Client Identification, Service type, indication of requiring reliable UE location information, Notification Target Address, Notification Correlation ID, Event Type (defined in clause 4.1a.5.1), Scheduled Location Time, request for user plane reporting to an LCS Client or AF, user plane address of an LCS Client or AF, user plane security information for an LCS Client or AF, cumulative event report timer, maximum number of user plane event reports to an LCS Client or AF, event report expected area, reporting indication, LPHAP indication and:

-	For periodic event type, optional input further includes the time interval between successive location reports, the total number of reports, location QoS.

-	For area event type, optional input further includes target geographical area(s) with optionally associated required QoS instance, whether the event to be reported is the UE being inside, entering into or leaving the target area, the duration of event reporting, the minimum and maximum time intervals between successive event reports, the maximum event sampling interval, whether location estimates and associated location QoS shall be included in event reports, and whether only one location report is required or more than one.

-	For motion event type, optional input further includes the threshold linear distance, the duration of event reporting, the minimum and maximum time intervals between successive event reports, the maximum event sampling interval, whether location estimates and associated location QoS shall be included in event reports, and whether only one location report is required or more than one.

Output, Required: Success/Failure indication

Output, Optional: Geodetic location, Local Location including Coordinate ID, civic location, age of location, Indoor/Outdoor indication, LOS/NLOS measurement indication, position methods used (in the case of success indication provided), failure cause (in the case of failure indication provided), achieved Location QoS Accuracy, the timestamp of the Location.

See clauses 6.3.1 and 6.8 for examples of usage of this service operation.

# * * * * Fourth change * * * *

#### 8.4.2.X	Ngmlc_Location_ProvideRanging service operation

Service operation name: Ngmlc_Location_ProvideRanging

Description: Provides Ranging information to the consumer NF.

Input, Required: UE identifiers (GPSI, SUPI), Client Type.

Input, Optional: Required Ranging QoS, LCS Client Identification, Ranging type (Ranging for relative distance and/or direction), Scheduled Location Time and:

Output, Required: Success/Failure indication

Output, Optional: Ranging result, Ranging methods used (in the case of success indication provided), failure cause (in the case of failure indication provided), achieved Ranging QoS Accuracy, the timestamp of the Ranging result.

# * * * * Fifth changes * * * *

8.3.2.2	Nlmf_Location_DetermineLocation service operation

Service operation name: Nlmf_Location_DetermineLocation

Description: Provides UE location information to the consumer NF.

NOTE:	For deferred location request, this service operation is used to implicitly subscribe to the notification of the UE location information.

Input, Required: Client Type, LCS Correlation Identifier.

Input, Optional: Serving cell identifierof the Primary Cell in the Master RAN node and the Primary Cell in the Secondary RAN node when available based on Dual Connectivity scenarios if the UE is using 3GPP access, required Location QoS instance(s), Supported GAD shapes, service type, indication of requiring reliable UE location information, AMF identity if a UE associated Namf_Communication service is to be invoked by LMF, Type of request for a 5GC-MO-LR, Embedded LPP message(s) for a 5GC-MO-LR (1-3 messages), subscribed assistance data for a 5GC-MO-LR, Deferred location type, Deferred location parameters, indication if UE supports LPP or not, UE connectivity state per access type, Notification Target Address, Notification Correlation ID, indication of UE geographical area determination for PLMN selection verification, UE Positioning Capability, UE User Plane Positioning Capability, Ranging/Sidelink Positioning Capability, TNAPId (see TS 29.571 [33]), TWAPId (see TS 29.571 [33]), Scheduled Location Time, request for user plane reporting to an LCS Client or AF, user plane address of an LCS Client or AF, user plane security information for an LCS Client or AF, cumulative event report timer, maximum number of user plane event reports to an LCS Client or AF, event report allowed area, reporting indication, UE unaware indication, LPHAP indication, the serving cell identity belongs to a MBSR indication, IAB-UE ID of the MBSR.

Output, Required: Success/Failure indication

Output, Optional: Geodetic Location, Local Location including Coordinate ID, Civic Location, Indoor/Outdoor indication, LOS/NLOS measurement indication, Position Methods Used (in the case of success indication provided), Serving LMF identification, Failure Cause (in the case of failure indication provided), achieved Location QoS Accuracy, UE geographical area where a PLMN is or is not allowed to operate, UE Positioning Capability, Ranging/Sidelink Positioning Capability, the timestamp of the Location, indication that the location determination will be sent directly to GMLC.

See clause 6.1, clause 6.2. clause 6.3.1 and clause 6.9.1 for examples of usage of this service operation.

# * * * * End of changes * * * *