SA WG2 Meeting #160-Ad Hoc-e	S2-2401789

E-meeting, January 22 – 29, 2024	(revision of S2-2400540r01)

Source:	LG Electronics

Title:	KI#2, New Sol: Provision of Store and Forward Satellite Specific Parameters

Document for:	Approval

Agenda Item:	19.1

Work Item / Release:	FS_5GSAT_ARCH_Ph3 / Rel-19

Abstract of the contribution: This paper proposes a new solution for KI#2.

Discussion

TR 23.700-29 includes the following key issue:

- Key Issue #2: Support of Store and Forward Satellite operation

5.2	Key Issue #2: Support of Store and Forward Satellite operation

5.2.1	Description

S&F Satellite operation is especially suited for the delivery of delay-tolerant/non-real-time satellite services (i.e. CIoT/MTC, SMS). To support S&F Satellite operation for such services, it is proposed to study the following items:

-	If applicable, what are the minimum necessary set of core network elements/network functions that should be placed on board the satellite(s) for the intended service(s);

-	Whether and how to trigger S&F Satellite operation, and how to execute S&F Satellite operation;

-	What enhancements on the related UE and network procedures are needed to support S&F Satellite operation, including:

-	Whether to inform the UE when the S&F Satellite operation is applied or not.

NOTE 1:	S&F for IoT NTN will be studied first and if there is remaining time available NR NTN can then be studied.

NOTE 2:	Coordination with SA3-LI on LI aspects is needed.

NOTE 3: 	The candidate solutions need to indicate which service(s) they are addressing (CIoT CP Optimizations, CIoT UP Optimizations, SMS).

Proposal

It is proposed to agree the following changes into TR 23.700-29.

* * * * Start of 1st Change * * * *

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| Solutions | 1 | 2 |  |  |
| X |  | X |  |  |

* * * * Start of Next Change * * * *

All New Texts

6.X	Solution #X: Provision of Store and Forward Satellite Specific Parameters

6.X.1	Description

This solution considers the following key issue:

- Key Issue #2: Support of Store and Forward Satellite operation

Especially, it focuses on the provisioning procedure needed before performing S&F Satellite operation

In the existing 5GS, the provisioning capability allows an external party to provision the information, such as expected UE behaviour and service specific parameters. The service specific information consists of information to support the specific service in 5G system. Provisioned data can be used by the other NFs.

This solution reuses this provisioning capability for Store and Forward Satellite Specific Service, and defines the specific parameters for 5G system.

The Store and Forward Satellite Specific Parameters can be the parameters provided from an AF (of the operator or a trusted 3rd party) by invoking the Nnef_ParameterProvision Service as described in clause 4.15.6.2 of TS 23.502 [3].

Editor's note: 	Whether and what parameters to expose as Store and Forward Satellite Specific Parameters is FFS.

To minimize the existing system impacts, EPS can include or update these kinds of parameters as the subscription information, based on SLA with a trusted 3rd party.

Provisioned Store and Forward Satellite Specific Parameters can be used by the other NFs such as SMF, PCF, etc., for example, being forwarded as subscription information, and considered for the policy generation and enforcement.

6.X.2	Procedures

Figure 6.X.2-1 shows the overall procedure for Provision of Store and Forward Satellite Specific Parameters

Figure 6.X.2-1: Overall Procedure for Provision of Store and Forward Satellite Specific Parameters

- The AF provides one or more parameter(s) to be created or updated, or deleted in a Nnef_ParameterProvision_Create or Nnef_ParameterProvision_Update or Nnef_ParameterProvision_Delete Request to the NEF. The payload of the Nnef_ParameterProvision_Update Request may include Store and Forward Satellite Specific Parameters.

- If the AF is authorised by the NEF to provision the parameters, the NEF requests to create, update and store, or delete the provisioned parameters as part of the subscriber data via Nudm_ParameterProvision_Create, Nudm_ParameterProvision_Update or Nudm_ParameterProvision_Delete Request message, the message includes the provisioned data and NEF reference ID

NOTE:	The remaining steps of the procedures are based on Figure 4.15.6.2-1 of TS 23.502 [3].

6.X.3	Impacts on services, entities and interfaces

NEF and UDM service operations:

-	Adding new parameter “Store and Forward Satellite Specific Parameters” as Input parameter of Nnef_ParameterProvision and Nudm_ParameterProvision

* * * * End of Changes * * * *