3GPP SA WG2 Meeting #160AHE	S2-2400135r06

22 – 29 January, 2024, Electronic

Source:	NEC

Title:	KI#2, New Sol: Energy efficiency subscriptions and control

Document for:	Approval

Agenda Item:	19.4

Work Item / Release:	FS_EnergySys / Rel-19

Abstract of the contribution: The contribution proposes a solution to Key Issue #2: Subscription and policy control to support energy efficiency and energy saving as service criteria.

1. Introduction

This contribution proposes a solution to Key Issue #2: Subscription and policy control to support energy efficiency and energy saving as service criteria.

2. Proposal

It is proposed to agree the following solution to 3GPP TR 23.700-66.

* * * Start of 1st change * * * *

## 6.0	Mapping of Solutions to Key Issues

Editor's note:	This clause describes the mapping between solutions and key issues.

Table 6.0-1: Mapping of Solutions to Key Issues

| Solutions | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
|  | 1 | 2 | … | X |
| X |  | X |  |  |
| … |  |  |  |  |

* * * End of 1st change * * * *

* * * Start of 2nd change * * * *

## 6.x	Solution #X: Energy efficiency subscriptions and control

### 6.x.1	Key Issue mapping

The solution applies to Key Issue #2 Subscription and policy control to support energy efficiency and energy saving as service criteria.

### 6.x.2	Functional Description

Assumptions:

- This solution assumes that the network efficiency and the energy saving are controlled by a new network function EECF (Energy Efficiency Control Function) node. However, the solution equally applies if the EECF node is a functional part of an already existing NF.

- It is assumed that the EECF is subscribed with multiple NFs for energy efficiency and energy usage related information monitoring and collection, e.g. a load or a traffic level monitoring. The EECF may subscribe and obtain such information from the monitored NF itself (e.g. AMF or SMF) or via the OAM (e.g. RAN) or via the NWDAF depending on the type and availability of the required information.

- It is assumed that the monitored NF can function in two energy saving mode.

### 6.x.3	Procedures

Figure 6.X.3: Energy Efficiency subscriptions and control

1.	The EECF triggers a subscription request for network energy efficiency and usage related information monitoring by sending Nnf_NFLoadStatus_Subscribe Request to the NF to be monitored. The EECF may provide the following parameters:

- energy usage entity – defines the network entity for which the energy efficiency report is required. It can be NF, RAN, S-NSSAI, PDU Session, for example.

- load level – defines the type of the energy efficiency and usage information to be monitored and reported to the EECF, for example the load of the network entity in terms of number of registered UEs with the AMF or the traffic level in terms of number of established PDU Sessions with the SMF.

- reporting mode - defines whether the energy efficiency and usage related information to be reported on event basis (e.g. threshold based) or periodically.

- reporting threshold – if the reporting mode is per event basis, the EECF may provide a reporting threshold which may be expressed as a load or a traffic level for the monitored entity in percentage, for example.

- reporting period – if the reporting mode is periodic, the EECF may provide a reporting periodicity.

2.  The NF confirms the subscription request to the EECF with the Nnf_LoadStatus_Subscribe Response.

3.  The NF monitors and evaluates the energy efficiency and usage in terms of load or traffic level per granularity required by the EECF. Based on the reporting mode, the NF triggers a reporting notification to the EECF either based on a event fulfilment when the NF’s load or traffic level crosses the provided reporting threshold or at the expiry of a reporting period.

Editor’s note: whether the NF is capable to evaluate its own Energy Consumption in order to derive the the EE is FFS

4.	The NF sends the Neecf_EnergyUsageExposure_Notify to the EECF in which the NF includes the identity of monitored network entity in the energy usage entity parameter and the load or traffic level of the monitored network entity in the load level parameter.

5.   If the NF reported load or traffic level has fallen to or below a load threshold value for that NF (which load threshold value may be configured in the EECF by the operator or retrieved by the EECF from another NF like PCF or O&M or provided by the AF via the NEF, the EECF may request the monitored NF to move to energy saving mode. When the reported load or traffic level moves above the load threshold value, the EECF may request the NF to start saving energy.

6. The EECF sends Neecf_EnergyEfficiency_Control Request (EnergySavingState activation/NotEnergySavingState activation) message to the NF.

7. The NF moves to energy saving mode with further actions specific to the monitored NF.

NOTE 2: The details of how exactly the energy is saved in the energy saving modeis for Stage 3 to explore.

### 6.x.4	Impacts on existing services, entities and interfaces

EECF – a new Energy Efficiency Control Function or a new functionality within an existing NF.

NF – new services by multiple NF for energy efficiency related information monitoring and provision to the EECF.

* * * End of 2nd change * * * *