3GPP TSG-WG SA2 Meeting #160-Ad Hoc-e	S2-2400309r04

Online, Jan 22 – 29, 2024	(revision of S2-2313909)

# Title:	[DRAFT] Reply LS on Rel-18 RedCap enhancements to address remaining ENs in TS 23.502

# Response to:	LS R3-234725/S2-2400004 from RAN3

# Release:	Release 18

# Work Item:	NR_redcap_enh-Core

Source:	[Huawei to be] SA2

To:	RAN3, RAN2

Cc:	CT4

Contact Person:

Name:	Steven Wenham

Tel. Number:

E-mail Address:	steven DOT wenham AT huawei DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	23.501 CR5254 (Rel-18)

1. Overall Description:

SA2 thanks RAN3 for the LS (R3-234725/S2-2400004) on Rel-18 RedCap enhancements to address remaining ENs in TS 23.502.

- RAN3 would like to further inform SA2 that RAN3 agreed to rename the NGAP “DL data notification” message to “RAN paging request” message. RAN3 also agreed to include the ‘DL signalling indication’ in the “RAN paging request” message to differentiate DL signalling and DL data.

Answer 1: SA2 thanks RAN3 for the information and SA2 has approved related CRs to correct our specs accordingly.

- In addition, RAN3 would like to inquire whether there is need to include the ‘eRedCap indication’ in the NGAP INITIAL UE MESSAGE message, 	to enable possible charging and policy control and differentiation of eRedCap devices, as was done in the past releases for Rel-17 RedCap UEs.

Answer 2:

SA2 considers it useful to include an eREDCAP indication the NGAP INITIAL UE MESSAGE for use for e.g. subscription based access restrictions (for which SA2 has approved the attached CRs), etc.

SA2 understands that the access stratum can differentiate the different types of UEs, i.e., RedCap vs. eRedCap and the UE radio capabilities are independently defined for RedCap UEs and eRedCap UEs.

As the radio capabilities are independently defined for a RedCap UE and an eRedCap UE the NAS Mobility Registration Update with UE Radio Capability Update as described in TS 23.501 clause. 5.4.4.1, would be triggered if a change occurred.

SA2 would like to understand whether it is possible that a UE can have both REDCAP and eREDCAP capabilities and can change from a REDCAP UE to an eREDCAP UE (and vice versa)?

If the UE can change from a REDCAP UE to an eREDCAP UE (and vice versa), what is the expected behaviour of a UE with regards to REDCAP/eREDCAP indication stated above in access stratum and provided during the RRC connection establishment procedure?

2. Actions:

To RAN3 group.

ACTION: 	SA2 asks RAN3 group to take the above into account and provide feedback if any.

To RAN2 group.

ACTION: 	SA2 asks RAN2 group to answer to the questions and provide feedback if any.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #161		26th February – 1st March, 2024		Athens, GR

TSG-SA2 Meeting #162		15th – 19th April, 2024			TBC, CN