3GPP TSG-WG SA2 Meeting #160-AH	S2-2400084r03

# Online, January 22-29, 2024

Source: 	China Mobile

Title: 	New Solution: Energy related information collection and exposure

Document for: 	Approval

Agenda Item: 	19.4

Work Item / Release:	FS_EnergySys / Rel-19

Abstract of the contribution: Propose a new solution for KI#1 on Network energy related information exposure.

1	Discussion

This paper proposes a new solution on KI#1.

To support network energy related information exposure, the following aspects are to be studied:

-	Whether and what network energy related information can be exposed.

-	At what granularity (e.g. per network slice, UE, NF, PDU Session, QoS flow, etc) the network energy related information can be exposed.

-	How the network energy related information is exposed.

-	How and what network energy related information from the Network entities (i.e. RAN nodes, 5GC NFs) can be obtained in order to support network energy related information exposure.

The r01 of S2-2400084 tries to merge the following proposals in the way that:

S2-2400134: the introduction of more parameters in from S2-2400134 in the step 1a request.

S2-2400233: in 4b, adding the alternative way of retrieving SMF information from UDM

S2-2400836: adding the description in 1e that For the PDU session level request, the NEF/EEF retrieves the PCF for the PDU session via BSF. For the UE level request, the NEF/EEF retrieves the PCF of each PDU session of UE via UDM

S2-2401069: adding the two more granularity in the AF request

S2-2401371: adding the the description of another example on EECF doing the calculation of energy consumption information when it receive multiple EC information from multiple resources for a same granularity.

2 Proposal

It is proposed to include the below changes into TR 23.700-66 v0.2.0.

FIRST CHANGE (all the text is new)

6.0	Mapping of Solutions to Key Issues

Editor's note:	This clause describes the mapping between solutions and key issues.

Table 6.0-1: Mapping of Solutions to Key Issues

| Solutions | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | ... |
| Solution x: Energy related information collection and exposure | X |  |  |  |
| … |  |  |  |  |

NEXT CHANGE (all the text is new)

6.x	Solution #X: Energy related information collection and exposure

6.x.1	Key Issue mapping

This paper proposes a new solution on KI#1.

6.x.2	Functional Description

This solution provides a framework of energy related information collection and exposure. Figure 6.x.2-1 describes the architecture of this solution.

Figure 6.x.2-1: Architecture assumption

To achieve the centralized collection and uniform handling for the energy consumption/efficiency related information, a new NF, Energy Efficiency Control Function (EECF), is introduced. The EECF includes the following functionalities:

Editor’s note: whether there is a need for a new 5GC NF or whether proposed functionality can be provided by existing NFs will be evaluated during evaluation and conclusion phase

-	Collection of the information that have impact on energy consumption/efficiency from other 5GC NFs and OAM. The information can be collected from 5GC NFs at the granularity of per NF, per S-NSSAI, per UE, per UE per service, etc., while the information collected from OAM can be per NF and per S-NSSAI. This solution considers the following information which impact energy consumption/efficiency:

-	Collecting from AMF: number of registered subscribers; for each registered UE: UE specific DRX values, paging time window, paging area, periodic registration update timer.

-	Collecting from SMF: number of PDU Sessions; for each PDU Session: QoS parameters.

-	Collecting from UPF: data volume, bit rate.

-	Collecting from NG-RAN through OAM: data volume, signal strength.

-  Collecting from OAM: renewable energy information, e.g. based on geographic areas, where networks deployed in different areas may utilize the energy derived from different sources such as the Sun, wind, rivers, etc., with differentiated proportions of consumption.

-	Expose the energy consumption/efficiency related information based on the request from 5GC NF, e.g., AF.

-	Storage, organization and calculation of energy consumption/efficiency related information. The calculation of energy consumption/efficiency is based on the associated information above at the granularity of per NF, per S-NSSAI, per UE, per UE per service, per PDU Session or per QoS level, etc. For example, the energy consumption of one particular service can be obtained by calculating the ratio of data volume of the corresponding PDU Session to the total data volume of the entire slice, and multiplying it by the energy consumption of the slice. Or if the energy consumption information EECF received from the RAN (via OAM) and the UPF are for the same UE, the EECF can aggregate the energy consumption information for each UE.

6.x.3	Procedures

AMF

UPF

SMF

EECF

OAM

AF

1a. Nnef_EventExposure_Subscribe Request

1b. Neecf_EnergyConsumpExposure_Subscribe Request

1c. Neecf_EnergyConsumpExposure_Subscribe Response

Nsmf_EnergyInfoCollect Request

Nsmf_EnergyInfoCollect Response

Nsmf_UPF_EnergyInfoCollect Request

Nupf_EnergyInfoCollect Response

Namf_EnergyInfoCollect Request

Namf_EnergyInfoCollect Response

5. Neecf_EnergyConsumpExposure_Notify

6.Nnef_EnergyConsumpExposure_Notify

4a. Obtain energy consumption information from OAM

SMF EC information collection

UPF EC information collection

AMF EC information collection

Nupf_EnergyInfoCollect Request

Nsmf_UPF_EnergyInfoCollect Response

BSF

PCF

NEF

1e. Nbsf_Management_Discovery

2. Npcf_PolicyAuthorization_Create/Update request

3. Npcf_PolicyAuthorization_Create/Update response（ID） Request

1. Trigger based on local operator policy

1d. Nnef_EventExposure_Subscribe Response

4b.Obtain energy consumption information from SMF

4c.Obtain energy consumption information from UPF

4d.Obtain energy consumption information from AMF

UDM

Figure 6.x.3-1: Procedure for Energy related information collection and exposure

If AF triggers the exposure of the energy consumption related information, then steps 1a, 1b, 1c, 1d, 1e are executed, otherwise step 1 is executed.

1a.	If AF triggers subscription of NEF on energy consumption related information with Nnef_EventExposure_Subscribe Request, in the request, AF provides the subscription information including Energy related information category, Energy related information granularity (e.g. per NF, per S-NSSAI, per UE, per UE per service, per PDU Session or per QoS level, etc), Location, reporting mode (defines whether the energy efficiency report to be provided on event basis or periodically)，reporting threshold (if the reporting mode is per event basis) and Time period (if the reporting mode is periodic).

1b. NEF send subscription request to EECF with Neecf_EnergyConsumpExposure_Subscribe Request.

1c. EECF confirms the request and send subscription confirmation to NEF with Neecf_EnergyConsumpExposure_Subscribe Response.

1d. NEF confirms the request to AF with Nnef_EventExposure_Subscribe Response.

1e. [Conditional]For the PDU session level request, the NEF/EECF finds the PCF for the PDU session via BSF. For the UE level request, the NEF/EECF finds the PCF of each PDU session of UE via UDM. If AF sends the AF request including the energy consumption related information per UE or per UE per service, EECF invokes Nbsf_Management_Discovery service operation (providing at least the UE address) to find out the address of the relevant PCF if the PCF address is not available. BSF provides the PCF address in the Nbsf_Management_Discovery response to EECF.

1. EECF is triggered to perform energy consumption/efficiency optimization based on the local operator policy.

2. EECF invokes the Npcf_PolicyAuthorization service to the PCF to request the NF ID associated to the energy consumption related information exposure.

3. The PCF processes the Npcf_PolicyAuthorization_Create/Update request and sends the requested NF ID in the Npcf_PolicyAuthorization_Create/Update response.

Editor Note: If corresponding NFs (e.g. AMF, SMF, UPF) change, how to notify EECF with the target NF ID is FFS.

4a. EECF may interact with OAM to obtain energy consumption related information per NF or per (DNN, S-NSSAI), e.g. energy consumption information of RAN, renewable energy information for an area.

4b. EECF interacts with NRF to request the NF ID, alternatively, EECF can also request the UDM to get the information of serving SMF through UDM_UECM_get service. EECF requests energy consumption related information from SMF with Nsmf_EnergyInfoCollect Request. SMF collects parameters including number of PDU Sessions, overall energy consumption of SMF instance. Optionally, SMF calculates energy consumption related information based on parameters above. SMF responses energy consumption related information to EECF with Nsmf_EnergyInfoCollect Response

4c. EECF interacts with NRF to request the NF ID. EECF requests energy consumption related information of UPF from SMF with Nsmf_UPF_EnergyInfoCollect Request. SMF requests energy consumption related information of UPF with Nupf_EnergyInfoCollect Request. UPF collects parameters data volume, bit rate. Optionally, UPF calculates energy consumption related information based on parameters. UPF responses energy consumption related information to SMF with Nupf_EnergyInfoCollect Response. SMF responses energy consumption related information of UPF to EECF with Nsmf_UPF_EnergyInfoCollect Response.

4d. EECF interacts with NRF to request the NF ID. EECF requests energy consumption related information from AMF with Namf_EnergyInfoCollect Request. AMF collects parameters number of registered subscribers. Optionally, AMF calculates energy consumption related information based on parameters. AMF responses energy consumption related information to EECF with Namf_EnergyInfoCollect Response.

5. EECF may calculate the energy consumption/efficiency based on the collected information. Based on the results, EECF send the energy related information requested by AF to NEF with Neecf_EnergyConsumpExposure_Notify.

6. NEF sends the energy related information requested by AF to AF with Nnef_EnergyConsumpExposure_Notify.

6.x.4	Impacts on existing services, entities and interfaces

EECF: New NF. Functionalities are as description of clause 6.x.2.

PCF: Providing the corresponding NF ID to EECF for energy consumption related information exposure.

AMF: Exposure the following information to EECF: number of registered subscribers, UE specific DRX values, paging time window, paging area, periodic registration update timer.

SMF: Exposure the following information to EECF: number of PDU Sessions, QoS parameters.

UPF: Exposure the following information to EECF: data volume, bit rate.

NG-RAN: Exposure the following information through OAM to EECF: data volume.

End of CHANGE