3GPP TSG-SA WG2 Meeting #160-Ad Hoc-e	S2-2400123r01

# Elbonia, January 22 – 29, 2024									                         (revision of S2-2312333)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0986 |  | rev |  | 1 |  |  | Current version: |  |  | 18.4.0 |  |  |  |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposed change affects: |  |  |  | UICC apps |  |  |  | ME |  |  | Radio Access Network |  |  | Core Network | x |  |

| Title: | Correction of accuracy metrics in ML model monitoring service operations |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph3 |  |  |  |  |  | Date: |  |  | 2024-01-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | It was agreed in Rel-18 that only the “Accuracy” is supported as the ML Mode/Analytics performance metric; however, the existing texts in procedures and services operation are misleading and implying possibility of other metrics as well. This CR clarifies that only the “Accuracy” metrics is supported in Rel-18. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding a NOTE mentioning that Accuracy is the only supported metric. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistency in the specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5C.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

********** First Change **********

## 5C.1	General

A NWDAF has the accuracy checking capability of Analytics and/or ML Model, where the NWDAF may provide the accuracy information to consumers when requested or use it for its internal processes.

Input data is collected from Data Producer NF when there is a request for inference/prediction per analytics ID in NWDAF for specific time period in future while ground truth data are collected from Data Producer NF corresponding to analytic ID requested at the time which the prediction refers to.

The ground truth data is the actual measured data observed at the time which the prediction refers to.

NOTE 1:	The ground truth data can be impacted when Analytics Feedback Information shows there is action triggered by the analytics output in the consumer.

Analytics/ML Model Accuracy Monitoring is to be achieved by comparing the predictions using the current trained ML model and its corresponding ground truth data i.e. the corresponding true observed events.

Analytics/ML Model Accuracy information is to represent a performance measure for analytics and ML Model respectively, which are composed of the number of correct predictions out of all predictions and the corresponding number of samples.

NOTE X: Only the Accuracy is supported as the Analytics/ML Model performance metric(s) in this release.

NOTE 2:	How MTLF/AnLF determines whether the prediction is correct one is up to implementation.

The NWDAF (containing AnLF/MTLF) with accuracy checking capability decides to initiate analytics accuracy monitoring based on:

-	A request from an analytics accuracy consumer. The analytics accuracy consumer may be an NWDAF containing AnLF, NWDAF containing MTLF or an analytics consumer NF.

-	Analytics Feedback Information which may be provided by an Analytics Consumer NF.

The AnLF with analytics accuracy checking capability as defined in clause 6.2D is able to provide or notify the accuracy information of Analytics IDs to the analytics consumers of such service and when the analytics accuracy does not meet the analytics consumer's requirements, the analytics consumer may stop using analytics for a period of time or obtain new analytics. In addition, updated analytics for the provided Analytics IDs may be provided to analytics consumers as requested, if the updated analytics is able to be generated within the correction time period. The AnLF with analytics accuracy checking capability is as defined in clause 6.2D.1 is able to determine analytics accuracy information based on e.g.:

-	Comparing predictions generated based on a ML model and its corresponding ground truth data

-	Determining analytics accuracy by comparing analytics accuracy using multiple ML models

The MTLF with ML Model accuracy checking capability as defined in clause 6.2E is able to determine ML Model degradation based on e.g.:

-	comparing/evaluating the data: including input data, analytics output and the ground truth data either collected from various data source NFs, DCCF, AnLF, ADRF or configured by OAM;

-	or AnLF providing notifications of the analytics accuracy information; or

-	AnLF providing analytics feedback information of the analytics generated by the ML model.

#### The NWDAF containing MTLF may reselect a new ML model or retrain the existing ML model and consequently notify the ML model accuracy degradation to the ML Model consumer(s). In addition, the NWDAF containing MTLF may consider the rating of untrusted AF(s) when used as data sources.

********** End of Changes **********