3GPP TSG-WG SA2 Meeting #160-Ad Hoc-e	S2- 2400283

# Online, Jan 22 – 29, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1025- | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on ML Model retrieval |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph3 |  |  |  |  |  | Date: |  |  | 2023-01-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Part1. As described in Annex X.10 of TS 33.501, allowd NF list should be used for authorization in the procedure for secured and authorized AI/ML model sharing between different vendors: NWDAF containing MTLF triggers the Nadrf_MLModelManagement_StorageRequest as described in TS 23.288 [105], optionally including an allowed NFc list. The absence of allowed NFc list indicates that only the MTLF which stored the model is allowed to retrieve the model. Currently, allowed NF list is missing procedure for ML model storage. Besides, the MTLF may storage multiple models via Nadrf_MLModelManagement_Storage service operation and these models may be related to different Analytics ID(s). In other words, the MTLF may only authorizes the NF consumer to retrieve certain ML model(s), not all models included in a storage Transaction ID. Thus in this case, the storage transaction Id shall not provide to the consumer. Part2. The names of ADRF services (i.e., Nadrf_MLModelManagement_StorageRequest and Nadrf_MLModelManagement_RetrievalRequest) in clause 6.2B.7 are not aligned with the definition. As agreed that a MTLF can request another MTLF to provide ML model by using the ML model provision service. Therefore, this CR proposes to clarify that the Nnwdaf_MLModelProvision_Subscribe / Nnwdaf_MLModelInfo_Request service will be used when the consumer MTLF requests from another MTLF the model which is to be retrieved from ADRF. Part3. Nnwdaf_MLModelProvision_Notify service can be used for model notify and/or update accuracy information. How does the NFc could determine the initial purpose of the message whether contains the model notify so as to trigger the procedure of model retrieval. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed that add Allowd NF list in the prodecure of ML model storage. Correct the names of ADRF services in clause 6.2B.7. Remove the description that the Nnwdaf_MLModelTraining_Subscribe service / Nnwdaf_MLModelTrainingInfo_Request will be used when the consumer is NWDAF containing MTLF. Add a ML model provide indicator in the Nnwdaf_MLModelProvision_Notify message. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The prodecure of ML model storage is not aligned with TS 33.501 and inconsistent descriptions about the ADRF service and ML model provision |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2A.1, 6.2A.2, 6.2B.5, 6.2B.7, 10.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR adds a new reference to TS 33.501. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[4]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System; Stage 2".

[5]	Void.

[6]	3GPP TS 28.532: "Management and orchestration; Generic management services".

[7]	3GPP TS 28.550: "Management and orchestration; Performance Assurance".

[8]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[9]	3GPP TS 28.545: "Management and orchestration; Fault Supervision (FS)".

[10]	3GPP TS 28.554: "Management and orchestration; 5G end to end Key Performance Indicators (KPI)".

[11]	ITU-T Recommendation P.1203.3: "Parametric bitstream-based quality assessment of progressive download and adaptive audiovisual streaming services over reliable transport - Quality integration module".

[12]	3GPP TS 38.215: "NR; Physical layer measurements".

[13]	Void.

[14]	3GPP TS 38.331: "NR; Radio Resource Control (RRC) protocol specification".

[15]	3GPP TS 36.331: "Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification".

[16]	3GPP TS 38.413: "NG-RAN; NG Application Protocol (NGAP)".

[17]	3GPP TS 29.244: "Interface between the Control Plane and the User Plane Nodes".

[18]	3GPP TS 29.510: "5G System; Network function repository services; Stage 3".

[19]	3GPP TS 28.533: "Management and orchestration; Architecture framework".

[20]	3GPP TS 37.320: "Radio measurement collection for Minimization of Drive Tests (MDT); Overall description; stage 2".

[21]	3GPP TS 28.201: "Charging management; Network slice performance and analytics charging in the 5G System (5GS); stage 2".

[22]	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

[23]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

[24]	3GPP TS 28.310: "Management and orchestration; Energy efficiency of 5G".

[25]	3GPP TS 29.518: "5G System; Access and Mobility Management Services; Stage 3".

[26]	3GPP TS 29.503: "Unified Data Management Services; Stage 3".

[27]	3GPP TS 26.114: "IP Multimedia Subsystem (IMS); Multimedia Telephony; Media handling and interaction".

[28]	3GPP TS 26.247: "Transparent end-to-end Packet-switched Streaming Service (PSS); Progressive Download and Dynamic Adaptive Streaming over HTTP (3GP-DASH)".

[29]	3GPP TS 26.118: "Virtual Reality (VR) profiles for streaming applications".

[30]	3GPP TS 26.346: "Multimedia Broadcast/Multicast Service (MBMS); Protocols and codecs".

[31]	3GPP TS 26.512: "5G Media Streaming (5GMS); Protocols".

[32]	3GPP TS 26.531: "Data Collection and Reporting; General Description and Architecture".

[33]	3GPP TS 22.261: "Service requirements for the 5G system; Stage 1".

[34]	3GPP TS 23.032: "Universal Geographical Area Description (GAD)".

[35]	3GPP TS 22.071: "Technical Specification Group Systems Aspects; Location Services (LCS)".

[36]	3GPP TS 29.508: "5G System; Session Management Event Exposure Service; Stage 3".

[37]	3GPP TS 29.572: "5G System; Location Management Services; Stage 3".

[38]	GSMA TS.06: "IMEI Allocation and Approval Process".

[39]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

[40]	ITU-T Y.1540: "Internet protocol data communication service - IP packet transfer and availability performance parameters".

[41]	3GPP TS 28.622: "Telecommunication management; Generic Network Resource Model (NRMs). Integration Reference Point (IRP): Information Service (IS)".

[42]	3GPP TS 32.422: "Subscriber and equipment trace: Trace control and configuration management".

[43]	3GPP TS 26.532: "Data Collection and Reporting; Protocols and Formats".

[44]	3GPP TS 38.455: "NG-RAN; NR Positioning Protocol A (NRPPa)".

[45]	3GPP TS 28.104: "Management and orchestration; Management Data Analytics (MDA)".

[46]	3GPP TS 28.537: "Management and orchestration; Management capabilities".

[47]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2".

[48]	3GPP TS 29.515: "Gateway Mobile Location Services; Stage 3".

[xx]	3GPP TS 33.501: "Security architecture and procedures for 5G system; Stage 2".

# * * * * Next change * * * *

6.2A.1	ML Model Subscribe/Unsubscribe

The procedure in Figure 6.2A.1-1 is used by an NWDAF service consumer, i.e. an NWDAF containing AnLF to subscribe/unsubscribe at another NWDAF, i.e. an NWDAF containing MTLF, to be notified when ML Model Information on the related Analytics becomes available, using Nnwdaf_MLModelProvision services as defined in clause 7.5. The ML Model Information is used by an NWDAF containing AnLF to derive analytics. The service is also used by an NWDAF to modify existing ML Model Subscription(s). An NWDAF can be at the same time a consumer of this service provided by other NWDAF(s) and a provider of this service to other NWDAF(s).

Figure 6.2A.1-1: ML Model for analytics subscribe/unsubscribe

1.	The NWDAF service consumer (i.e. an NWDAF containing AnLF) subscribes to, modifies, or cancels subscription for a (set of) trained ML Model(s) associated with a/an (set of) Analytics ID(s) by invoking the Nnwdaf_MLModelProvision_Subscribe / Nnwdaf_MLModelProvision_Unsubscribe service operation. The parameters that can be provided by the NWDAF service consumer are listed in clause 6.2A.2. The service consumer optionally indicates its support for multiple ML models if available.

When a subscription for a trained ML model associated with an Analytics ID is received, the NWDAF containing MTLF may:

-	determine whether existing trained ML Model(s) can be used for the subscription; or

-	determine whether triggering further training for the existing trained ML models is needed for the subscription.

If the NWDAF containing MTLF determines that further training is needed, this NWDAF may initiate data collection from NFs, (e.g. AMF/DCCF/ADRF), UE Application (via AF) or OAM as described in clause 6.2, to generate the ML model.

If the service invocation is for a subscription modification or subscription cancelation, the NWDAF service consumer includes an identifier (Subscription Correlation ID) to be modified in the invocation of Nnwdaf_MLModelProvision_Subscribe.

2.	If the NWDAF service consumer subscribes to a (set of) trained ML model(s) associated to a (set of) Analytics ID(s), the NWDAF containing MTLF notifies the NWDAF service consumer by invoking Nnwdaf_MLModelProvision_Notify service operation with:

-	a set of pair(s) of unique ML Model Identifier and ML Model Information associated with each Analytics ID requested by the service consumer.

NOTE 1:	The structure and format of the ML Model identifier and its uniqueness are up to stage 3.

NOTE 2:	Parameters defined for Multiple ML models are for Analytics accuracy enhancement.

The content of trained ML Model Information that can be provided by the NWDAF containing MTLF is specified in clause 6.2A.2.

The NWDAF containing MTLF also invokes the Nnwdaf_MLModelProvision_Notify service operation to notify an available re-trained ML model when the NWDAF containing MTLF determines that the previously provided trained ML Model required re-training at step 1.

When the step 1 is for a subscription modification (i.e. including Subscription Correlation ID), the NWDAF containing MTLF may provide either a new trained ML model different to the previously provided one, or re-trained ML model by invoking Nnwdaf_MLModelProvision_Notify service operation.

When the content includes ML model provide indicator, the NWDAF service consumer (i.e. an NWDAF containing AnLF) triggers ML model retrieval procedure (e.g. clause 6.2B.7) to retrieve ML model(s).

# * * * * Next change * * * *

6.2A.2	Contents of ML Model Provisioning

The consumers of the ML model provisioning services (i.e. an NWDAF containing AnLF) as described in clause 7.5 and clause 7.6 may provide the input parameters as listed below:

-	Information of the analytics for which the requested ML model is to be used, including:

-	A list of Analytics ID(s): identifies the analytics for which the ML model is used.

-	[OPTIONAL] NF consumer information: identifies the vendor of NWDAF containing AnLF.

NOTE 1:	NF consumer information such as Vendor ID is defined in Stage 3.

-	[OPTIONAL] Use case context: indicates the context of use of the analytics to select the most relevant ML model ML model.

NOTE 2:	The NWDAF containing MTLF can use the parameter "Use case context" to select the most relevant ML model, when several ML models are available for the requested Analytics ID(s). The values of this parameter are not standardized.

-	[OPTIONAL] ML Model Interoperability Information. This is vendor-specific information that conveys, e.g., requested model file format, model execution environment, etc. The encoding, format, and value of ML Model Interoperable Information is not specified since it is vendor specific information, and is agreed between vendors, if necessary for sharing purposes.

-	[OPTIONAL] ML Model Filter Information: enables the NWDAF containing MTLF to select which ML model for the analytics is requested, e.g. S-NSSAI, Area of Interest. Parameter types in the ML Model Filter Information are the same as parameter types in the Analytics Filter Information which are defined in procedures.

-	[OPTIONAL] Target of ML Model Reporting: indicates the object(s) for which ML model is requested, e.g. specific UEs, a group of UE(s) or any UE (i.e. all UEs).

-	[OPTIONAL] Requested representative ratio: a minimum percentage of UEs in the group whose data is a non-empty set and can be used in the model training when the Target of ML Model Reporting is a group of UEs.

-	ML Model Reporting Information with the following parameters:

-	(Only for Nnwdaf_MLModelProvision_Subscribe) ML Model Reporting Information Parameters as per Event Reporting Information Parameter defined in Table 4.15.1-1, TS 23.502 [3].

-	[OPTIONAL] ML Model Target Period: indicates time interval [start, end] for which ML model for the Analytics is requested. The time interval is expressed with actual start time and actual end time (e.g. via UTC time).

-	[OPTIONAL] Inference Input Data information: contains information about various settings that are expected to be used by AnLF during inferences such as:

-	the "Input Data" that are expected be used, each of them optionally accompanied by metrics that show the granularity with which this data will be used (i.e., a sampling ratio, the maximum number of input values, and/or a maximum time interval between the samples of this input data).

NOTE 3:	This can be a subset of the possible Input Data specified for a certain analytics type.

-	the data sources that are expected to be used as a list of NF instance (or NF set) identifiers.

-	A Notification Target Address (+ Notification Correlation ID) as defined in clause 4.15.1 of TS 23.502 [3], allowing to correlate notifications received from the NWDAF containing MTLF with this subscription.

-	[OPTIONAL] Indication of supporting multiple ML models.

-	[OPTIONAL] Accuracy level(s) of Interest.

-	[OPTIONAL] Number of ML model(s), indicating the maximum number of ML models that the NWDAF containing MTLF could provide to the NWDAF containing AnLF.

NOTE 4:	Multiple ML models Filter Information are composed by Accuracy level(s) of Interest and Number of ML model(s).

-	[OPTIONAL] Time when model is needed: indicates the latest time when the consumer expects to receive the ML model(s).

-	[OPTIONAL] ML Model Monitoring Information:

-	[OPTIONAL] ML Model metric: i.e. ML Model Accuracy.

-	[OPTIONAL] ML model monitoring reporting mode: such as Accuracy reporting interval or pre-determined status. Depending on the reporting mode, the NWDAF containing MTLF reports the model accuracy to NWDAF containing AnLF either periodically or when the ML model accuracy is crossing an ML Model Accuracy threshold, i.e. the accuracy either becomes higher or lower than the ML Model Accuracy threshold.

-	[OPTIONAL] ML Model Accuracy Threshold: indicating the accuracy threshold of the ML Model requested by the consumer (as a kind of pre-determined status). It also can be used as an indication that the MTLF is triggered to execute the accuracy monitoring operations for the ML Model provisioned to AnLF.

-	[OPTIONAL] DataSetTag and ADRF ID if available: indicates the inference data (including input data, prediction and the ground truth data at the time which the prediction refers to) stored in ADRF which can be used by MTLF to retrain or reprovision of the ML model.

-	[OPTIONAL] ML Model Identifier: indicates the Model that the data corresponding to the DataSetTag is related to (in the case of subscription modification).

The NWDAF containing MTLF provides to the consumer of the ML model provisioning service operations as described in clause 7.5 and 7.6, the output information as listed below:

-	(Only for Nnwdaf_MLModelProvision_Notify) The Notification Correlation Information.

-	For each Analytics ID requested by the service consumer, a set of pair (s) of unique ML Model identifier and the following information.

-	ML Model Information, which includes:

-	the ML model file address (e.g. URL or FQDN); or

-	ADRF (Set) ID.

When ADRF (Set) ID is provisioned and the MTLF authorizes the NF Service Consumer to retrieve all ML models corresponding to a Storage Transaction ID, a Storage Transaction ID may also be provisioned.

-    [OPTIONAL] ML model provide indicator: indicates that the ML model corresponding to the ML Model identifier is provided or updated (e.g. new ML model or re-trained ML model).

-	[OPTIONAL] ML model degradation indicator: indicates whether the provided ML model is degraded.

-	[OPTIONAL] Validity period: indicates time period when the provided ML Model Information applies.

-	[OPTIONAL] Spatial validity: indicates Area where the provided ML Model Information applies.

NOTE 5:	Spatial validity and Validity period are determined by MTLF internal logic and it is a subset of AoI if provided in ML Model Filter Information and of ML Model Target Period, respectively.

-	[OPTIONAL] ML model representative ratio: indicating the percentage of UEs in the group whose data is used in the ML model training when the Target of ML Model Reporting is a group of UEs.

-	[OPTIONAL] Training Input Data Information: contains information about various settings that have been used by MTLF during training, such as:

-	the "Input Data" that have been used, each of them optionally accompanied by metrics that show the data characteristics and granularity with which this data has been used (i.e. a sampling ratio, the maximum number of input values and/or a maximum time interval between the samples of this input data, data range including maximum and minimum values, mean and standard deviation and data distribution when applicable) and the time, i.e. timestamp and duration, when this data was obtained.

-	the data sources related to the "Input Data" that were used for ML model training, which have been identified by a list of NF instance (or NF set) identifiers.

NOTE 6:	This can be a subset of the possible Input Data specified for a certain analytics type.

NOTE 7:	Data source information enables ML Model selection when different models are available for an Analytics ID, or it enables a consumer to avoid selecting a ML model that used data from a specific data source at a particular time or used data characterized by specific data characteristics.

-	[OPTIONAL]ML Model Accuracy Information: indicates the accuracy of the ML model if ML Model accuracy threshold is requested, which includes:

-	the accuracy value of the ML model.

-	[OPTIONAL] ML model metric (i.e. ML Model Accuracy).

# * * * * Next change * * * *

6.2B.5	ML Model Storage in ADRF

The procedure depicted in figure 6.2B.5-1 is used by NWDAF containing MTLF to store ML model in an ADRF.

Figure 6.2B.5-1: ML Model Storage in ADRF

0.	NWDAF containing MTLF determines to store ML model in ADRF based on MTLF policy.

1.	NWDAF containing MTLF requests to store a (set of) ML model(s) to the ADRF by invoking Nadrf_MLModelManagement_StorageRequest service, optionally including allowed NF instance list for the ML Model identifier as described in TS 33.501 [xx].

2.	The ADRF locally maintains the association between the ML Model identifier and the NF instance ID of NWDAF containing MTLF and allowed NF instance list (if there is any in step1).

[Optional] If instead of the ML model(s), the ML model address(es) is/are included in request, ADRF downloads the ML model(s) based on the ML model address(es) and locally stores the ML model(s).

3.	The ADRF sends Nadrf_MLModelManagement_StorageRequest Response message to the consumer including the ML Model storage result indication.

# * * * * Next change * * * *

6.2B.7	ML model retrieval from ADRF

The procedure depicted in Figure 6.2B.7-1 is used by consumers (NWDAF containing MTLF and NWDAF containing AnLF) to retrieve ML models from an ADRF.

Figure 6.2B.7-1: Procedure for ML model(s) retrieval from ADRF

1.	The NWDAF service consumer (NWDAF containing AnLF or NWDAF containing MTLF) subscribes/requests for a (set of) trained ML Model(s) associated with a/an (set of) Analytics ID(s) by invoking the Nnwdaf_MLModelProvision_Subscribe / Nnwdaf_MLModelInfo_Request service.

2.	The NWDAF containing MTLF determines whether the set of ML Model(s) associated with a/an (set of) Analytics ID(s) should be retrieved from the ADRF.

When NWDAF containing MTLF authorizes the NF consumer to retrieve the ML model(s) stored in the ADRF directly, steps 3 and 4 is skipped.

If NWDAF containing MTLF determines that the set of ML Model(s) corresponding Analytics ID(s) requested in step 1 needs to be retrieved from ADRF and the NF consumer is agnostic to where the ML model(s) is stored, then Steps 3 and 4 is performed.

NOTE:	How NWDAF containing MTLF and ADRF authorizes the NF Consumer is in scope of SA WG3.

3.	The ADRF service consumer (NWDAF containing MTLF) requests for the ML model stored in ADRF by invoking the Nadrf_MLModelManagement_RetrievalRequest Request (Storage Transaction Identifier or one or more unique ML model Identifier(s)) service operation.

4.	The ADRF verifies the service consumer (NWDAF containing MTLF) as described in Annex X.10 of TS 33.501 [xx]. If verification is successful, the ADRF sends Nadrf_MLModelManagement_RetrievalRequest Response (address(es) e.g., URL or FQDN of Model file(s) stored in ADRF) service operation.

5.	The NWDAF containing MTLF notifies/ response to the NWDAF service consumer with the tuple Analytics ID, one or more tuples of unique ML Model identifier and ML Model Information. The ML model information may contain the ML model file address (e.g. URL or FQDN) or ADRF (Set) ID. The ADRF(Set) ID is included only when the NWDAF containing MTLF authorizes the NF consumer to retrieve the ML model(s) stored in the ADRF in step 2. When ADRF (Set) ID is provided and the NWDAF containing MTLF authorizes the NF Service Consumer to retrieve all ML models corresponding to a Storage Transaction ID, the Storage Transaction ID may be provided. In other cases, the NWDAF containing MTLF only provides ML Model identifier(s).

6.	If in step 5, the NWDAF service consumer (NWDAF containing AnLF or NWDAF containing MTLF) received ADRF (Set) ID (where the ML model(s) requested in step 1 is stored) and/or the ML model provide indicator, then the NWDAF service consumer may invoke the Nadrf_MLModelManagement_RetrievalRequest  (Storage Transaction Identifier or one or more unique ML Model identifier(s)) service operation to get the ML model stored in ADRF.

7.	The ADRF verifies the service consumer as described in Annex X.10 of TS 33.501 [xx].  If verification is successful, the ADRF sends Nadrf_MLModelManagement_RetrievalRequest Response (ML Model identifier(s) and address(es) of Model file(s) stored in ADRF to the NWDAF service consumer.

# * * * * Next change * * * *

10.3.2	Nadrf_MLModelManagement_StorageRequest service operation

Service operation name: Nadrf_MLModelManagement_StorageRequest

Description: The consumer NF uses this service operation to request the ADRF to store or update ML Model(s). ML Model(s) or ML Model address(es) stored in NWDAF containing MTLF are provided to the ADRF in the request message.

Inputs, Required: NF instance ID of the NWDAF containing MTLF and set of:

-	ML model(s); or

-	the tuple (unique ML Model identifier and address (e.g. URL or FQDN) of Model file and Storage size required for each of the ML Model).

NOTE:	The ADRF can download ML Model files based on the ML Model addresses provided by the NWDAF containing MTLF. How the ADRF downloads the ML Model and locally stores the ML Model is left for implementation.

Inputs, Optional:  Allowed NF instance list for the ML Model identifier.

Outputs Required: Result Indication, one of the following:

-	[Conditional] one or more tuples of unique ML Model identifier and address (e.g. URL or FQDN) of Model file stored in ADRF.

Outputs, Optional: Storage Transaction Identifier.

# * * * * End change * * * *