SA WG2 Meeting #160-AH-e	S2-2400289r01

January 22 - 29, 2024, Electronic Meeting

Source:	Wiliot

Title:	FS_AmbientIoT New Terms and Architectural Assumptions

Document for:	Approval

Agenda Item: 	19.14

Work Item / Release:	FS_AmbientIoT/Rel-19

Abstract of the contribution: This paper proposes architectural assumptions and requirements for FS_ AmbientIoT TR 23.700-13.

# 1	Discussion

This paper proposes FS_AmbientIoT architectural assumptions and requirements based on TR 22.840 and TR 38.848 content.

# 2	Proposal

It is proposed to include the following changes in TR 23.700-13 v0.0.0.

* * * * Start of Changes * * * *

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

- Ambient IoT device - Ambient IoT device is ultra-low complexity as well as ultra-low power, batteryless with harvesting energy capabilities (e.g. RF, Solar, etc.)

- Energized Ambient IoT device – Ambient IoT device with harvested energy level enabling Rx or Tx operation.

* * * * Next Changes * * * *

## 4.1	Architectural Assumptions

The architecture in this study will be based on the following assumptions:

- An ambient IoT device’s memory can retain its state for a limited time, e.g. holding 1Kbyte for 1-minute takes 10nJ

Note: This is due to memory retention energy cost

- Ambient IoT Device communication availability will depend on whether the device’s is Energized or not.

- Ambient IoT Devices only support data transfer over control plane

- In this release of the specification only trusted 3rd party AFs are considered

* * * * End of Changes * * * *