3GPP TSG-WG SA2 Meeting #160-Ad Hoc-e	S2-2400302r01

# Online, Jan 22 – 29, 2024		(revision of S2-2310927)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0339 | rev | 2 | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on IPv6 only for policy control and session binding for L3 N3IWF Relay |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph2 |  |  |  |  |  | Date: |  |  | 2024-01-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | It was identified that policy control and session binding for a L3 U2N Relay can only be supported if a /64 IPv6 Prefix from a shorter IPv6 prefix is assigned. There was some discussion about whether it was mandatory to assign the /64 IPv6 Prefix from a shorter IPv6 prefix. It is not mandary to assign the /64 IPv6 Prefix from a shorter IPv6 prefix, however in order specifically to support policy control and session binding it is required, i.e. it is mandary if you want to support it, otherwise it should remain optional. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that if the /64 IPv6 Prefix from a shorter IPv6 prefix is not assigned then policy control and session binding is not available, similar to as if IPv4 was used. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Clarification on the requirements for assignment of the /64 IPv6 prefix and policy control and session binding to avoid misunderstandings. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

#### 5.4.1.3	Policy control and session binding to support 5G ProSe Layer-3 UE-to-Network Relay without N3IWF

To enable support for policy control for 5G ProSe Layer-3 Remote UEs accessing 5GC via a 5G ProSe Layer-3 UE-to-Network Relay without involving N3IWF, the policy control functionality specified in TS 23.503 [9] is applied with the following functionalities:

-	For non-emergency service over the 5G ProSe Layer-3 UE-to-Network Relay, the SMF and PCF shall be configured with DNN(s) dedicated for UE-to-Network Relay connectivity, as specified in clause 5.1.4.1.

NOTE 1:	If Local Breakout configuration is supported for relay connectivity, the DNN dedicated for UE-to-Network Relay connectivity needs to be well-known DNN to allow seamless operation across various operators' networks.

NOTE 2:	For emergency service over the 5G ProSe Layer-3 UE-to-Network Relay (as specified in clause 5.4.4.3), emergency DNN configured in the Relay's AMF (as specified in clause 5.16.4 of TS 23.501 [4]) is used.

-	The AF discovers the PCF serving the 5G ProSe Layer-3 UE-to-Network Relay PDU Session as specified in clause 6.1.1.2 of TS 23.503 [9].

-	The PCF may validate any 5G ProSe Layer-3 Remote UE related service information from the AF based on roaming agreement and the DNN dedicated for UE-to-Network Relay connectivity for non-emergency service from the 5G ProSe Layer-3 Remote UE or emergency DNN for emergency service from the 5G ProSe Layer-3 Remote UE.

NOTE 3:	For 5G ProSe Layer-3 UE-to-Network Relay connectivity, the UE identity that the SMF has provided (i.e. 5G ProSe Layer-3 UE-to-Network Relay Identity) and a UE identity provided by the AF (i.e. 5G ProSe Layer-3 Remote UE Identity) can be different, while the Session binding with the PDU Session is valid.

-	The 5G ProSe Layer-3 Remote UEs may be assigned a /64 IPv6 Prefix from a shorter IPv6 prefix by the 5G ProSe Layer-3 UE-to-Network Relay. A /64 IPv6 Prefix from a shorter IPv6 prefix needs to be assigned to support policy control and session binding of a 5G ProSe Layer-3 Remote UE behind a 5G ProSe Layer-3 UE-Network Relay.

-	For a PDU Session to the DNN dedicated for 5G ProSe Layer-3 UE-to-Network Relay connectivity and using IPv6 prefix delegation (i.e. the assigned IPv6 network prefix is shorter than 64), the PCF shall perform session binding based on the IPv6 network prefix only. A successful session binding occurs whenever a longer prefix received from an AF matches the prefix value of the PDU Session. PCF shall not use the UE Identity for session binding for this PDU Session.

NOTE 4:	Support for policy control of Remote UEs behind a 5G ProSe Layer-3 UE-Network Relay using IPv4 is not available.

NOTE x:	If a /64 IPv6 Prefix from a shorter IPv6 prefix is not assigned to the 5G ProSe Layer-3 Remote UE by the 5G ProSe Layer-3 UE-Network Relay then support for policy control and session binding of 5G ProSe Layer-3 Remote UE behind a 5G ProSe Layer-3 UE-Network Relay not available.

# * * * * End of changes * * * *