3GPP TSG-WG SA2 Meeting #160-Ad Hoc-e	S2-2400305r04

# Online, Jan 22 – 29, 2024		(revision of S2-240xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0409 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update on UE-to-UE Relay Discovery and Link Establishment/Modification for alignment with SA3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph2 |  |  |  |  |  | Date: |  |  | 2024-01-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In current TS 33.503, the direct discovery set includes User Info IDs of the discoverer and discoveree 5G ProSe End UE. So, the UE-to-UE Relay cannot see the User Info ID of the discoveree 5G ProSe End UE during UE-to-UE Relay Discovery procedure. Some updates are needed to align with SA2 for the identifiers for 5G ProSe UE-to-UE Relay Discovery, the U2U Relay Discovery procedure, the Layer-2 Link Establishment/Modification and the Candidate UE-to-UE Relay Discovery, and to ensure consistent terminology. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Clarify that when the U2U Relay sends Solicitation message, it uses unique Layer-2 IDs that can be associated to per RSC and the Layer-2 ID of the discoverer UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment with SA3 on the design of multiple procedures. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.8.4.2, 6.3.2.4.3, 6.4.3.7.1, 6.7.1.1, 6.7.1.4, 6.7.2. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

5.8.4.2	Common identifiers for 5G ProSe UE-to-UE Relay Discovery

The following parameters are used for the 5G ProSe UE-to-UE Relay Discovery Announcement message (Model A), where Source Layer-2 ID and Destination Layer-2 ID are used for sending and receiving the message and User Info ID and Relay Service Code are contained in the message:

-	Source Layer-2 ID: the 5G ProSe UE-to-UE Relay self-selects a Source Layer-2 ID for 5G ProSe UE-to-UE Relay Discovery Announcement message.

-	Destination Layer-2 ID: the Destination Layer-2 ID for 5G ProSe UE-to-UE Relay Discovery Announcement message is selected based on the configuration as described in clause 5.1.5.1.

-	User Info ID of 5G ProSe UE-to-UE Relay: provides information about the 5G ProSe UE-to-UE Relay.

-	list of User Info ID of 5G ProSe End UE: provides information about the 5G ProSe End UE.

-	Relay Service Code: information to indicate the connectivity service the 5G ProSe UE-to-UE Relay provides to 5G ProSe End UEs.

The following parameters are used  for the 5G ProSe UE-to-UE Relay Discovery Solicitation message (Model B) between discoverer 5G ProSe End UE and 5G ProSe UE-to-UE Relay, where Source Layer-2 ID and Destination Layer-2 ID are used for sending and receiving the message and User Info ID and Relay Service Code are contained in the message:

-	Source Layer-2 ID: the discoverer 5G ProSe End UE self-selects a Source Layer-2 ID for 5G ProSe UE-to-UE Relay Discovery Solicitation message.

-	Destination Layer-2 ID: the Destination Layer-2 ID for 5G ProSe UE-to-UE Relay Discovery Solicitation message is selected based on the configuration as described in clause 5.1.5.1.

-	User Info ID of discoverer 5G ProSe End UE: provides information about the discoverer 5G ProSe End UE.

-	User Info ID of discoveree 5G ProSe End UE: provides information about the discoveree 5G ProSe End UE.

-	Relay Service Code: information about connectivity service that the discoverer 5G ProSe End UE is interested in.

The following parameters are used in the 5G ProSe UE-to-UE Relay Discovery Response message (Model B) between discoverer 5G ProSe End UE and 5G ProSe UE-to-UE Relay, where Source Layer-2 ID and Destination Layer-2 ID are used for sending and receiving the message and User Info ID and Relay Service Code are contained in the message:

-	Source Layer-2 ID: the 5G ProSe UE-to-UE Relay self-selects a Source Layer-2 ID for 5G ProSe UE-to-UE Relay Discovery Response message.

-	Destination Layer-2 ID: set to the Source Layer-2 ID of the received 5G ProSe UE-to-UE Relay Discovery Solicitation message.

-	User Info ID of discoveree 5G ProSe End UE: provides information about the discoveree 5G ProSe End UE.

-	User Info ID of 5G ProSe UE-to-UE Relay: provides information about the 5G ProSe UE-to-UE Relay.

-	Relay Service Code: identifies the connectivity service the 5G ProSe UE-to-UE Relay provides to 5G ProSe End UEs that matches the Relay Service Code from the corresponding Discovery Solicitation message.

The following parameters are used for the 5G ProSe UE-to-UE Relay Discovery Solicitation message (Model B) between 5G ProSe UE-to-UE Relay and discoveree 5G ProSe End UE, where Source Layer-2 ID and Destination Layer-2 ID are used for sending and receiving the message and User Info ID and Relay Service Code are contained in the message:

-	Source Layer-2 ID: the 5G ProSe UE-to-UE Relay self-selects a Source Layer-2 ID for 5G ProSe UE-to-UE Relay Discovery Solicitation message.

-	Destination Layer-2 ID: the Destination Layer-2 ID for 5G ProSe UE-to-UE Relay Discovery Solicitation message is selected based on the configuration as described in clause 5.1.5.1.

-	User Info ID of discoverer 5G ProSe End UE: provides information about the discoverer 5G ProSe End UE.

-	User Info ID of discoveree 5G ProSe End UE: provides information about the discoveree 5G ProSe End UE.

-	User Info ID of 5G ProSe UE-to-UE Relay: provides information about the 5G ProSe UE-to-UE Relay.

-	Relay Service Code: identifies the connectivity service the 5G ProSe UE-to-UE Relay provides to 5G ProSe End UEs.

The following parameters are used in the 5G ProSe UE-to-UE Relay Discovery Response message (Model B) between 5G ProSe UE-to-UE Relay and discoveree 5G ProSe End UE, where Source Layer-2 ID and Destination Layer-2 ID are used for sending and receiving the message and User Info ID and Relay Service Code are contained in the message:

-	Source Layer-2 ID: the discoveree 5G ProSe End UE self-selects a Source Layer-2 ID for 5G ProSe UE-to-UE Relay Discovery Response message.

-	Destination Layer-2 ID: set to the Source Layer-2 ID of the received 5G ProSe UE-to-UE Relay Discovery Solicitation message.

-	User Info ID of discoveree 5G ProSe End UE: provides information about the discoveree 5G ProSe End UE.

-	User Info ID of discoverer 5G ProSe End UE: provides information about the discoverer 5G ProSe End UE.

-	Relay Service Code: identifies the connectivity service the 5G ProSe UE-to-UE Relay provides to 5G ProSe End UEs that matches the Relay Service Code from the corresponding Discovery Solicitation message.

NOTE:	The UE implementation needs to ensure that when the UE self-selects Source Layer-2 IDs, the self-selected Source Layer-2 IDs are different between 5G ProSe Direct Discovery (including 5G ProSe UE-to-Network Relay Discovery and 5G ProSe UE-to-UE Relay Discovery) in clause 6.3.2 and 5G ProSe Direct Communication (including 5G ProSe UE-to-Network Relay Communication and 5G ProSe UE-to-UE Relay Communication) in clause 6.4, 6.5 and 6.7 and are different from any other provisioned Destination Layer-2 IDs as described in clause 5.1 and any other self-selected Source Layer-2 IDs used in a simultaneous 5G ProSe Direct Discovery (including 5G ProSe UE-to-Network Relay Discovery and 5G ProSe UE-to-UE Relay Discovery) with a different discovery model.

# * * * * Second change * * * *

6.3.2.4.3	Procedure for 5G ProSe UE-to-UE Relay Discovery with Model B

Depicted in Figure 6.3.2.4.3-1 is the procedure for 5G ProSe UE-to-UE Relay Discovery with Model B.

Figure 6.3.2.4.3-1: 5G ProSe UE-to-UE Relay Discovery with Model B

1.	The discoverer 5G ProSe End UE (UE-1) sends a 5G ProSe UE-to-UE Relay Discovery Solicitation message. The 5G ProSe UE-to-UE Relay Discovery Solicitation message contains the Type of Discovery Message, RSC and the Direct Discovery set which includes the User Info IDs of the discoverer 5G ProSe End UE (UE-1) and the discoveree 5G ProSe End UE (UE-2). The 5G ProSe UE-to-UE Relay Discovery Solicitation message is sent using the Source Layer-2 ID and Destination Layer-2 ID as described in clause 5.8.4.

A 5G ProSe UE-to-UE Relay determines the Destination Layer-2 ID for signalling reception as specified in clause 5.1.

The discoverer 5G ProSe End UE may include an Announce Prohibited Indication in the UE-to-UE Relay Discovery Solicitation message. If a 5G ProSe UE-to-UE Relay receives a Relay Discovery Solicitation message with an Announce Prohibited Indication it does not consider the 5G ProSe End UE as discovered during this procedure for inclusion in 5G ProSe UE-to-UE Relay Discovery with Model A, see clause 6.3.2.4.2, step 1.

2.	If the RSC contained in the solicitation message matches any of the (pre)configured RSC(s), as specified in clause 5.1.5.1, of a 5G ProSe UE-to-UE Relay, the 5G ProSe UE-to-UE Relay sends a 5G ProSe UE-to-UE Relay Discovery Solicitation message. The 5G ProSe UE-to-UE Relay Discovery Solicitation message contains the Type of Discovery Message, the Direct Discovery set which includes the User Info IDs of the discoverer 5G ProSe End UE (UE-1) and the discoveree 5G ProSe End UE (UE-2), User Info ID of UE-to-UE Relay, RSC, . 5G ProSe UE-to-UE Relay Discovery Solicitation message is sent using the Source Layer-2 ID and Destination Layer-2 ID as described in clause 5.8.4.

A 5G ProSe End UE determines the Destination Layer-2 ID for signalling reception as specified in clause 5.1.

3.	If the RSC contained in the solicitation message matches any of the (pre)configured RSC(s), as specified in clause 5.1.5.1, of the discoveree 5G ProSe End UE (UE-2), and the discoveree 5G ProSe End UE (UE-2) matches the User Info ID of the discoveree 5G ProSe End UE (UE-2) contained in the solicitation message, then the discoveree 5G ProSe End UE (UE-2) responds to the 5G ProSe UE-to-UE Relay with a 5G ProSe UE-to-UE Relay Discovery Response message. The 5G ProSe UE-to-UE Relay Discovery Response message contains the Type of Discovery Message, RSC, the Direct Discovery set which includes the User Info ID of the discoverer 5G ProSe End UE (UE-1) and User Info ID of discoveree 5G ProSe End UE (UE-2). The 5G ProSe UE-to-UE Relay Discovery Response message is sent using the Source Layer-2 ID and Destination Layer-2 ID as described in clause 5.8.4. If the discoveree 5G ProSe End UE (UE-2) receives multiple UE-to-UE Relay Discovery Solicitation messages from different 5G ProSe UE-to-UE Relays with the same RSC and the User Info ID of the discoveree 5G ProSe End UE (UE-2), it may choose to respond or not to a 5G ProSe UE-to-UE Relay (e.g. based on the PC5 signal strength of each message received).

4.	The 5G ProSe UE-to-UE Relay sends a 5G ProSe UE-to-UE Relay Discovery Response message. The 5G ProSe UE-to-UE Relay Discovery Response message contains the Type of Discovery Message, User Info ID of UE-to-UE Relay, RSC, User Info ID of the discoverer 5G ProSe End UE (UE-1) and User Info ID of the discoveree 5G ProSe End UE (UE-2). The 5G ProSe UE-to-UE Relay Discovery Response message is sent using the Source Layer-2 ID and Destination Layer-2 ID as described in clause 5.8.4.

# * * * * XXX change * * * *

6.4.3.7.1	Common part for Layer-2 link management over PC5 reference point for 5G ProSe UE-to-UE Relay

For the 5G ProSe Communication via 5G ProSe UE-to-UE Relay as described in clause 6.7.1 and clause 6.7.2:

-	The Direct Communication Request message over the first hop PC5 reference point includes:

-	User Info ID of source 5G ProSe End UE: the identity of the source 5G ProSe End UE requesting relay operation.

-	User Info ID of 5G ProSe UE-to-UE Relay: the identity of the UE-to-UE Relay provided to the source 5G ProSe End UE during 5G ProSe UE-to-UE Relay Discovery procedure.

-	User Info ID of target 5G ProSe End UE: the identity of the target 5G ProSe End UE provided to the source 5G ProSe End UE during UE-to-UE Relay Discovery procedure.

-	(optional) Destination Layer-2 ID of target 5G ProSe End UE: the unicast destination Layer-2 ID of the target 5G ProSe End UE determined by the source 5G ProSe End UE as specified in clause 5.8.2.4.

-	ProSe Service Info: the information about the ProSe identifier(s) requesting Layer-2 link establishment.

-	RSC: the connectivity service provided by the 5G ProSe UE-to-UE Relay as requested by the source 5G ProSe End UE.

-	Security Information: the information for the establishment of security for the first hop PC5 link establishment.

NOTE 1:	The Security Information is defined by SA WG3.

The Direct Communication Request message over the second hop PC5 reference point includes:

-	User Info ID of source 5G ProSe End UE.

-	User Info ID of target 5G ProSe End UE.

-	User Info ID of 5G ProSe UE-to-UE Relay.

-	ProSe Service Info: the information about the ProSe identifier(s).

-	RSC: the connectivity service provided by the 5G ProSe UE-to-UE Relay as requested by the source 5G ProSe End UE.

-	Security Information: the information for the establishment of security for the second hop PC5 link establishment.

NOTE 2:	The Security Information is defined by SA WG3.

The Direct Communication Accept message over the second hop PC5 reference point includes:

-	User Info ID of target 5G ProSe End UE.

The Direct Communication Accept message over the first hop PC5 reference point includes:

-	User Info ID of target 5G ProSe End UE.

-	User Info ID of 5G ProSe UE-to-UE Relay.

The Link Modification Request message over the first hop PC5 reference point includes:

-	User Info ID of target 5G ProSe End UE: the identity of the target 5G ProSe End UE provided to the source 5G ProSe End UE during UE-to-UE Relay Discovery procedure.

-	(optional) Destination Layer-2 ID of target 5G ProSe End UE: the unicast destination Layer-2 ID of the target 5G ProSe End UE determined by the source 5G ProSe End UE as specified in clause 5.8.2.4.

The Link Modification Request message over the second hop PC5 reference point includes:

-	User Info ID of source 5G ProSe End UE.

-	User Info ID of target 5G ProSe End UE.

The Link Modification Accept message over the second hop PC5 reference point includes:

-	User Info ID of target 5G ProSe End UE.

The Link Modification Accept message over the first hop PC5 reference point includes:

-	User Info ID of target 5G ProSe End UE.

# * * * * Forth change * * * *

6.7.1.1	Layer-2 link establishment for PC5 communication via 5G ProSe Layer-3 UE-to-UE Relay

Figure 6.7.1.1-1 shows the procedure for Layer-2 link establishment via 5G ProSe Layer-3 UE-to-UE Relay.

Figure 6.7.1.1-1: Layer-2 link establishment via 5G ProSe Layer-3 UE-to-UE Relay

1.	Service authorization and provisioning are performed for source 5G ProSe Layer-3 End UE, target 5G ProSe Layer-3 End UE and 5G ProSe Layer-3 UE-to-UE Relay as described in clause 6.2.

2.	The source 5G ProSe Layer-3 End UE performs discovery of a 5G ProSe Layer-3 UE-to-UE Relay as described in clause 6.3.2.4.

3.	The source 5G ProSe Layer-3 End UE sends a Direct Communication Request message to initiate the unicast Layer-2 link establishment procedure with the 5G ProSe Layer-3 UE-to-UE Relay. The parameters included in the Direct Communication Request message are described in clause 6.4.3.7.

The Source Layer-2 ID of the Direct Communication Request message is self-assigned by the source 5G ProSe Layer-3 End UE and the Destination Layer-2 ID is set to the Source Layer-2 ID of the discovery message of the 5G ProSe Layer-3 UE-to-UE Relay.

The source 5G ProSe Layer-3 End UE gets application information and optional ProSe Application Requirements from ProSe application layer, and determines the end-to-end QoS parameters as described in clause 5.6.3.1.

4.	If the User Info ID of 5G ProSe Layer-3 UE-to-UE Relay in the Direct Communication Request message matches the 5G ProSe UE-to-UE Relay's User Info ID and the RSC in the Direct Communication Request matches one RSC that the relay is (pre)configured with, as specified in clause 5.1.5.1, the 5G ProSe Layer-3 UE-to-UE Relay responds by establishing the security with the source 5G ProSe Layer-3 End UE. When the security protection is enabled, the source 5G ProSe Layer-3 End UE sends the parameters as described in clause 6.4.3.7 to the 5G ProSe Layer-3 UE-to-UE Relay.

If the Ethernet MAC address of source 5G ProSe Layer-3 End UE is already used by another 5G ProSe Layer-3 End UE, then the 5G ProSe Layer-3 UE-to-UE Relay rejects the direct link establishment indicating that the MAC address is not unique.

The Source Layer-2 ID used for the security establishment procedure is self-assigned by the 5G ProSe Layer-3 UE-to-UE Relay and the Destination Layer-2 ID is set to the Source Layer-2 ID of the received Direct Communication Request message.

The 5G ProSe Layer-3 UE-to-UE Relay shall choose different Source Layer-2 IDs for PC5 links of different types of traffic, i.e., IP traffic, Ethernet traffic and Unstructured traffic.

If the PC5 link is used for transferring Unstructured traffic, the 5G ProSe Layer-3 UE-to-UE Relay shall choose different Source Layer-2 IDs for different pair of source and target 5G ProSe Layer-3 End UEs.

Upon receiving the security establishment procedure messages, the source 5G ProSe Layer-3 End UE obtains the 5G ProSe Layer-3 UE-to-UE Relay's Layer-2 ID for future communication, for signalling and data traffic for this unicast link.

5.	After the Security Establishment procedure in step 4 is completed, the 5G ProSe Layer-3 UE-to-UE Relay sends a Direct Communication Request message to initiate the unicast Layer-2 link establishment procedure with the target 5G ProSe Layer-3 End UE. The parameters included in the Direct Communication Request message are described in clause 6.4.3.7.

The Source Layer-2 ID of the Direct Communication Request message is self-assigned by the 5G ProSe Layer-3 UE-to-UE Relay and the Destination Layer-2 ID is the unicast Layer-2 ID of target 5G ProSe Layer-3 End UE associated with the User Info ID of target 5G ProSe Layer-3 End UE.

The 5G ProSe Layer-3 UE-to-UE Relay shall choose different Source Layer-2 IDs for PC5 links of different types of traffic, i.e., IP traffic, Ethernet traffic and Unstructured traffic.

If the PC5 link is used for transferring Unstructured traffic, the 5G ProSe Layer-3 UE-to-UE Relay shall choose different Source Layer-2 IDs for different pair of source and target 5G ProSe Layer-3 End UEs.

6.	If the User Info ID of target 5G ProSe Layer-3 End UE and RSC included in the Direct Communication Request match the target UE's User Info ID and the RSC that the target UE is (pre)configured with as specified in clause 5.1.5.1, the target 5G ProSe Layer-3 End UE responds by establishing the security with the 5G ProSe Layer-3 UE-to-UE Relay. When the security protection is enabled, the 5G ProSe Layer-3 UE-to-UE Relay sends the parameters as described in clause 6.4.3.7 to the target 5G ProSe Layer-3 End UE.

The Source Layer-2 ID used for the security establishment procedure is self-assigned by the target 5G ProSe Layer-3 End UE and the Destination Layer-2 ID is set to the Source Layer-2 ID of the received Direct Communication Request message.

Upon receiving the security establishment procedure messages, the 5G ProSe Layer-3 UE-to-UE Relay obtains the target 5G ProSe Layer-3 End UE's Layer-2 ID for future communication, for signalling and data traffic for this unicast link.

7.	The target 5G ProSe Layer-3 End UE sends a Direct Communication Accept message to the 5G ProSe Layer-3 UE-to-UE Relay that has successfully established security with. The parameters included in the Direct Communication Accept message are described in clause 6.4.3.7.

NOTE:	The 5G ProSe Layer-3 UE-to-UE Relay can detect that the Ethernet MAC address of target 5G ProSe Layer-3 End UE is already used by another 5G ProSe Layer-3 End UE when it receives the Direct Communication Accept message.

8.	For IP traffic, IPv6 prefix or IPv4 address is allocated for the target 5G ProSe Layer-3 End UE as defined in clause 5.5.1.4.

9.	After receiving the Direct Communication Accept message from the target 5G ProSe Layer-3 End UE, the 5G ProSe Layer-3 UE-to-UE Relay sends a Direct Communication Accept message to the source 5G ProSe Layer-3 End UE that has successfully established security with. The parameters included in the Direct Communication Accept message are described in clause 6.4.3.7.

10.	For IP traffic, IPv6 prefix or IPv4 address is allocated for the source 5G ProSe Layer-3 End UE as defined in clause 5.5.1.4.

11.	For IP communication, the 5G ProSe Layer-3 UE-to-UE Relay may store an association of User Info ID and the IP address of target 5G ProSe Layer-3 End UE into its DNS entries and the 5G ProSe Layer-3 UE-to-UE Relay may act as a DNS server to other UEs. The source 5G ProSe Layer-3 End UE may send a DNS query to the 5G ProSe Layer-3 UE-to-UE Relay to request IP address of target 5G ProSe Layer-3 End UE after step 10 if the IP address of target 5G ProSe Layer-3 End UE is not received in step 9 and the 5G ProSe Layer-3 UE-to-UE Relay returns the IP address of the target 5G ProSe Layer-3 End UE to the source 5G ProSe Layer-3 End UE.

For Ethernet communication, the 5G ProSe Layer-3 UE-to-UE Relay maintains the association between PC5 links and Ethernet MAC addresses received from the 5G ProSe Layer-3 End UE.

For Unstructured traffic communication, for each pair of source and target 5G ProSe Layer-3 End UEs, the 5G ProSe Layer-3 UE-to-UE Relay maintains the 1:1 mapping between the PC5 link with source 5G ProSe Layer-3 End UE and the PC5 link with target 5G ProSe Layer-3 End UE.

12.	The source 5G ProSe Layer-3 End UE communicates with the target 5G ProSe Layer-3 End UE via the 5G ProSe Layer-3 UE-to-UE Relay.

In the case of one source 5G ProSe Layer-3 End UE communicates with multiple target 5G ProSe Layer-3 End UEs, the PC5 link between the source 5G ProSe Layer-3 End UE and the 5G ProSe Layer-3 UE-to-UE Relay can be shared for multiple target 5G ProSe Layer-3 End UEs per RSC while the PC5 links may be established individually between the 5G ProSe Layer-3 UE-to-UE Relay and target 5G ProSe Layer-3 End UEs per RSC. For the shared PC5 link, the Layer-2 link modification procedure shall be used. The parameters used in the Layer-2 link modification procedure are described in clause 6.4.3.7.

In the case of multiple source 5G ProSe Layer-3 End UEs communicate with one target 5G ProSe Layer-3 End UE, the PC5 link between the 5G ProSe Layer-3 UE-to-UE Relay and the target 5G ProSe Layer-3 End UE can be shared per RSC while the PC5 links may be established individually between the source 5G ProSe Layer-3 End UEs and the 5G ProSe Layer-3 UE-to-UE Relay per RSC. For the shared PC5 link, the Layer-2 link modification procedure shall be used. The parameters used in the Layer-2 link modification procedure are described in clause 6.4.3.7.

# * * * * Fifth change * * * *

6.7.1.4	Layer-2 link modification for PC5 communication via 5G ProSe Layer-3 UE-to-UE Relay

Figure 6.7.1.4-1 shows the Layer-2 link modification procedure via Layer-3 UE-to-UE Relay. This procedure is used to add/modify/remove PC5 QoS Flow(s) in the existing PC5 unicast link as described in clause 6.4.3.7.3.

Figure 6.7.1.4-1: Layer-2 link modification procedure via Layer-3 UE-to-UE Relay

0.	UE-1 and UE-to-UE Relay and UE-to-UE Relay and UE-2 have a unicast link established as described in clause 6.7.1.1.

1.	UE-1 sends a Link Modification Request to UE-to-UE Relay as described in clause 6.4.3.7.3.

2.	Upon reception of the Link Modification Request message from UE-1, the UE-to-UE Relay sends a Link Modification Request to UE-2 as described in clause 6.4.3.7.3.

3.	UE-2 responds with a Link Modification Accept message to the UE-to-UE Relay as described in clause 6.4.3.7.3.

4.	Upon reception of the Link Modification Accept message from UE-2, the UE-to-UE Relay responds with a Link Modification Accept message to the UE-1 as described in clause 6.4.3.7.3.

# * * * * Sixth change * * * *

6.7.2	5G ProSe Communication via 5G ProSe Layer-2 UE-to-UE Relay

This procedure applies to 5G ProSe Layer-2 UE-to-UE Relay.

Figure 6.7.2-1: 5G ProSe Communication via 5G ProSe Layer-2 UE-to-UE Relay

Service authorization and provisioning has been performed for the 5G ProSe Layer-2 UE-to-UE Relay and the 5G ProSe End UEs as described in clause 6.2 before this procedure.

1.	Model A or Model B 5G ProSe UE-to-UE Relay Discovery as described in clause 6.3.2.4 is performed and a source 5G ProSe End UE selects a suitable 5G ProSe Layer-2 UE-to-UE Relay for the communication with a target 5G ProSe End UE.

2.	The source 5G ProSe End UE decides whether to use an existing PC5 link with the 5G ProSe UE-to-UE Relay for the required service. If an existing PC5 link is used then the Layer-2 link modification procedure as specified in clause 6.4.3.7 is used towards a 5G ProSe UE-to-UE Relay, otherwise a Layer-2 link establishment procedure is used towards a 5G ProSe UE-to-UE Relay.

This procedure is towards the selected 5G ProSe UE-to-UE Relay and for Layer-2 link establishment, the security establishment is performed before step 3 is initiated.

3.	The 5G ProSe Layer-2 UE-to-UE Relay decides whether to use an existing PC5 link between the 5G ProSe UE-to-UE Relay and the target 5G ProSe End UE for the required service and initiates Layer-2 link establishment procedure or Layer-2 link modification procedure as specified in clause 6.4.3.7 with the target 5G ProSe End UE.

This procedure is performed towards the target 5G ProSe End UE using the unicast Layer-2 ID.

The 5G ProSe Layer-2 UE-to-UE Relay sends a Direct Communication Accept message or Link Modification Accept message to the source 5G ProSe End UE after step 3 is completed.

4.	The source 5G ProSe End UE establishes an end-to-end connection for unicast mode communication with the target 5G ProSe End UE as described in clause 6.4.3.7.

The data is transferred between the source 5G ProSe End UE and the target 5G ProSe End UE via the 5G ProSe Layer-2 UE-to-UE Relay. The 5G ProSe Layer-2 UE-to-UE Relay forwards all the data messages between the source 5G ProSe End UE and the target 5G ProSe End UE, as specified in TS 38.300 [12].

# * * * * End of changes * * * *