3GPP TSG-WG SA2 Meeting #160-AH	S2-2400325r05

# Online, January 22 - 29, 2024

Source: 	China Mobile, Intel

Title: 	New key issue: Enhancements for EAS (re)discovery and UPF (re)selection with reducing impact on central 5GC NFs

Document for: 	Approval

Agenda Item: 	19.9

Work Item / Release:	FS_eEDGE_5GC_ph3 / Rel-19

Abstract of the contribution: Propose a key issue about WT#1.1: Reducing impact on central 5GC NFs to support EAS (re)discovery and UPF (re)selection.

# 1	Discussion

This paper is to propose a new key issue according to the WT#1.1 of FS_eEDGE_5GC_ph3.

The FS_eEDGE_5GC_ph3 objectives for WT#1.1:

- WT#1.1. Handle the edge network and EAS related information (e.g. UPF and EAS deployment information, DNAIs) locally with less impact to 5GC central NFs (e.g. central SMF) to address more flexible EAS (re)discovery/(re)selection, local UPF (re)selection.

The content from S2-2400291,S2-2400718,S2-2400801,S2-2401226 are merged into this paper.

# 2 Proposal

It is proposed to include the below changes into TR 23.700-49 v0.0.0.

FIRST CHANGE (all the text is new)

## 5.X	Key Issue #X: Enhancements for EAS (re)discovery and UPF (re)selection with reducing impact on central 5GC NFs

### 5.X.1	Description

This key issue is to investigate whether and how to reduce impact on central NFs with support EAS (re)discovery and UPF (re)selection.

The Edge Computing related specifications in Rel-17/18 have impact on central 5GC Control Plane NFs. For example, the EHE-related information (FQDN, IP address or EAS Deployment Information) , local UPF information (deployment information, or load) may be changed dynamically or frequently, while need to be updated at the central SMF. And this central SMF also gives a bottleneck point to the DNS message handling as all decision is made by the SMF.This may also cause signalling overheads across operator’s data centers for a deployment scenario where Control Plane NF(s) such as SMF is deployed in the central data center while other involved NFs such L-NEF, AF or EASDF are located in the local data center.

To reduce these impact on central SMF, the following aspects should be studied:

-	Whether and which edge computing related information (e.g. UPF and EAS deployment information, DNAIs) can be locally managed to support EAS (re)discovery and UPF (re)selection.

-	Which entity(ies) determines whether the edge computing related information is managed, locally, centrally or both for EAS (re)discovery and/or local UPF (re)selection;

-	Whether existing procedures (e.g. discovery, ULCL/BP, mobility and influence/exposure related procedures) suffice or whether there are gaps to be addressed when above information is handled locally or both locally and centrally.

End of CHANGE