SA WG2 Meeting #160AH-e	S2-2400329r01

22 - 29 January, 2024, Elbonia

Source:	China Mobile, Huawei, HiSilicon

Title:	KI#3, New sol: Solution of Data Channel interworking

Document for:	Approval

Agenda Item:	19.2

Work Item / Release:	FS_NG_RTC_Ph2 / Rel-19

Abstract: This paper proposes to provide a solution for KI#3 of FS_NG_RTC_Ph2.

1. Discussion

The Key Issuse#3 of SA2 R19 FS_NG_RTC_Ph2 addresses the following aspect:

- how to enhance IMS architecture and procedures to provide some form of IMS data channel applications service experience by data channel interworking with MTSI UE supporting only legacy media types where it is appropriate.

This document provides a solution to address above aspect of KI#3.

3. Proposal

It is proposed to capture the following changes into TR 23.700-77.

# * * * * First change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G system, Stage 2".

[4]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System".

[5]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS)".

[6]	3GPP TS 26.114: "Media handling and interaction".

[7]	3GPP TS 22.261: "Service requirements for the 5G system".

[8]	3GPP TR 26.813: "Study of Avatars in Real-Time Communication Services".

[9]	3GPP TS 22.156: "Mobile Metaverse Services".

[10]	3GPP TS 24.229: "IP Multimedia Call Control based on SIP and SDP; Stage 3".

[11]	3GPP TS 24.607: "Originating Identification Presentation (OIP) and Originating Identification Restriction (OIR) using IP Multimedia (IM) Core Network (CN) subsystem".

[x]	3GPP TS 22.873: "Study on evolution of the IP Multimedia Subsystem (IMS) multimedia telephony service".

# * * * * Second change * * * *

6	Solutions

6.0	Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| 1 | X |  |  |  |  |  |  |  |
| X |  |  | X |  |  |  |  |  |

Table6.0-1: Mapping of Solutions to Key Issues

# * * * * Third change * * * *

6.X Solution #X:Data Channel Interworking mechanism between DCMTSI UE and MTSI UE

6.X.1	Description

This solution addresses Key Issue#3 "Data channel interworking with MTSI UE" to support data channel interworking mechanism between DCMTSI UE and MTSI UE, which covers:

- how to enhance IMS architecture and procedures to provide some form of IMS data channel applications service experience by data channel interworking with MTSI UE supporting only legacy media types where it is appropriate.

This solution proposes a method to achieve the one-sided interactive communication between a DCMTSI client in terminal and an MTSI client in terminal.

6.X.2	Procedures

6.X.2.1	Interworking between a DCMTSI client and an MTSI client

6.X.2.1.1	General

The procedures described in this clause is applied to some data channel use cases in which it’s a audio session and only unidirectional interaction is needed between both UEs. Real-time Screen Sharing as specified in 3GPP TS 22.873 [x] is used as an example to describe the procedure. For example, in Real-time Sharing Screen use case, only UE A supports IMS data channel and it establishes application data channels with the MF in its local network and transfers its mobile screen data, including RTP stream and RTCP stream, over the application data channels. The MF decapsulates RTP data and RTCP data from the data channels, encapsulates and sends these data through video media streams to UE B. Figure 6.X.2.1-1 depicts the workflow below.

Figure 6.X.2.1-1: Interworking between a DCMTSI client in terminal and an MTSI client in terminal

6.X.2.1.2	Interworking between a DCMTSI client in terminal and an MTSI client in terminal during BootstrapData Channel establishment

Figure 6.X.2.1.2-1 demonstrates the procedure of UE-initiated bootstrap data channel establishment between network A and UE A (a DCMTSI client in terminal), while an IMS audio call is established between UE A and UE B (an MTSI client in terminal).

Figure 6.X.2.1.2-1: Bootstrap data channel establishment procedure for interworking between a DCMTSI client in terminal and an MTSI client in terminal

1-2. UE #1 initiates the bootstrap DC establishment procedure along with audio media components, originating DCSF receives the DC negotiation result from IMS AS, which indicates the bootstrap DC established successfully in local network.

3~6. The INVITE request is sent to remote network with SDP containing data channel media component for terminating UE #2. In the bootstrap DC negotiation result returned from remote network to originating DCSF, the port of bootstrap data channel between the originating DCSF and UE #2 is set to zero, which means that terminating network or UE #2 does not support or does not use IMS data channel in this session. The IMS AS sends 18X message to UE A to indicate that the data channel for local side has been established successfully in local network. The DCSF also determine that UE B or network B does not support data channel, at least UE B or network B does not use data channel in this session.

- The dedicated bearer/QCI Flow of data channel is created between UE A and its local network.

8-11. After the precondition procedure is completed, UE A initiates SCTP association and DTLS connection establishment procedures and establishes a bootstrap data channel with the MF A. Then UE A sends the data channel application list request to DCSF A. After DCSF A responses the request, UE A downloads the application (for instance, Real-time Screen Sharing application) accordingly.

6.X.2.1.3	Interworking between a DCMTSI client in terminal and an MTSI client in terminal during Application Data Channel establishment

The following procedure uses Real-time Screen Sharing application as the example. When UE A retrieves Real-time Screen Sharing application from the DCSF A and runs the application, the following procedure shown in Figure 6.X.2.1-3 is performed to transfer UE A’s mobile screen data to UE B.

Figure 6.X.2.1.3-3: Application channel establishment procedure for interworking between a DCMTSI client in terminal and an MTSI client in terminal

1~3. UE#1 initiates a re-INVITE request to establish an application data channel for Real-time Screen Sharing application , in which two application data channels (for instance,stream 1000 and 1001) and data channel application information (for instance, application ID) are contained. Stream 1000 is used to transfer video RTP stream of screen shots sequence and stream 1001 is used to transfer RTCP stream. The IMS AS notifies the DCSF as specified in Rel-18.

4~5. Since the DCSF has determined that remote network or UE#2 does not support or does not use IMS data channel, the DCSF decides to start one-side data channel procedure in this IMS session and to anchor the data channel media on the MF.

6~9. The DCSF invokes Nimsas_MediaControl_MediaInstruction to the IMS AS and instructs the IMS AS to create resources and association for two application data channels (stream 1000 and 1001) on originating side and video media on terminating side. The IMS AS invokdes Nmf_MediaResourceManagement to the MF to allocate resource accordingly. The DCSF also includes the instruction of how to transform data channel media to video media which is sent to the MF via IMS AS.

Editor’s note:	The details of instruction of how to transform data channel media to video media is FFS and needs coordination with SA4.

10-17. After the resource reservation, the IMS AS sends a re-INVITE request with the SDP offer for anchoring the video media of UE#2 to the MF. UE#2 returns 200 OKfor the re-NVITE. DCSF is notified about the terminating network media negotiation result.

18-21. The IMS AS sends 200 OK for re-INVITE initiated by UE#1 and UE#1 returns ACK.

22. UE#1 initiates SCTP association and DTLS connection establishment procedures and establishes two application data channels (stream 1000 and 1001) with the MF.

The application data channels (stream 1000 and 1001) between the MF and UE#1 are associated with the RTP and RTCP video streams between the MF and UE#2. The RTP and RTCP stream of UE#1’s screenshot sequence is received by UE#2 from video streams.

# * * * * End of changes * * * *