SA WG2 Meeting #S2-160AHE	S2-2400386r01

# 22 - 29 January, 2024, Electronic													revision of S2-24xx

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 4682 | rev |  | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on filtering criteria of member UE selection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIMLsys |  |  |  |  |  | Date: |  |  | 2024-01-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| a |  |  |  |  |  |  |  |  |  |  |
| Reason for change: |  | As it has been agreed in S2-2313630 in SA2 160 meeting, ‘data volume UL/DL’ and ‘A request for geographical distribution (i.e. the AoIs) of the UEs' were newly introduced as mandatory parameters into the request or subscription of NWDAF consumer for E2E data volume transfer time analytics in TS 23.288. However, in the current spec, the NEF is not aware of such parameters; and therefore, the NEF is not able indicate the mandatory parameters to NWDAF, as required. In order to support the mandatory parameters as it has been agreed, the AF should indicate the parameters to the AF as part of the Member UE filtering criteria information when the AF triggers the member UE selection functionality towards the NEF. The data Volume UL/DL indicated by the AF could be the expected data volume of the up-coming AIML service or the observed or measured data volume based AF knowledge, which will be used by the NWDAF to derive the data volume transfer time for the required UEs. Further alignments between TS 23.288 and TS 23.502 regarding the information in the consumer request are clarified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Introduce ‘data volume UL/DL’ and ‘request for geographical distribution (i.e. the AoIs) of the UEs' into End-to-end data volume transfer time Member UE filtering criteria. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - ‘Data Volume UL/DL’ and ‘A request for geographical distribution (i.e. the AoIs) of the UEs' cannot be supported as mandatory parameters indicated by the consumers to NWADF for E2E data volume transfer time analytics. - Unclear spec text |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.15.13.2, 4.15.13.6.1, 4.15.13.6.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * *Start of Changes * * *

#### 4.15.13.2	Member UE Filtering Criteria for 5GS assistance to Member UE selection

Table 4.15.13.2-1 provides a summary of the Member UE filtering criteria that the AF may request.

Table 4.15.13.2-1: Description of Member UE filtering criteria

| Member UE filtering criteria | Description of filtering criteria for member UEs selected by NEF | UE filtering information | Detailed description clause |
| --- | --- | --- | --- |
| QoS | The Quality of Service of the member UEs match or exceed the QoS of the filtering criteria | NF service: Nsmf_EventExposure or Nudm_EvenExposure, Filter: target=SUPI, traffic descriptor (e.g. Application ID), DNN/S-NSSAI Event ID: QoS Monitoring | 4.15.13.3 |
| Access Type and/or RAT Type of the PDU Session | Indicate the Access Type and/or RAT Type of the member UEs for the PDU Session used by the application (e.g. 3GPP/NR, Non-3GPP/WLAN, additional Access Type and RAT Type for MA PDU session) | NF service: Nsmf_EventExposure Filter: a list of GPSI(s) or SUPI(s), DNN/S-NSSAI, Event ID: Change of Access Type and/or Change of RAT Type | 5.2.8.3 |
| End-to-end data volume transfer time | Indicate the target end-to-end data volume transfer time that refers to a time for completing the transmission of a specific data volume between UE and AF, e.g. the average and variance of End-to-end data volume transfer time. | NF service: Nnwdaf_AnalyticsSubscription/ Nnwdaf_AnalyticsInfo Filter: Target = GPSI(s) or SUPI(s) Analytics ID: E2E data volume transfer time | 4.15.13.6 |
| UE current location | Indicate the certain area that the member UEs are currently located in. | NF service: Namf_EventExposure Filter: a list of GPSI(s) or SUPI(s) Event ID: Location Report | 4.15.13.4 |
| UE historical location | Indicate the certain area that the member UEs appeared in a historical period of time | NF service: Nnwdaf_AnalyticsSubscription / Nnwdaf_AnalyticsInfo Filter: Visited AoI = Target AOI, target period = historical nomadic period Analytics ID = UE mobility | 4.15.13.4 |
| UE direction | Indicate the member UEs should include different moving directions | NF service: Nnwdaf_AnalyticsSubscription / Nnwdaf_AnalyticsInfo Filter: UE Direction Analytics ID= UE Mobility | 4.15.13.4 |
| UE separation distance (NOTE 1) | Indicate the member UEs should comply with a minimum separation distance between each other | NF service: Nnwdaf_AnalyticsSubscription/ Nnwdaf_AnalyticsInfo Filter: Proximity Attributes Analytics ID: Relative Proximity | 4.15.13.4 |
| Service Experience | Indicates member UEs fulfilling certain Service Experience criteria e.g., MOS value | NF service: Nnwdaf_AnalyticsSubscription/ Nnwdaf_AnalyticsInfo Filter: S-NSSAI, DNN, Application ID, DNAI, AoI, Service Experience Contribution weight, reporting threshold (NOTE 2), Service Experience Type (NOTE 3) Analytics ID=Service Experience | 4.15.13.5 |
| DNN | Indicate the DNN of the member UEs for the PDU Session used by the application | NF service: Nsmf_EventExposure Filter: a list of GPSI(s) or SUPI(s), Event ID: QFI allocation | 5.2.8.3 |
| NOTE 1: This criterion should only be applied when the number of UEs is in the range of 10's or less. NOTE 2: The Service Experience Contribution Weights signal the relative importance of each UE's Service Experience value (i.e. MOS), as defined in TS 23.288 [50]. For example, it might be that the service experience of a UE in relation to other UEs may not be as important e.g. because the data provided by such UE is not as critical to the service. NOTE 3: Indicates the type of service experience analytics, e.g. AI/ML traffic where a customized MoS apply. |  |  |  |

* * *Next Change * * *

##### 4.15.13.6.1	General

An AF may invoke Nnef_MemberUESelectionAssistance_Subscribe service operation with end-to-end data volume transfer time related filtering criteria for receiving a list of UEs that fulfil the filtering criteria.

In addition to the mandatory parameters, the AF may also include in the request:

-	End-to-end data volume transfer time filtering criteria: this may include the average end-to-end data volume transfer time for a specific data volume between UE and AF and/or the variance of the end-to-end data volume transfer time.

-	An Area of Interest: location area of the candidate UEs.

-	Time windows for selecting the candidate UEs: start time and stop time.

-	Data Volume UL/DL: the expected, observed or measured data volume from UE to AF and/or from AF to UE of which the end-to-end data volume transfer time to be derived.

-	The target number of data transmission repetitions or target time interval between data transmissions.

-	A request for geographical distribution of UEs.

##### 4.15.13.6.2	Member UE Selection assistance with end-to-end data volume transfer time related filtering criteria

Figure 4.15.13.6.2-1: Assistance to member UE selection for end-to-end data volume transfer time related filtering criteria

1.	AF subscribes to the member UE selection assistance functionality by invoking Nnef_MemberUESelectionAssistance_subscribe request including the Application ID, DNN/S NSSAI, AoI, and the end-to-end data volume transfer time related filtering criteria including the average end-to-end data volume transfer time and/or the variance of the transfer time, the Data Volume UL/DL between AF and UE, the target number of data transmission repetitions or target time interval between data transmissions, request for geographical distribution (i.e. the AoIs) of the UEs.

2.	NEF verifies the authorization of the AF Request and identifies which information needs to be collected and executed based on the end-to-end data volume transfer time related filtering criteria provided by the AF.

3.	S-NSSAI/DNN are not included in the request from the AF, the NEF derives the S-NSSAI and DNN which this Application has access to. NEF discovers and selects the NWDAF(s) by invoking Nudm_UECM_Get or Nnrf_NFDiscovery_Request including Analytics ID = E2E data volume transfer time, AoI, S-NSSAI, etc.

4.	NEF sends an Analytics request/subscribe to NWDAF by invoking Nnwdaf_AnalyticsSubscription_Subscribe / Nnwdaf_AnalyticsInfo_Request including Analytics ID = E2E data volume transfer time, Application ID, DNN, S-NSSAI, AoI, and target UEs based on the initial UE list obtained from the AF, the target number of data transmission repetitions or target time interval between data transmissions, the Data Volume UL/DL, request for geographical distribution (i.e. the AoIs) of the UEs, etc.

5.	The NWDAF collects data from multiple sources for end-to-end data volume transfer time analytics as specified in clause 6.18.2 of TS 23.288 [50].

6.	The NWDAF provides the required output analytics to the consumer NF as specified in clause 6.18.4 of TS 23.288 [50] by means of either Nnwdaf_AnalyticsInfo_Request response or Nnwdaf_AnalyticsSubscription_Notify, depending on the service used in step 4.

7.	Based on the analytics received from the NWDAF, the NEF consolidates results and derives the list(s) of candidate UE(s) that fulfil the filtering criteria requested by the AF. The NEF may use the average and/or the variance of end-to-end data volume transfer time of specific volumes of UL/DL data indicated by the AF to derive the list(s) of candidate UEs that meet the requirements from the AF.

8.	NEF sends a Nnef_MemberUESelectionAssistance_Notify request to the AF including the list(s) of candidate UE(s) and additional information.

* * *End of Changes * * *