3GPP SA WG2 Meeting #160AH-e	S2-2400461r01

# E-meeting, January 22 – 29, 2024		(revision of S2-23XXXXX)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 4687 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to MemberUESelectionAssistance for updating the list of target member UEs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIMLsys |  |  |  |  |  | Date: |  |  | 2024-01-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The current description of Nnef_MemberUESelectionAssistance_subscribe can only update the filtering criteria of the existing subscription, not the list of target member UEs. However, existing procedure may misleading that the AF may update the list of target member UEs. 2b. [CONDITIONAL] If the AF request contains a Subscription Correlation ID, the NEF correlates the Nnef_MemberUESelectionAssistance_Subscribe request to an existing subscription according to the Subscription Correlation ID. The NEF uses the list of target member UE(s) received in step 1 for the Member UE update using the updated filtering criteria. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Clarifies that the Member UE Selection Assistance subscription is applied for initial list of target member UEs - Editorial changes |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Nnef_MemberUESelectionAssistance_subscribe can update the list of target member UEs. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.15.13.0, 4.15.13.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change * * * *

4.15.13.0	General

With the initial subscription request, the AF provides a list of target member UEs in the form of a list of GPSIs or a list of UE IP addresses and at least one Member UE filtering criterion as part of the service operation parameters to assist the candidate UEs selection. Upon receiving the AF request, NEF triggers corresponding 5GC procedures to retrieve from 5GC NFs the information for each UE in the list of target member UEs. Before sending the list(s) of candidate UEs to the AF, NEF consolidates all the information collected from other 5GC NFs and derives one or more list(s) of candidate UEs and possibly additional information according to the Member UE filtering criteria requested by the AF.

AF may subsequently update the Member UE filtering criteria and notify the NEF with Subscription Correlation ID. AF does not include the list of target member UEs if the list of target member UEs is not changed. AF may request to update the parameters (e.g. expiry time, time window for selecting the candidate UEs) for the subscribed Member UE filtering criteria with Subscription Correlation ID. AF may also request to update by adding new Member UE filtering criteria or removing part of the subscribed Member UE filtering criteria.

The Member UE selection assistance capability can be used to assist the AF to select the list of member UEs to support application service (e.g. FL operation) and it is further defined in clause 5.46.2 of TS 23.501 [2].

Additionally, AF may leverage the 5GC network exposure for Member UE selection without the NEF assistance as described in clause 5.46.2 of TS 23.501 [2]. An example of how the AF leverages the 5GC network exposure for Member UE selection is described in (informative) Annex I.

AF may also unsubscribe the previous subscription for Member UE selection assistance information as described in clause 4.15.13.7.

4.15.13.1	Member UE selection general information flow

This clause describes the Member UE selection procedure that is generally applicable independently of the Member UE filtering criteria sent by the AF.

Figure 4.15.13.1-1: 5GC assistance to Member UE selection and update

1.	AF subscribes the Member UE selection assistance information by sending a first Nnef_MemberUESelectionAssistance_subscribe request including a list of target member UEs, one or more member UE filtering criteria as listed in Table 4.15.13.2-1 and optionally, time window(s). Subsequently, the AF may only update the Member UE filtering criteria of the subscription as described in clause 4.15.13.0 by invoking Nnef_MemberUESelectionAssistance_subscribe and providing a Subscription Correlation ID, i.e. the AF does not provide the list of target member UEs again.

NOTE:	The time window(s) for selecting the candidate UE(s) is used by the NEF when subscribing/requesting to NWDAF. The NEF maps the time window(s) for selecting the candidate UE(s) to the Analytics target period, which should be included in the Nnwdaf_AnalyticsSubscription_Subscribe or Nnwdaf_AnalyticsInfo_Request service operations.

2a.	[CONDITIONAL] If the AF request does not contain a Subscription Correlation ID, the NEF verifies the authorization of the AF Request and identifies which information needs to be collected for each UE in the list of target member UEs and executes the corresponding service operations based on the Member UE filtering criteria provided by the AF, e.g. events, analytics ID(s), notifications, etc.

2b.	[CONDITIONAL] If the AF request contains a Subscription Correlation ID, the NEF correlates the Nnef_MemberUESelectionAssistance_Subscribe request to an existing subscription according to the Subscription Correlation ID. The NEF uses the list of target member UEs received in initial subscription (i.e. no Subscription Correlation ID in the subscription request) for the Member UE update using the updated filtering criteria.

3.	NEF interacts with different 5GC network functions to collect the required information for each UE in the list of target member UEs. The set of interactions between the NEF and the 5GC NFs depend on the Member UE filtering criteria provided by the AF. See Table 4.15.13.2-1 for details.

4.	Based on the collected information from other 5GC NFs, the NEF consolidates all the information to derive the list(s) of candidate UEs which fulfil the Member UE filtering criteria in the AF request. The NEF may derive recommended time window(s), considering the validity period(s) of the analytics used for Member UE selection criteria. The recommended time window(s) may be a subset of the time window(s) received from the AF. In different recommended time windows, the list of candidate UE(s) which fulfil the Member UE filtering criteria may be different.

5.	NEF sends a Nnef_MemberUESelectionAssistance_Notify request to the AF including the list(s) of candidate UEs and possibly additional information. See clause 5.2.6.32.4 for details.

* * * * End of Changes * * * *