3GPP TSG-WG SA2 Meeting #160-Ad Hoc-e	S2-2400561r17

Online, Jan 22 – 29, 2024	(revision of S2-240xxxx)

Source:	vivo (rapporteur), MediaTek Inc. (rapporteur), China Mobile, OPPO, Ericsson? Samsung, Orange? NTT DoCoMo, Nokia, Nokia Shanghai-Bell

Title:	New KI for NWDAF-assisted policy control and QoS enhancement

Document for:	Approval

Agenda Item:	19.15

Work Item / Release:	FS_AIML_CN / Rel-19

Abstract: It is proposed a new Key Issue for WT #3.1.

1. Introduction/Discussion

Agreed R19 FS_AIML_CN SID (SP-231800) contains the following.

- WT3: Study enhancements to support NWDAF-assisted policy control and address network abnormal behaviour - WT3.1 – Study whether and what additionally needs to be supported in order to enhance 5GC NF operations (i.e. policy control and QoS) assisted by NWDAF. The work will firstly identify the specific use cases to be considered, in order to identify the appropriate scope. The work will analyse the result impacts on NWDAF (e.g. the need to understand specific NF functionality), and the compatibility of new solutions wrt existing analytics, in order to determine the need and benefits of new solutions. -…. NOTE H: The study will focus primarily on existing enforcement mechanisms when available and identify new ones when no existing ones can be used.

Currently, NWDAF can derive statistics and predictions, which can be utilized by the consumer to make decisions. For example, the PCF can determine QoS parameters to be applied for a specific service based on the Service Experience analytics from NWDAF as described in clause 6.4 of TS 23.288.

In this study, it is proposed to include a new key issue for NWDAF-assisted policy control and QoS.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-84.

# * * * * First change * * * *All new

X.Y	Key Issue #Z – NWDAF-assisted policy control and QoS enhancement

X.Y.1	Description

This Key issue aims to study whether and what additionally needs to be supported in order to enhance 5GC NF operations related to policy control and QoS with the assistance of the NWDAF.

NOTE 1:	As known, NWDAF gathers quite a lot of data from 5GC NFs, AF and OAM and thus can further assist the PCF in making PCC decisions (which traditionally determine QoS parameters based on its own data and knowledge as well optional statistics and predictions collected from the NWDAF).

In this key issue, the following aspects will be studied:

- Identification of use cases where policy control and QoS can be further enhanced with assistance from NWDAF.

- Whether and how to introduce new 5GC functionality e.g. of the NWDAF and/or PCF to enhance the policy control and QoS, considering operator’s policies.

- Whether and what additional input information is needed by the NWDAF for providing an assistance to policy control and QoS, and how to gather it.

- Whether and what output information, on top of already provided, the NWDAF can provide to assist with policy control and QoS enhancements.

- Whether and how to evaluate the quality of the enhanced NWDAF assistance to policy control and QoS.

NOTE 2: 	The study will focus primarily on existing enforcement mechanisms when available and identify new ones only when no existing ones can be used.

# * * * * End of changes * * * *