3GPP TSG-WG SA2 Meeting #160-Ad Hoc-e	S2-2400562r08

Online, Jan 22 – 29, 2024	(revision of S2-240xxxx)

Source:	vivo (rapporteur), MediaTek Inc. (rapporteur), China Mobile, KPN, ZTE, ETRI, Tencent, Tencent Cloud

Title:	New key issue for NWDAF enhancements to support network abnormal behaviours (i.e. Signalling storm) mitigation

Document for:	Approval

Agenda Item:	19.15

Work Item / Release:	FS_AIML_CN / Rel-19

Abstract: It is proposed a new Key Issue for WT #3.2.

# 1. Introduction/Discussion

Based on the last SA#102 Plenary meeting for approved R19 FS_AIML_CN (SP-231800), this paper proposes a KI related to WT3.2.

- WT3: Study enhancements to support NWDAF-assisted policy control and address network abnormal behaviour - WT3.1 – Study whether and what additionally needs to be supported in order to enhance 5GC NF operations (i.e. policy control and QoS) assisted by NWDAF. The work will firstly identify the specific use cases to be considered, in order to identify the appropriate scope. The work will analyse the result impacts on NWDAF (e.g. the need to understand specific NF functionality), and the compatibility of new solutions wrt existing analytics, in order to determine the need and benefits of new solutions. - WT3.2 – Study prediction, detection, prevention, and mitigation of network abnormal behaviours i.e. signalling storm with the assistance of NWDAF.

Signalling storm will heavily consume network resources and result in  service outages. The signalling storm mitigation study is to leverage AI/ML Technology to mitigate network abnormal behaviours/signalling storm happened with the assistance of NWDAF.

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-84.

# * * * * First change * * * *All new

### X.1	Key Issue #X: NWDAF enhancements to support network abnormal behaviours (i.e. Signalling storm) mitigation

#### X.1.1	Description

This Key issue aims to provide solutions for prediction, detection, prevention, and mitigation of network abnormal behaviours i.e. signalling storm with the assistance of NWDAF. In particular, the following aspects shall be addressed:

-  (optionally) Identify which scenarios can result in a signalling storm situation and identify what are the main use cases  to be focused in Rel-19 that cause the signalling storm;

-	Whether and how to detect or predict signalling storm with the assistance of NWDAF?

- Whether and how existing Analytics IDs, or new Analytic ID(s) can be used to assist signalling storm analytics.?

-

How to enhance the procedures among the related entities with the assistance of NWDAF, 5GC NFs and/or with the support of OAM to support prevention and mitigation for the signalling storm (e,g. NF discovery and (re)selection, policy control).

NOTE 1: In terms of data access right, privacy and security improvement, cooperation with SA3 is needed.

# * * * * End of changes * * * *